3GPP TSG-RAN WG3 #115-e	R3-22xxxx

21 Feb -3 Mar 2022

Online

Agenda item:	15.4

Source: 	ZTE

Title: 	(TP to BL CR of TS38.413) Alignment of MDT and QoE Measurements

Document for:   Agreement

- Introduction

This paper provides TP for 38.413 on the configuration for alignment of MDT and QoE measurements.

- 2. TP to BL CR of 38.413

Start of changes

#### 9.3.1.xx3	UE Application Layer Measurement Information

This IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for Application layer Measurement Configuration | O |  | Octet string (1..)[FFS] | Indicates application layer measurement configuration, see Annex L in [xx]. Present in case of initial QoE configuration.[FFS] | - | - |
| Measurement Configuration Application Layer ID | O |  |  | This IE is present only when the message containing it is NG-based handover related. Otherwise, this IE is not needed. The IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [18]. | - | - |
| CHOICE Area Scope of QMC | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>NG-RAN CGI | M |  | 9.3.1.73 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>TAI | M |  | 9.3.3.11 |  | - | - |
| >PLMN area based |  |  |  |  |  | - |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - | - |
| Service Type | M |  | 9.3.1.xx4 | This IE indicates the service type of UE application layer measurements. | - | - |
| QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [x]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3-bytes Octet String. |  |  |
| Measurement Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | The IP address of the entity receiving the QoE measurement report. |  |  |
| S-NSSAI List | O | 0 .. 1 |  |  |  |  |
| >S-NSSAI Item |  | 1 .. <maxnoofS-NSSAIforQMC> |  |  |  |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  |  |  |
| CHOICE MDT Alignment Information | O |  |  | Indicates the MDT measurements with which alignment is required. | - | - |
| >S-based MDT |  |  |  |  |  |  |
| >>NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.1), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). | - | - |
| >M-based MDT |  |  |  | Present in case of initial QoE configuration. |  |  |
| >>Any Available MDT | M |  | ENUMERATED (true, …) | This IE indicates that the NG-RAN should align the QoE measurement with any ongoing MDT at the same time in the same area for the UE, as specified in TS38.300[8]. | - | - |
| RAN Visible QoE Metric Indication | O |  | 9.3.1.xx5 |  |  |  |

Next change

### 9.4.5	Information Element Definitions

<unchaned omitted>

#### -- A

<unchaned omitted>

AMF-TNLAssociationToUpdateItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AnyAvailableMDT ::= ENUMERATED {

true,

...

}

AreaOfInterest ::= SEQUENCE {

areaOfInterestTAIList			AreaOfInterestTAIList						OPTIONAL,

areaOfInterestCellList			AreaOfInterestCellList						OPTIONAL,

areaOfInterestRANNodeList		AreaOfInterestRANNodeList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterest-ExtIEs} }	OPTIONAL,

...

}

<unchaned omitted>

#### -- M

<unchaned omitted>

MDT-AlignmentInfo ::= CHOICE {

s-basedMDT		NGRANTraceID,

m-basedMDT		AnyAvailableMDT,

choice-Extensions		ProtocolIE-SingleContainer { { MDT-AlignmentInfo-ExtIEs} }

}

MDT-AlignmentInfo-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<unchaned omitted>

#### -- U

<unchaned omitted>

UEAppLayerMeasInfor ::= SEQUENCE {

serviceType								ServiceType,

qoEReference							QoEReference,

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..1000))			OPTIONAL,

measConfigAppLayerID					INTEGER (0..61, ...) 					OPTIONAL,

areaScopeOfQMC							AreaScopeOfQMC,

measCollEntityIPAddress					TransportLayerAddress,

sliceSupportListQMC				SliceSupportListQMC 					OPTIONAL,

mDT-AlignmentInfo						MDT-AlignmentInfo						OPTIONAL,

measurementStatus						MeasurementStatus						OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { UEAppLayerMeasInfor-ExtIEs} }	OPTIONAL,

...

}

<unchaned omitted>

End of changes