3GPP TSG-RAN WG3 Meeting #115-e	R3- 22xxxx

E-meeting, 21 Feb – 3 Mar 2022

Title: 	(TP for SON BLCR for38.300) Coverage and Capacity Optimization

Source: 	Samsung

Agenda item:	10.2.2

Document Type:	other

1. Introduction

This is a TP for 38.401.

Annex  – TP for 38.401

Start of the change

7.Y	Support for CCO

7.Y.1	General

The NR Capacity and Coverage Optimization (CCO) Function in non-split gNB case is specified in TS 38.300 [2]. The objective of this function is to detect and mitigate coverage and cell edge interference issues.

7.Y.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contains relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

7.Y.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU informs the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves a CCO issue via coverage modification. The gNB-DU informs the gNB-CU of the new coverage state. The gNB-CU notifies the neighbour NG-RAN node or neighbour gNB-CU as specified in TS 38.300 [2].