3GPP TSG-RAN WG3 #116-e																		R3-223905

Electronic Meeting, 9th – 19th May 2022 Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0694 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extended PDCP Discard Timer over E1 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, ZTE,CATT,Huawei,Nokia, Nokia Shanghai Bell, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2022-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, a new IE DiscardtimerExt was introduced in TS 38.331 to support URLLC enhancement. However, this IE is not supported in E1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | a new IE DiscardtimerExt was introduced in PDCP configuration to support smaller values for PDCP discard timer. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CU-UP could not be configured with smaller values for PDCP discard timer. And some URLLC use cases such as motion control could not be supported in CP/UP split architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.x, 9.3.1.38, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: ASN.1 and new IE name update |  |  |  |  |  |  |  |  |

////////////////////////////////////////////////////////////////////////start of change ////////////////////////////////////////////////////////////////////////

9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| ROHC Parameters | O |  | 9.3.1.40 |  | - | - |
| T-Reordering Timer | O |  | 9.3.1.41 |  | - | - |
| Discard Timer | O |  | 9.3.1.42 | This IE is ignored if the Discard Timer Extended IE is present. | - | - |
| UL Data Split Threshold | O |  | 9.3.1.43 |  | - | - |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. This IE is ignored when the “Additional PDCP duplication Information” IE is present. | - | - |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] | - | - |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] | - | - |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication | - | - |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established. | - | - |
| PDCP Status Report Indication | O |  | ENUMERATED (downlink, uplink, both, …) | For AM DRB, “downlink” indicates that the PDCP entity is configured to send PDCP status report(s) to the UE, and “uplink” indicates that the UE is configured to send PDCP status report(s), as specified in TS 38.323 [17]. “both” indicates that both “downlink” and “uplink” should be applied. | YES | ignore |
| Additional PDCP duplication Information | O |  | ENUMERATED (three, four, …) | Indicates the number of PDCP duplication configured when it is more than 2 for the DRB | YES | ignore |
| EHC Parameters | O |  | 9.3.1.90 |  | YES | ignore |
| Discard Timer Extended | O |  | 9.3.1.x |  | YES | reject |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.3.1.x	Discard Timer Extended

This IE indicates the extended PDCP discard timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Discard Timer Extended |  |  | ENUMERATED (0.5, 1, 2, 4, 6, 8, …) | Indicates the PDCP discard timer. The values are expressed in ms. For more information see DiscardTimerExt-r16 in PDCP-Config IE in TS 38.331 [10]. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

id-SecurityIndicationModify,

id-DataForwardingSourceIPAddress,

id-DiscardTimerExtended,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

#### -- D

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DataForwardingtoNG-RANQoSFlowInformationList	CRITICALITY ignore	EXTENSION DataForwardingtoNG-RANQoSFlowInformationList	PRESENCE optional},

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

DataForwardingtoE-UTRANInformationList ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingtoE-UTRANInformationListItem

DataForwardingtoE-UTRANInformationListItem ::= SEQUENCE {

data-forwarding-tunnel-information							UP-TNL-Information,

qoS-Flows-to-be-forwarded-List						QoS-Flows-to-be-forwarded-List,

iE-Extensions		ProtocolExtensionContainer { { DataForwardingtoE-UTRANInformationListItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingtoE-UTRANInformationListItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

inter-system-direct-path-available,

...,

intra-system-direct-path-available

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DiscardTimerExtended	::=	ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8,... }

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-PDCP-StatusReportIndication					CRITICALITY ignore	EXTENSION PDCP-StatusReportIndication					PRESENCE	optional}|

{ ID id-AdditionalPDCPduplicationInformation		CRITICALITY ignore	EXTENSION AdditionalPDCPduplicationInformation	PRESENCE optional	}|

{ ID id-EHC-Parameters								CRITICALITY ignore	EXTENSION EHC-Parameters									PRESENCE optional}|

{ ID id-DiscardTimerExtended								CRITICALITY reject	EXTENSION DiscardTimerExtended									PRESENCE optional},

...

}

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info									ProtocolIE-ID ::= 86

id-QoSMonitoringRequest											ProtocolIE-ID ::= 87

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 88

id-gNB-CU-CP-Measurement-ID										ProtocolIE-ID ::= 89

id-gNB-CU-UP-Measurement-ID										ProtocolIE-ID ::= 90

id-RegistrationRequest											ProtocolIE-ID ::= 91

id-ReportCharacteristics										ProtocolIE-ID ::= 92

id-ReportingPeriodicity											ProtocolIE-ID ::= 93

id-TNL-AvailableCapacityIndicator								ProtocolIE-ID ::= 94

id-HW-CapacityIndicator											ProtocolIE-ID ::= 95

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= 96

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= 97

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= 98

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= 99

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= 100

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 101

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= 102

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= 103

id-AdditionalPDCPduplicationInformation							ProtocolIE-ID ::= 104

id-RedundantPDUSessionInformation								ProtocolIE-ID ::= 105

id-RedundantPDUSessionInformation-used							ProtocolIE-ID ::= 106

id-QoS-Mapping-Information										ProtocolIE-ID ::= 107

id-DLUPTNLAddressToUpdateList									ProtocolIE-ID ::= 108

id-ULUPTNLAddressToUpdateList									ProtocolIE-ID ::= 109

id-NPNSupportInfo												ProtocolIE-ID ::= 110

id-NPNContextInfo												ProtocolIE-ID ::= 111

id-MDTConfiguration												ProtocolIE-ID ::= 112

id-ManagementBasedMDTPLMNList									ProtocolIE-ID ::= 113

id-TraceCollectionEntityIPAddress								ProtocolIE-ID ::= 114

id-PrivacyIndicator												ProtocolIE-ID ::= 115

id-TraceCollectionEntityURI										ProtocolIE-ID ::= 116

id-URIaddress													ProtocolIE-ID ::= 117

id-EHC-Parameters												ProtocolIE-ID ::= 118

id-DRBs-Subject-To-Early-Forwarding-List						ProtocolIE-ID ::= 119

id-DAPSRequestInfo												ProtocolIE-ID ::= 120

id-CHOInitiation												ProtocolIE-ID ::= 121

id-EarlyForwardingCOUNTReq										ProtocolIE-ID ::= 122

id-EarlyForwardingCOUNTInfo										ProtocolIE-ID ::= 123

id-AlternativeQoSParaSetList									ProtocolIE-ID ::= 124

id-ExtendedSliceSupportList									ProtocolIE-ID ::= 125

id-MCG-OfferedGBRQoSFlowInfo									ProtocolIE-ID ::= 126

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-EarlyDataForwardingIndicator									ProtocolIE-ID ::= 140

id-QoSFlowsDRBRemapping											ProtocolIE-ID ::= 141

id-DataForwardingSourceIPAddress								ProtocolIE-ID ::= 142

id-SecurityIndicationModify										ProtocolIE-ID ::= 143

id-IAB-Donor-CU-UPPSKInfo											ProtocolIE-ID ::= 144

id-DiscardTimerExtended												ProtocolIE-ID ::= xxx

END

-- ASN1STOP

////////////////////////////////////////////////////////////////////////end of change////////////////////////////////////////////////////////////////////////