3GPP TSG-RAN WG3 Meeting #116-e	R3-223763

Online, 9-19 May 2022

Agenda item:	9.3.9

Source:	Huawei (moderator)

Title:	Summary of offline: Retrieve UE context for NPN only cell

Document for:	Discussion and Decision

# 1	Introduction

This paper summarizes the following email discussion:

CB: # 58_NPNOnlyCell

- Evaluate the solutions and make the decision

(HW - moderator)

Summary of offline disc R3-223763

# 2	For the Chair’s Notes

TBD

# 3	Discussion (Phase 1)

Based on the previous discussions, the following issue is very clear to all companies (also captured at the previous meeting):

The issue is acknowledged: How could the anchor NG-RAN node acquires the PCI and ARFCN information based on the first PLMN Cell ID to calculate the target key in case of the NPN-only cell?

Minutes in previous meetings: No need to involve with RAN2 with the given analysis. Down-select the two options at the next meeting. Option 2: Add a note “In case of NPN-only cell, it is assumed that SIB1 contains the same value included in the CellIdentity in the first entry of the NPN-IdentityInfoList also in the first entry of the PLMN-IdentityInfoList.”; Option 4: introduce a new NR CGI IE (including the first PLMN Identity and the first NR Cell Identity in SIB1) at Xn setup/RAN configuration update message. To be continued at this basis…

As indicated in R3-223462, there are two options below:

- Option 2: introduce a note in the Served Cell Information NR IE

  - “NOTE 2: In case of NPN-only cells the SIB1 configuration replicates the content of the cellIdentity-r16 of the first entry in the npn-IdentityInfoList-r16 into the cellIdentity of the first entry of the plmn-IdentityInfoList.”

- Option 4: introduce a new IE in the Served Cell Information NR IE

  - “Include the NR CGI of First PLMN in the Served Cell Information NR in case of NPN-only cell.”

Also in R3-223462, it provides some pros/cons analysis.

|  | Pros | Cons |
| --- | --- | --- |
| Option 2 - A new Note | - no additional IEs to be defined | - may require RAN2 work and involvement - impose restrictions on the assignment of Cell IDs |
| Option 4 - Signalling | - no RAN2 work and involvement - allow flexible assignment of Cell IDs | - new IEs to be defined over the XnAP |

The moderator understands that companies views were already collected at the previous meeting, and appreciates your further input to close this topic.

Question 1: which option is your preferred option, or which option is not acceptable, and why?

| Company | Comments |
| --- | --- |
| Huawei | Option 4 is preferred as indicated in R3-223462. |
| Moderator Summary: - TBD. |  |

If the situation is not changed compared with the previous meeting, then we can consider potential way-forward, and close this topic.

Below provides possible way-forward for your consideration:

- WF#1: Leave as it is (i.e. no further action)

- WF#2: MCC minute in Chairman notes: It is up to network implementation to determine the target key for NPN only cell case in case of UE context retrieval procedure (maybe updated based on further input)

- WF#3: others?

Question: Please provide your preference of way forward, or any other suggestions to be discussed at the 2nd round.

| Company | Comments |
| --- | --- |
| Moderator Summary: - TBD. |  |

# 4	Discussion (Phase 2)

# 5	Conclusions, Recommendations

TBD

# References

| R3-223462 | Retrieve UE context for NPN only cell (Huawei, Deutsche Telekom, Orange, Nokia, Nokia Shanghai Bell) | Discussion E///: Signaling solution is not acceptable ZTE: Prefer Op2 without any signaling impact on RAN2 and RAN3 CB: # 58_NPNOnlyCell - Evaluate the solutions and make the decision (HW - moderator) Summary of offline disc R3-223763 |
| --- | --- | --- |
| R3-223463 | Retrieve UE context for NPN only cell (Huawei, Deutsche Telekom, Orange, Nokia, Nokia Shanghai Bell) | CR0734r3, TS 38.423 v16.9.0, Rel-16, Cat. F |
| R3-223464 | Retrieve UE context for NPN only cell (Huawei, Deutsche Telekom, Orange, Nokia, Nokia Shanghai Bell) | CR0816r, TS 38.423 v17.0.0, Rel-17, Cat. A |