3GPP TSG-RAN WG3 Meeting #117-e	R3-xxxxxx

E-meeting, Aug 15-Aug 24, 2022

Agenda Item:	14.4

Source:	Qualcomm Incorporated (moderator)

Title:	Summary of Offline Discussion on CB: # MobilityEnh3_Others

Document for:	Approval

# Introduction

CB: # MobilityEnh3_Others

- Supports or prioritize SCG selective activation or MCG selective activation?

- Selective activation of the cell groups within one CU (i.e. Intra-CU inter-DU case) include both MCG change and SCG change?

- CPAC procedures as baseline?

- Deprioritized MN-initiated CPC scenario for selective activation?

- Prioritize on Intra-SN SCG selective activation and Inter-SN SCG selective activation?

- Capture agreements and open issues.

(Qualcomm - moderator)

Summary of offline disc: R3-225019.

# For the Chairman’s Notes

To be added.

# Discussion

## Scenarios: Discussion on support/prioritization of SCG selective activation or MCG selective activation

Contributions from companies [11], [10], [9], [8], [2], [1], [3], have considered the question on which of the following scenarios to support/prioritize in the Rel-18 work for Objective 2: SCG selective activation or MCG selective activation.

The statement of the objective in the WID [13] seems to indicate that SCG selective activation should be prioritized in the Rel-18 work. Other contributions from companies [4], [5], [6], [7], [12], seem to assume that SCG selective activation should be considered first and discuss some related design proposals.

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

Question 1: Do companies agree that SCG selective activation should be prioritized in the Rel-18 work? Please provide any additional views you may have in the comments.

| Company | Answer | Comment |
| --- | --- | --- |

## Scenarios: Discussion on support/prioritization of selective cell group (CG) activation within one CU or across CUs (Intra-CU vs. Inter-CU)

The contribution [2] proposes that the two scenarios on selective cell group (CG) activation within one CU (Intra-CU, Inter-DU case) and across CUs (Inter-CU) should be considered. It proposes that the Intra-CU scenario should be prioritized including both MCG change and SCG change.

Question 2: Do companies agree that selective CG activation within one CU, i.e., the Intra-CU case, should be prioritized in the Rel-18 work? Please provide any additional views you may have in the comments.

| Company | Answer | Comment |
| --- | --- | --- |

## On considering Rel-16/Rel-17 CPAC procedures as baseline

The contributions [10], [9], [12], [11], [8], [7], [6], [5], [4], [3], [1], have proposed using the Rel-16/Rel-17 CPAC procedures as baseline for the Rel-18 work on Objective 2. This seems aligned with the statement of the objective in the WID [13], where L3 enhancements are mentioned.

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

The contribution [2] proposes that the L1/L2 mobility framework is suitable for handling the Intra-CU case whereas the CPAC framework is suitable for handling the Inter-CU case. Some companies [9], [12], think that the work in Rel-18 can proceed now with considering CPAC procedures as baseline, and that harmonization with the L1/L2 mobility framework can be considered later when there is sufficient progress on both Objectives 1 and 2 of the work item.

Question 3: In case companies have answered “Yes” to Question 1, do companies agree that the Rel-16/Rel-17 CPAC procedures can be used as the baseline for the Rel-18 work? Companies are also requested to provide their views on the aspect of “harmonization with the L1/L2 mobility framework”.

| Company | Answer | Comment |
| --- | --- | --- |

## Scenarios: Discussion on support/prioritization of MN-initiated CPC, CPA, or SN-initiated CPC, for selective activation of the SCG

In Rel-16/Rel-17 CPAC, the procedures can be initiated by the MN or the SN. In the contribution [1], it is proposed that the MN initiated Inter-SN CPC scenario can be deprioritized for selective activation of the SCG. The reason stated in [1] is that this scenario is mainly defined for offloading traffic to a better target PSCell (SCG). Other contributions [5], [9], [11], think that the MN as well as the SN initiated CPC scenarios can be considered. The contribution [8] proposes to consider the SN initiated Intra-SN CPC scenario first because it is probably the least complex.

Question 4: In case companies have answered “Yes” to Question 1, do companies agree that the MN initiated Inter-SN CPC scenario can be deprioritized for selective activation of the SCG? Please also indicate your preferences and/or priority order among the following possible scenarios in the Rel-18 work: (a) CPA (b) MN initiated Inter-SN CPC (c) SN initiated Inter-SN CPC (d) SN initiated Intra-SN CPC.

| Company | Answer | Comment |
| --- | --- | --- |

## Other scenario related discussion

The following scenarios were also proposed in the company contributions to be included in the scope of the Rel-18 work on Objective 2. In the following question, we try to collect company views on these.

Question 5: Do companies think that the following scenarios should be included in the scope of the Rel-18 work on Objective 2? Please also indicate any other scenarios that you think should be included in the scope of the Rel-18 work.

- SCG failure handling enhancements to enable PSCell addition and PSCell change after SCG failure [1].

- Network configuration of selective activation based on UE mobility pattern, e.g., UE History Information, VisitedCellInfoList [3].

- Focus on the Intra-MN, Inter-SN mobility case [6].

- Signaling support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration [10].

| Company | Answer | Comment |
| --- | --- | --- |

## Keeping conditional CPAC configurations after cell change

In their contributions companies have indicated that a primary focus of the objective is to enable subsequent cell changes (PSCell changes/additions) by keeping conditional reconfigurations after a cell change (PSCell change/addition) [1], [3], [8], [9], [10], [11], [12]. This is included in the statement and discussion of the objective in the WID document [13] and may thus generally be agreeable.

Companies have also indicated that some Uu and Xn signaling changes are required to support this [5], [9], [11], [12].

Question 6: Do companies agree that a primary focus of the objective is to enable subsequent cell changes (PSCell changes/additions) by keeping conditional reconfigurations after a cell change (PSCell change/addition)? Do companies agree that Xn signaling changes are required to support this objective?

| Company | Answer | Comment |
| --- | --- | --- |

## Any other issues for discussion

Question 7: Please indicate if there are any other issues or proposals on the topic that you would like to discuss.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations

To be added.

# References

- R3-224253 NR-DC with selective activation of cell groups (Nokia, Nokia Shanghai Bell)	discussion

- R3-224271 Discussion on NR-DC with selective activation of the cell groups (ZTE)	discussion

- R3-224287 Duscussion of selective activation (NTT DOCOMO INC.)	discussion

- R3-224297 NR-DC with selective activation of the cell groups (NEC)	discussion

- R3-224323 Consideration on NR-DC with selective activation of SCG (Huawei)	discussion

- R3-224344 Subsequent CPC/CPA after PSCell Change (vivo)	discussion

- R3-224395 Consideration on support of subsequent CPC/CPA (China Telecommunication)	discussion

- R3-224437 On selective cell group activation (Lenovo)	discussion

- R3-224511 Configuration and activation of multiple cell groups in NR-DC (Qualcomm Incorporated)	discussion

- R3-224521 NR-DC with selective activation (Ericsson)	discussion

- R3-224795 Discussion on NR-DC with selective activation of the cell groups (CATT)	discussion

- R3-224836 Considerations on selective activation of the cell groups (Samsung)	discussion

- RP-221799	“Revised WID: Rel-18 Further NR mobility enhancements”, MediaTek, Jun 2022