3GPP TSG-RAN WG3 Meeting #117bis-s                     R3-22xxxx

# E-meeting, 10th – 18th October 2022

Title:	[Draft] LS on concurrent gap configuration over F1

Release:	Rel-17

Work Item:	NR_MG_enh-Core

Source:	ZTE (to be RAN3)

To:	RAN2

Cc:	N/A

Contact Person:

Name:	Jiren Han

E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

In Rel-17, the concurrent measurement gap has been introduced in RAN2. To support this feature in the split gNB scenario, the concurrent gap configuration over F1 has been discussed, i.e. which entity shall configure the association between the measurement gap and the measurement object. Obviously, there are two options to address the issue.

Option 1: The gNB-CU shall decide and configure the gap association Information

Option 2: The gNB-DU shall generate the gap association information and sent it to the gNB-CU

For Option 1, some companies think the gNB-CU can decide the gap association based on the gapToAddModList-r17 IE and Measurement Timing Configuration since the gNB-CU is responsible for the Measurement Object definition and overall MeasConfig.

For Option 2, some companies think gNB-DU can decide the gap association information since the gNB-DU decides the gap configuration and gap type. In addition, as the gap priority is also decided by the gNB-DU, it is optimal to make the gNB-DU decide the gap association information.

As the MeasConfig and MeasGapConfig are defined by RAN2 and the concurrent measurement gap has been supported in RAN2 in Rel-17, RAN2 may provide some suggestions on the concurrent gap configuration related issue.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2 to give some suggestions on which option should be applied for the concurrent measurement gap configuration over F1.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #118       14th – 18th Nov. 2022			 Toulouse, France