3GPP TSG-RAN WG3 Meeting #118	R3-226264

Toulouse, FR, 14th – 18th Nov 2022

Title: 	(TPs for BLCRs for TS 38.401 and TS 38.423) SL relay: Inter-gNB mobility

Source: 	Huawei

Agenda item:	16.3

Document Type:	other

# 1. Introduction

In last RAN3 meeting, we have reached the following agreements/ working assumptions/ FFSs for R18 SL relay enhancement.

WA: Source gNB selects the target path type (direct or indirect) For direct/indirect to indirect path switching, enhance Xn: HANDOVER REQUEST to include at least the Remote UE L2 ID and Relay UE L2 ID. FFS whether to include a single Target Relay L2 ID or a list of Target candidate Relay L2 IDs. For inter-gNB path switching scenarios, RAN3 should specify mechanisms to support service continuity for L2 U2N relays in NG based handovers as well after supporting service continuity for L2 U2N relays in Xn based handovers, If there is some conclusion from SA2, and then to support NG based HO. WA: During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID.

Figure. 1. Service continuity scenarios in R18 SL relay enhancement

In this paper, we will continue discussing the left open issues to support the service continuity scenarios as shown in Figure 1.

# 2. Discussion

In legacy inter-gNB handover procedure, the source gNB decides to initiate the handover for a UE based on the measurement report, and determines the target cell. The target gNB will perform the admission control and determines the UE’s configuration at the target cell, i.e., the target gNB does not have the chance to select the target cell. That is to say, it is source gNB’s responsibility or KPI (key performance indicator) to guarantee the handover success of the UE.

Observation 1: In legacy inter-gNB handover procedure, it is source gNB’s KPI to guarantee the handover success of a UE.

We think it is better to follow the legacy principle for R18 SL relay enhancement and agree the working assumption that source gNB selects the target path type (direct or indirect). In last meeting, majority companies preferred this options. Besides, RAN2 are waiting for RAN3 decision on this issue, thus RAN3 should notify RAN2 when the WA is agreed.

Proposal 1: Turn the WA that source gNB selects the target path type (direct or indirect) into agreement and inform RAN2 about the agreement.

For the target relay UE selection issue, there are three options as below.

- Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

- Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

Option 1 is more aligned with the legacy HO procedure, and the only impact is to carry the target relay UE ID in the Xn signalling from source gNB to target gNB. Option 2/3 might allow target gNB to choose a Relay UE with better Uu link other than only considering sidelink measurement results reported by the remote UE, but it is only applicable in case the Relay UE is in RRC_CONNECTED state. Even multiple relay UE is provided by the source gNB, target gNB can reject, which causes split responsibilities. Besides, multiple candidate relay UE information should be carried in the HO request message, which can complicate Xn signalling. Considering the simplicity, we slightly prefer to choose Option 1.

Proposal 2: Source gNB selects the target Relay UE if it decides to switch the UE to an indirect path, and sends the target related information to the target gNB.

In the following, we will also show the whole working procedures of each path switch cases to check the above proposals, and discuss whether there is any other potential impacts on RAN3.

2.1 Inter-gNB indirect-to-direct path switching

Remote UE

Relay UE

S-gNB

9. sidelink release

T-gNB

3. HO request

4. HO ACK

6. Access to target cell

8. RRC Reconfiguration

UL and DL data

5. RRCReconfiguration

1. Measurement configuration and reporting

2. HO decision

7. RRCReconfigurationComplete

Remote UE

Relay UE

S-gNB

9. sidelink release

T-gNB

3. HO request

4. HO ACK

6. Access to target cell

8. RRC Reconfiguration

UL and DL data

5. RRCReconfiguration

1. Measurement configuration and reporting

2. HO decision

7. RRCReconfigurationComplete

Fig. 2.  Inter-gNB indirect-to-direct path switching procedure

In inter-gNB indirect-to-direct path switching, source gNB decides to switch the UE to a target cell under the target gNB based on the measurement report, if the indirect path under the source gNB works not well, e.g. the sidelink measurement quantity between the Relay UE and the Remote UE is lower than a threshold. Figure 2 shows the inter-gNB indirect-to-direct path switch procedure which is extended from Rel-17 intra-gNB path switching procedure as defined in clause 16.12.6.1 of TS 38.300.

Step 1. Remote UE triggers the measurements report when the sidelink measurement quantity between Relay UE and Remote UE becomes deteriorated. Remote UE reports the Uu measurements of neighbour cells according the measurement configuration.

Step 2. Source gNB decides to switch the Remote UE to a target cell in the target gNB based on the reporting results.

Step 3. Source gNB sends the HANDOVER REQUEST message to target gNB over the Xn interface/ NG interface, which contains target cell ID, Remote UE’s UE context, and other necessary information. (In the figure, Xn-based path switching is taken as the example)

Step 4. Target gNB determines RRC configuration for the Remote UE at the target cell, and sends it via HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

Step 5. Source gNB sends the RRC configuration to Remote UE.

Step 6. The UE (Remote UE in previous steps) accesses to the target cell in the target gNB.

Step 7. The UE sends the RRC Reconfiguration complete message to target gNB via the Uu link.

Step 8. Source gNB sends RRC Reconfiguration message to the Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB.

Step 9. Sidelink between Remote UE and Relay UE is released.

In the above procedures, the handover preparation procedure in step 3 and step 4 is used to establish the necessary resource on the target gNB for the UE, which is the same as those used for a normal UE’s handover procedure.

Proposal 3: In inter-gNB indirect-to-direct path switching, the HO preparation procedure between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN3 spec impact.

2.2 Inter-gNB direct-to-indirect/indirect-to-indirect path switching

Remote UE

Target Relay UE

S-gNB

8. sidelink establishment if not exist

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

3. HO decision

2. Measurement configuration and reporting

4. HO request

5. HO ACK

9. RRC Reconfiguration complete message

6. RRC Reconfiguration message

UL and DL data

Remote UE

Target Relay UE

S-gNB

8. sidelink establishment if not exist

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

3. HO decision

2. Measurement configuration and reporting

4. HO request

5. HO ACK

9. RRC Reconfiguration complete message

6. RRC Reconfiguration message

UL and DL data

Fig. 3.  Inter-gNB direct-to-indirect path switching procedure

In inter-gNB direct-to-indirect path switching, source gNB determines to switch the Remote UE (For simplicity, it is called Remote UE regardless of whether it is connected to Relay UE or not) to a target Relay under the target gNB when the Uu RSRP of the source cell works not well. Figure 3 shows the inter-gNB direct-to-indirect path switching procedure, which is extended from Rel-17 intra-gNB direct-to-indirect path switch procedure for L2 U2N Remote UE as defined in clause 16.12.6.2 in TS 38.300.

Step 1/2. Remote UE reports the candidate Relay UE(s), which are discovered by discovery procedure. The results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the broadcasted discovery message from Relay UE).

Step 3. Based on the measurement results, the source gNB decides to do the direct-to-indirect path switch. Based on proposal 1 and 2, source gNB decides to switch the Remote UE to a target Relay UE in the target gNB based on the measurement reporting.

Step 4: Source gNB includes the target Relay UE ID and the Relay UE’s serving Cell ID in the HANDOVER REQUEST message.

Proposal 4: In inter-gNB direct to indirect path switching, the source gNB decides the target Relay UE and indicate the target Relay UE ID in HO request message.

Step 5. Target gNB determines RRC configuration for the Remote UE at the target relay UE, and sends it via HANDOVER REQUEST ACKNOWLEDGE to the source gNB.

Step 6. Source gNB sends the RRC configuration to Remote UE.

Step 7. Source gNB sends the relaying configurations to the Relay UE via RRC message.

Step 8. The Remote UE establishes the sidelink RRC connection with the target Relay UE if it does not exist.

Step 9. The Remote UE responses the RRC complete message to gNB via the Relay UE.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

4. HO request

5. HO ACK

11. sidelink release

10. RRC Reconfiguration

UL and DL data

2. Measurement configuration and reporting, contain the target Relay UE info

6. RRC Reconfiguration message

8. sidelink establishment if not exist

9. RRC Reconfiguration complete message

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

7. RRC Reconfiguration for Remote UE

T-gNB

4. HO request

5. HO ACK

11. sidelink release

10. RRC Reconfiguration

UL and DL data

2. Measurement configuration and reporting, contain the target Relay UE info

6. RRC Reconfiguration message

8. sidelink establishment if not exist

9. RRC Reconfiguration complete message

Fig. 4.  Inter-gNB indirect-to-indirect path switching procedure

Figure 4 shows the inter-gNB indirect-to-indirect path switch procedure. In step 1 and step 2, Remote UE triggers to report the candidate Relay UE(s), when the sidelink measurement quantity between Relay UE and Remote UE shows deteriorated link quality. The following steps 3 to 9 are the same as those for inter-gNB direct-to-indirect path switch, and the HO preparation between source gNB and target gNB can refer to the above discussion. The difference is the source gNB needs to release the source relay UE, which is implemented in step 10. Besides, the Remote UE or the source Relay UE can release the PC5 connection in between after receiving the RRC reconfiguration.

Proposal 5: In inter-gNB indirect-to-indirect path switching, the HO preparation between Source gNB and Target gNB is performed in the same way as inter-gNB direct to indirect path switching, without additional RAN3 spec impact.

2.3 Intra-gNB indirect-to-indirect path switching

Figure 5 shows the indirect-to-indirect path switching procedure. Compared to the inter-gNB case, the gNB of Remote UE will select the target Relay UE and determine the RRC configuration in the target cell, so procedures over Xn interface is not necessary. Reusing R17 SL relay configurations is enough in this case.

Observation 2: For intra-gNB indirect-to-indirect path switching, legacy mechanism for R17 SL relay can be reused, without RAN3 spec impact.

Under CU-DU split structure, the procedures in Figure 8.19.4.1-1 and Figure 8.19.4.2-1 of TS 38.401 can be reused for inter-gNB-DU indirect-to-indirect path switching and intra-gNB-DU indirect-to-indirect path switching separately. Compared with the existing direct-to-indirect path switching procedure, the only extra steps are gNB-CU should release the relaying configurations at the source relay UE side.

Proposal 6: For intra-gNB indirect-to-indirect path switching under CU-DU split structure, the legacy mechanism for R17 inter-gNB-DU direct-to-indirect path switching and intra-gNB-DU direct-to-indirect path switching can be reused, without additional RAN3 spec impact.

Proposal 7: Add simple stage-2 description for Intra-gNB indirect-to-indirect path switching by re-using existing stage2 description for direct-to-indirect.

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

9. sidelink release

8. RRC Reconfiguration

5. RRC Reconfiguration for Remote UE

2. Measurement configuration and reporting, contain the target Relay UE info

UL and DL data

4. RRC Reconfiguration message

6. sidelink establishment if not exist

7. RRC Reconfiguration complete message

Remote UE

Relay UE

S-gNB

Target Relay UE

3. HO decision

1. U2N Relay discovery

9. sidelink release

8. RRC Reconfiguration

5. RRC Reconfiguration for Remote UE

2. Measurement configuration and reporting, contain the target Relay UE info

UL and DL data

4. RRC Reconfiguration message

6. sidelink establishment if not exist

7. RRC Reconfiguration complete message

Fig. 5.  Intra-gNB indirect-to-indirect path switching procedure

# 3. Conclusion

Based on the above discussion, we have the following proposes:

Observation 1: In legacy inter-gNB handover procedure, it is source gNB’s KPI to guarantee the handover success of a UE.

Proposal 1: Turn the WA that source gNB selects the target path type (direct or indirect) into agreement and inform RAN2 about the agreement.

Proposal 2: Source gNB selects the target Relay UE if it decides to switch the UE to an indirect path, and sends the target related information to the target gNB.

Inter-gNB indirect-to-direct path switching

Proposal 3: In inter-gNB indirect-to-direct path switching, the HO preparation procedure between source gNB and target gNB is performed in the same way as legacy inter-gNB HO, without additional RAN3 spec impact.

Inter-gNB direct-to-indirect/indirect to indirect path switching

Proposal 4: In inter-gNB direct to indirect path switching, the source gNB decides the target Relay UE and indicate the target Relay UE ID in HO request message.

Proposal 5: In inter-gNB indirect-to-indirect path switching, the HO preparation between Source gNB and Target gNB is performed in the same way as inter-gNB direct to indirect path switching, without additional RAN3 spec impact.

Intra-gNB indirect to indirect path switching

Observation 2: For intra-gNB indirect-to-indirect path switching, legacy mechanism for R17 SL relay can be reused, without RAN3 spec impact.

Proposal 6: For intra-gNB indirect-to-indirect path switching under CU-DU split structure, the legacy mechanism for R17 intra-gNB-DU direct-to-indirect path switching and intra-gNB-DU direct-to-indirect path switching can be reused, without additional RAN3 spec impact.

Proposal 7: Add simple stage-2 description for Intra-gNB indirect-to-indirect path switching by re-using existing stage2 description for direct-to-indirect.

# 4. Reference

- RP-221262, Revised WID on NR sidelink relay enhancements, LG Electronics

- 3GPP TS 38.300, V17.1.0, NR; Overall description; Stage 2

# Text proposals

---------------------------------------------------------------TP for TS 38.401------------------------------------------------------------

### 8.19.4	Service Continuity for L2 U2N relay

#### 8.19.4.X	Intra-gNB-DU switch from indirect to indirect path

The signalling flow for U2N Remote UE switch from indirect to indirect path with gNB-DU change can refer to 8.19.4.1 with the extra steps that gNB-CU releases the relaying configurations at the source relay UE side.

#### 8.19.4.Y	Intra-gNB-DU switch from indirect to indirect path

The signalling flow for U2N Remote UE switch from indirect to indirect path without gNB-DU change can refer to 8.19.4.2 with the extra steps that gNB-CU releases the relaying configurations at the source relay UE side.

------------------------------------------------------------TP for TS 38.401 ends---------------------------------------------------------

---------------------------------------------------------------TP for TS 38.423------------------------------------------------------------

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Target Relay UE ID | O |  | 9.2.3.x | This IE indicates the identity of target relay UE when the source NG-RAN decides to swith the UE to an indirect path. |  |  |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

#### 9.2.3.x	Target Relay UE ID

This IE contains the identity of the target Relay UE when the source NG-RAN decides to switch the UE to an indirect path.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target Relay UE ID | M |  | BIT STRING(SIZE(24)) | The IE contains the SL-SourceIdentity for the target Relay UE, as defined in TS 38.331 [10]. |

-----------------------------------------------------TP for 38.473 ends--------------------------------------------------------------------