3GPP TSG-RAN WG3 Meeting #118	R3-226325

Toulouse France, 14 – 18 November 2022

Title: 	(TPs to TS 38.300, TS 38.413, TS 38.401 BL CRs) Support of Network-Controlled Repeater

Source: 	Huawei

Agenda item:	22.2

Document Type:	Other

# 1. Introduction

The following four potential NCR management solutions in TR 38.867 [1] have been discussed since the SI phase.

- Solution 1: The NCR firstly accesses to RAN and CN as a normal UE, NCR dedicated slice is deployed to enable NCR authorization, and afterwards the NCR validation is used to check the validity of NCR which is done at RAN side.

- Solution 2: The NCR is identified at RAN side and the authorization/validation are performed by local RAN OAM. CN is absent in this solution.

- Solution 3: NCR identification is done at RAN side, and NCR authorization is done at CN side, similar to the handling of IAB-MT.

- Solution 4: NCR authorization is performed at CN side, and the NCR is identified in the authorization process, e.g., based on the UE subscription. The NCR authorization information is sent from the AMF to the gNB.

RAN3 sent the LS to SA3 and SA5 for the feasibility of solution 1 and solution 2 [2]. The LS feedback from SA3 provided the following views [4]:

To SA3 Q1a: Is there any security issue for solution 2 which does not provide Uu security, non-protected NCR indication info and the OAM container in Step 5? Answer to RAN3: Yes. For solution 2, SA3 believes that this information can be tampered due to the lack of Uu security. It exposes the OAM indirectly to attacks over the air interface. To SA3 Q1b: Does SA3 believe that the NCR needs to be securely validated? Any security issue for configuring locally stored information in the gNB in Solution 1? Answer to RAN3: For the 1st question in Q1b, SA3 is not clear about what does "validation" mean. For the 2nd question in Q1b, SA3 cannot provide answers before the security validation related steps in solution1 are clarified. In addition, the feasibility of such additional steps and what kind of information is stored in RAN are also unclear. Further clarification is expected.

On the other hand, as concluded in last meeting, the following aspects may be regarded as issues in RAN3 with contribution driven:

- How the NCR selects a cell belongs to a gNB that supports NCR operation (e.g., OAM configure a list of cell can be accessed by NCR, cell broadcast)?

- Once NCR is authorized, whether gNB-CU indicates to the gNB-DU about NCR authorization?

- Whether the needs for gNB-CU or gNB-DU to configure which cell(s) can be used for NCR device accessing, e.g., due to overload?

In this contribution, we will further discuss the F1AP impact common for all solutions, and the down selection of the solutions, and provide corresponding TPs.

# 2. Discussion

## 2.1 F1AP impact common for all solutions

1) How the NCR selects a cell belongs to a gNB that supports NCR operation (e.g., OAM configure a list of cell can be accessed by NCR, cell broadcast)?

To enable the NCR-MT to access to an NCR capable cell, the NCR supported indicator may be broadcasted in SIB1 by the gNB thus the NCR only selects an NCR capable cell for accessing.

Observation 1: An NCR-support indication in SIB1 is to be introduced, details up to RAN2.

It was agreed in last meeting that “gNB-CU knows whether the connected gNB-DU supports NCR based on OAM configuration”. With that, the gNB-CU also needs to indicate to the gNB-DU about whether a cell of the DU allows NCR access or not, to assist the gNB-DU to provide NCR-support indicator over SIB1. Considering the cells to be barred for UEs and IABs are both indicated in the F1AP: GNB-CU CONFIGURATION UPDATE message, we prefer to also introduce an “NCR Barred” in this F1AP message, to let the NCR capable gNB-DU making decision on whether sending NCR-support indicator over SIB1.

Observation 2: Similar to IAB, the gNB-CU needs to indicate the “NCR Barred” to the gNB-DU about whether a cell of the DU allows NCR access or not.

2) Whether the needs for gNB-CU or gNB-DU to configure which cell(s) can be used for NCR device accessing, e.g., due to overload?

As mentioned in above, we prefer the gNB-CU configures the cells under the NCR capable gNB-DU whether to allow the NCR accessing by F1AP, e.g., by the GNB-CU CONFIGURATION UPDATE message. The gNB-CU can make modifications by re-sending the F1AP message, e.g., due to overload at the gNB-CU side. And it is up to the gNB-DU to broadcast NCR-support over SIB1, and it is up to the gNB-DU on whether take the cell load into account when making such decision.

Observation 3: In case the gNB-CU does not indicate “NCR Barred” to the gNB-DU for a cell, it is up to the gNB-DU to make decision on whether broadcast NCR-support over the SIB1.

3) Once an NCR is authorized, whether gNB-CU indicates to the gNB-DU about NCR authorization?

It was agreed in last meeting that “The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface”, considering that the NCR will be controlled by the gNB-DU, once the NCR is authorized, the gNB-CU has to indicate to the gNB-DU about that the accessing node is authorized as NCR.

Observation 4: Once NCR is authorized, the gNB-CU needs to indicate the gNB-DU about NCR authorization.

Based on the proposals above, we can get the following detailed proposal on the F1AP signalling:

Proposal 1: Introduce NCR Authorized IE in F1AP: UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

Proposal 2: Introduce NCR Barred IE in F1AP: GNB-CU CONFIGURATION UPDATE message.

## 2.2 Down selection of the solutions

According to the LS feedback from SA3 in the introduction, solution 2 has security concern and can be first excluded. Note that solution 2 also not aligned with RAN3 agreement that the NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface.

Observation 6: Solution 2 in NCR management should be excluded.

As for solution 1, SA3 does not get the idea and the motivation of “validation”, and so do we. In our view, considering we have already had agreement on CN authorization, we do not see the need to have a new RAN validation mechanism.

The role of the RAN part in solution 1 is not authorization, but a new “RAN validation” mechanism. Some companies proposed it because they think there is a need to prevent the case in which someone steal an NCR device and re-deploy it to an unexpected place. However, we don’t think such demand really exists. As a network device, the placement of NCR is per-configured by the operator, we can prevent the NCR accessing a wrong cell via, e.g.  gNB broadcasts the NCR support indication in system information (e.g., SIB1), OAM configures to the NCR about which cell(s) it should access to.

We do not think there is a need for RAN3 to consider “anti-stolen”, otherwise, we may have too many things to be worried about, there are so many devices, e.g. DUs, CUs, IAB nodes…

Observation 7: RAN validation is not needed in addition to CN authorization.

Based on the analysis above, we should exclude solution1 and focus on the details of solution 3 and solution 4.

Observation 8: Solution 1 in NCR management should be excluded.

Not that it was clarified in RAN3 #117b e-meeting that RAN3 has the understanding that V2X like Solution 4 has NAS impact. In our understanding that solution 4 assuming all AMF is capable for handling the NCR authorization which seems unrealistic, and transmitting the NCR authorization NAS message to an AMF not capable for NCR may cause unexpected errors. While in solution 3, RAN is aware of the NCR accessing, and it select the NCR capable AMF, which could above the issues for solution 4.

In TR 38.867, the call flow for solution 3 is given as follows:

Figure 1. NCR management solution 3 call flow

In CU DU split architecture, MSG1 and MSG2 in random access are exchanged between NCR and the gNB-DU, while the RRC signalling is exchanged between NCR and the gNB-CU via the gNB-DU. Upon receiving the NCR authorized indication from CN, the gNB-CU will transmit an NCR authorized indication to the gNB-DU, so that the gNB-DU can make further configurations to this authorized NCR. The NCR authorized indication transmitted from the gNB-CU to the gNB-DU is carried by the F1AP: UE CONTEXT SETUP REQUEST message. With that, Figure 1 can be redrawn to Figure 2 as follows to support solution 3 in disaggregated architecture.

Figure 2. NCR management solution 3 call flow in CU-DU split architecture

Based on above analyses, we propose to capture the NCR-MT access procedure in disaggregate architecture in the specifications, therefore we provide the TP to TS 38.401 BL CR to capture it.

Proposal 3: Adopt solution 3 as the solution for NCR management.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

On F1AP impact common for all solutions

Observation 1: An NCR-support indication in SIB1 is to be introduced, details up to RAN2.

Observation 2: Similar to IAB, the gNB-CU needs to indicate the “NCR Barred” to the gNB-DU about whether a cell of the DU allows NCR access or not.

Observation 3: In case the gNB-CU does not indicate “NCR Barred” to the gNB-DU for a cell, it is up to the gNB-DU to make decision on whether broadcast NCR-support over the SIB1.

Observation 4: Once NCR is authorized, the gNB-CU needs to indicate the gNB-DU about NCR authorization.

Proposal 1: Introduce NCR Authorized IE in F1AP: UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages.

Proposal 2: Introduce NCR Barred IE in F1AP: GNB-CU CONFIGURATION UPDATE message.

On Down selection of the solutions

Observation 6: Solution 2 in NCR management should be excluded.

Observation 7: RAN validation is not needed in addition to CN authorization.

Observation 8: Solution 1 in NCR management should be excluded.

Proposal 3: Adopt solution 3 as the solution for NCR management.

It is also proposed to:

- Endorse [6] the F1AP CR (for Proposal 1 and 2) in which the changes are needed regardless of the down selection of solutions.

- Agree the TPs to TS38.300 38.401 and TS38.413 BL CRs in section 5, 6, and 7 to adopt solution 3.

# 4. Reference

- 3GPP TR 38.867, Study on NR network-controlled repeaters.

- R2-2209328,	LS on NCR Solutions (R3-225253; contact: ZTE),	RAN3

- R2-2210294,	Work plan for Network-controlled repeaters	ZTE, Corporation, Sanechips

- S3-223080, Reply LS on NCR Solutions, SA3

- Chairman notes, TSG-RAN WG3 Meeting #117bis-e, E-meeting, October 2022.

- R3-226326 Support of Network-Controlled Repeater, F1AP CR, Huawei

# 5 TP for TS 38.300 BL CR

-------------------------------------------------------------Start of change-------------------------------------------------------------

## 4.x	Network Controlled Repeater

### 4.x.x	NCR Identification and Authorization

NCR identification is done at RAN side, and NCR authorization is done at CN side. The general procedure for NCG identification and authorization is illustrated in Figure 4.x.x-1:

Figure 4.x.x-1. NCR Identification and Authorization

1/2. During NG-C setup procedure, the AMF should inform the gNB whether it supports NCR, by including “NCR-supported” indicator in NG SETUP RESPONSE message.

3.The gNB broadcasts the support of NCR access in the cell. (pending to RAN2 discussion)

4~8. NCR access to the cell and establishes RRC connection and provide the NCR indication in Msg5.

9. The gNB selects an AMF which supports NCR, and forwards the NCR indicator to the AMF.

10. the AMF performs NCR authorization based on the subscription information.

11. in case the NCR is authorized, the AMF provides NCR Authorization indication to the gNB in the Initial Context Setup Request procedure.

-------------------------------------------------------------End of change-------------------------------------------------------------

# 6 TP for 38.401 BL CR

-------------------------------------------------------------Start of change-------------------------------------------------------------

## 8.x	NCR Initial Access

The signalling flow for NCR Initial Access is shown in Figure xx. This procedure includes the NCR identification and authorization.

Figure xx. NCR Initial Access procedure

1. The gNB-CU sets up NG interface with the AMF.

2. When the AMF sends NG SETUP RESPONSE to the gNB-CU, the NCR Supported indicator is included to indicate whether the AMF supports the functions for NCR, e.g., NCR authorization.

3. The NCR sends random access preamble (MSG1) to the gNB-DU to initiate the RACH process.

4. The gNB-DU responses RAR (MSG2) to the NCR.

5. The NCR sends RRCSetupRequest (MSG3) to the gNB-DU.

6. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

7. The gNB-DU sends the RRCSetup message to the UE.

8. The UE sends the RRCSetupComplete (MSG5) message to the gNB-DU, including the NCR Indication info.

9. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

10. The gNB-CU selects an NCR capable AMF to serve the NCR.

11. The gNB-CU sends the INITIAL UE MESSAGE message to the AMF, with the NCR Indication info included. The AMF receiving this message will perform the NCR authorization process.

12. The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU, including an indicator of the NCR authorization status.

13. The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the UE context in the gNB-DU, including the indicator of the NCR authorization status.

-------------------------------------------------------------End of change-------------------------------------------------------------

# 7. TP for TS 38.413 BL CR

-------------------------------------------------------------Start of change-------------------------------------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

AMF	Access and Mobility Management Function

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NCR	Network Controlled Repeater

NID	Network Identifier

NGAP	NG Application Protocol

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

-------------------------------------------------------------Next change-------------------------------------------------------------

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Index to RAT/Frequency Selection Priority IE, if available in the AMF.

If the NAS-PDU IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall pass it transparently towards the UE.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Radio Capability in the UE context;

-	store the received Index to RAT/Frequency Selection Priority in the UE context and use it as defined in TS 23.501 [9];

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context and, if the NG-RAN node is required to activate security for the UE, take this security key into use;

-	if supported, store the received SRVCC Operation Possible in the UE context and use it as defined in TS 23.216 [31];

-	store the received NR V2X Services Authorization information, if supported, in the UE context;

-	store the received LTE V2X Services Authorization information, if supported, in the UE context;

-	store the received NR UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services;

-	store the received LTE UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services;

-	store the received PC5 QoS Parameters, if supported, in the UE context and use it as defined in TS 23.287 [33];

-	store the received Management Based MDT PLMN List information, if supported, in the UE context;

-	if supported, store the received IAB Authorization information in the UE context;

-	store the received 5G ProSe Authorization information in the UE context, if supported, and use it for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services;

-	store the 5G ProSe UE PC5 Aggregate Maximum Bit Rate in the UE context, if supported, and use it for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services;

-	store the 5G ProSe PC5 QoS Parameters, if supported, in the UE context and use it as defined in TS 23.304 [47];

-	store the received NCR Authorization information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

If the PDU Session Resource Setup Request List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The NG-RAN node shall report to the AMF in the INITIAL CONTEXT SETUP RESPONSE message the result for each PDU session resource requested to be setup as defined in the PDU Session Resource Setup procedure.

-------------------------------------------------------------Next change-------------------------------------------------------------

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall

-	if supported, store the received IAB Authorization information in the UE context;

-	if supported, store the received NCR Authorization information in the UE context.

If the Security Key IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall store it and perform AS key re-keying according to TS 33.501 [13].

------------------------------------------------------------Next change-------------------------------------------------------------

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

//skip unchanged part

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports NCR, the AMF shall include the NCR Supported IE in the NG SETUP RESPONSE message.

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| NCR Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the AMF to provide UE Context information changes to the NG-RAN node.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| NCR Authorized | O |  | 9.3.1.xxx |  | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| //skip unchanged part |  |  |  |  |  |  |
| NPN Access Information | O |  | 9.3.3.46 |  | YES | reject |
| RedCap Indication | O |  | 9.3.1.228 |  | YES | ignore |
| NCR Indication | O |  | ENUMERATED (true, ...) | Indication of an NCR. | YES | reject |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.2.6.2	NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| …… | …… | …… | …… | …… | …… | …… |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| NCR Supported | O |  | ENUMERATED (true, ...) | Indication of support for NCR. | YES | ignore |

-------------------------------------------------------------Next change-------------------------------------------------------------

#### 9.3.1.xxx	NCR Authorized

This IE provides information about the authorization status of the NCR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NCR Authorized | M |  | ENUMERATED (authorized, not authorized, ...) | Indicates the NCR authorization status. |

-------------------------------------------------------------Next change-------------------------------------------------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

//skip unchanged part

UL-CP-SecurityInformation,

UnavailableGUAMIList,

URI-address,

UserLocationInformation,

WarningAreaCoordinates,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType,

WUS-Assistance-Information,

RIMInformationTransfer,

NCR-Authorized,

NCR-Supported,

NCRIndication

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-AllowedNSSAI,

id-AMFName,

//skip unchanged part

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer,

id-NCR-Authorized,

id-NCR-Supported,

id-NCRIndication

FROM NGAP-Constants;

//skip unchanged part

-- **************************************************************

--

##### -- Initial Context Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL CONTEXT SETUP REQUEST

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-OldAMF									CRITICALITY reject	TYPE AMFName											PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate						PRESENCE conditional	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListCxtReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListCxtReq			PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE mandatory	}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey										PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional		}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList							PRESENCE optional		}|

{ ID id-UERadioCapability						CRITICALITY ignore	TYPE UERadioCapability									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP										PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional		}|

{ ID id-NAS-PDU									CRITICALITY ignore	TYPE NAS-PDU											PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator						PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest			PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging				CRITICALITY ignore	TYPE UERadioCapabilityForPaging						PRESENCE optional		}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning								PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible							PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY ignore	TYPE IAB-Authorized										PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction			CRITICALITY ignore	TYPE Enhanced-CoverageRestriction					PRESENCE optional		}|

{ ID id-Extended-ConnectedTime					CRITICALITY ignore	TYPE Extended-ConnectedTime							PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo					CRITICALITY ignore	TYPE UE-DifferentiationInfo							PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional 	}|

{ ID id-CEmodeBrestricted						CRITICALITY ignore	TYPE CEmodeBrestricted									PRESENCE optional		}|

{ ID id-UE-UP-CIoT-Support						CRITICALITY ignore	TYPE UE-UP-CIoT-Support									PRESENCE optional		}|

{ ID id-RGLevelWirelineAccessCharacteristics	CRITICALITY ignore	TYPE RGLevelWirelineAccessCharacteristics		PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList										PRESENCE optional		}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo							PRESENCE optional		}|

{ ID id-QMCConfigInfo							CRITICALITY ignore	TYPE QMCConfigInfo										PRESENCE optional		}|

{ ID id-TargetNSSAIInformation					CRITICALITY ignore	TYPE TargetNSSAIInformation							PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList						PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters					PRESENCE optional		}|

{ ID id-NCR-Authorized							CRITICALITY ignore	TYPE NCR-Authorized										PRESENCE optional		},

...

}

//skip unchanged part

-- **************************************************************

--

##### -- UE Context Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

#### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority								PRESENCE optional		}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP									PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY ignore	TYPE UEAggregateMaximumBitRate					PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE optional		}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator					PRESENCE optional		}|

{ ID id-NewAMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-NewGUAMI								CRITICALITY reject	TYPE GUAMI											PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY ignore	TYPE IAB-Authorized									PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters								PRESENCE optional 	}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID							PRESENCE optional		}|

{ ID id-RGLevelWirelineAccessCharacteristics	CRITICALITY ignore	TYPE RGLevelWirelineAccessCharacteristics	PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo						PRESENCE optional		}|

{ ID id-QMCConfigInfo							CRITICALITY ignore	TYPE QMCConfigInfo									PRESENCE optional		}|

{ ID id-QMCDeactivation							CRITICALITY ignore	TYPE QMCDeactivation									PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList					PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized						PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters				PRESENCE optional		}|

{ ID id-NCR-Authorized							CRITICALITY ignore	TYPE NCR-Authorized									PRESENCE optional		},

...

}

//skip unchanged part

-- **************************************************************

--

##### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		}|

{ ID id-IAB-Supported				CRITICALITY ignore	TYPE IAB-Supported				PRESENCE optional		}|

{ ID id-Extended-AMFName			CRITICALITY ignore	TYPE Extended-AMFName			PRESENCE optional		}|

{ ID id-NCR-Supported				CRITICALITY ignore	TYPE NCR-Supported				PRESENCE optional		},

...

}

//skip unchanged part

-- **************************************************************

--

##### -- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialUEMessage-IEs} },

...

}

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU										PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY reject	TYPE UserLocationInformation						PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause					CRITICALITY ignore	TYPE RRCEstablishmentCause						PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI							CRITICALITY reject	TYPE FiveG-S-TMSI									PRESENCE optional		}|

{ ID id-AMFSetID								CRITICALITY ignore	TYPE AMFSetID										PRESENCE optional		}|

{ ID id-UEContextRequest						CRITICALITY ignore	TYPE UEContextRequest								PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional		}|

{ ID id-SelectedPLMNIdentity					CRITICALITY ignore	TYPE PLMNIdentity									PRESENCE optional		}|

{ ID id-IABNodeIndication						CRITICALITY reject	TYPE IABNodeIndication								PRESENCE optional		}|

{ ID id-CEmodeBSupport-Indicator				CRITICALITY reject	TYPE CEmodeBSupport-Indicator					PRESENCE optional		}|

{ ID id-LTEM-Indication							CRITICALITY ignore	TYPE LTEM-Indication								PRESENCE optional		}|

{ ID id-EDT-Session								CRITICALITY ignore	TYPE EDT-Session									PRESENCE optional		}|

{ ID id-AuthenticatedIndication					CRITICALITY ignore	TYPE AuthenticatedIndication						PRESENCE optional		}|

{ ID id-NPN-AccessInformation					CRITICALITY reject	TYPE NPN-AccessInformation						PRESENCE optional		}|

{ ID id-RedCapIndication						CRITICALITY ignore	TYPE RedCapIndication								PRESENCE optional		}|

{ ID id-NCRIndication							CRITICALITY reject	TYPE NCRIndication									PRESENCE optional		},

...

}

-------------------------------------------------------------Next change-------------------------------------------------------------

### 9.4.5	Information Element Definitions

//skip unchanged part

-- N

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

choice-Extensions		ProtocolIE-SingleContainer { {N3IWF-ID-ExtIEs} }

}

N3IWF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NAS-PDU ::= OCTET STRING

NASSecurityParametersFromNGRAN ::= OCTET STRING

NB-IoT-DefaultPagingDRX ::= ENUMERATED {

rf128, rf256, rf512, rf1024,

...

}

NB-IoT-PagingDRX ::= ENUMERATED {

rf32, rf64, rf128, rf256, rf512, rf1024,

...

}

NB-IoT-Paging-eDRXCycle ::= ENUMERATED {

hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NB-IoT-Paging-TimeWindow ::= ENUMERATED {

s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16,

...

}

NB-IoT-Paging-eDRXInfo ::= SEQUENCE {

nB-IoT-Paging-eDRXCycle 	NB-IoT-Paging-eDRXCycle,

nB-IoT-Paging-TimeWindow 	NB-IoT-Paging-TimeWindow 								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NB-IoT-Paging-eDRXInfo-ExtIEs} } OPTIONAL,

...

}

NB-IoT-Paging-eDRXInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NB-IoT-UEPriority ::= INTEGER (0..255, ...)

NCR-Authorized ::= ENUMERATED {

authorized,

not-authorized,

...

}

NCR-Supported ::= ENUMERATED {

true,

...

}

NCRIndication ::= ENUMERATED {

true,

...

}

NetworkInstance ::= INTEGER (1..256, ...)

NewSecurityContextInd ::= ENUMERATED {

true,

...

}

NextHopChainingCount ::= INTEGER (0..7)

NextPagingAreaScope ::= ENUMERATED {

same,

changed,

...

}

-------------------------------------------------------------Next change-------------------------------------------------------------

### 9.4.7	Constant Definitions

//skip unchanged part

id-FiveG-ProSeAuthorized								ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-MBSSessionModificationFailureTransfer				ProtocolIE-ID ::= 348

id-MBSSessionModificationRequestTransfer				ProtocolIE-ID ::= 349

id-MBSSessionModificationResponseTransfer				ProtocolIE-ID ::= 350

id-MBS-QoSFlowToReleaseList								ProtocolIE-ID ::= 351

id-MBS-SessionTNLInfo5GC								ProtocolIE-ID ::= 352

id-TAINSAGSupportList									ProtocolIE-ID ::= 353

id-SourceNodeTNLAddrInfo								ProtocolIE-ID ::= 354

id-NGAPIESupportInformationRequestList					ProtocolIE-ID ::= 355

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-NCR-Authorized										ProtocolIE-ID ::= xxx

id-NCR-Supported										ProtocolIE-ID ::= yyy

id-NCRIndication										ProtocolIE-ID ::= zzz

END

-- ASN1STOP

-------------------------------------------------------------End of change-------------------------------------------------------------