3GPP TSG-RAN WG3 Meeting #118	R3-226437

10th – 18th Nov, 2022

Agenda Item:	13.3

Source: 	Lenovo

Title: 	Mobility enhancements for mobile IAB-node and its served UE

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, enhancement for mobile IAB-node mobility is discussed, including aspects related to information sharing between two logical IAB-DUs and pre-configuration for IAB-DU migration.

- Discussion

Information sharing between logical IAB-DUs

RAN3 117-e [2]: For mobile DU migration, RAN3 to discuss the benefit and whether to support the sharing info of configuration and/ or UE context between two logical DUs in the mobile IAB-node. RAN3 117bis-e [3]: RAN3 to discuss whether and which information can be shared between two logical DUs in case of IAB-DU migration.

In case of inter-donor mobile IAB-DU migration, the mobile IAB-node will support two logical IAB-DUs which have their F1 connections to source and target IAB-donor-CU, respectively. And the served connected UEs perform handover from the cell of the source logical IAB-DU1 to the cell of the target logical IAB-DU2.

As for the served UEs of mobile IAB-node, since source logical IAB-DU1 and target logical IAB-DU2 are co-located physically and the serving mobile IAB-node doesn’t change for UE during the whole full migration procedure or IAB-DU migration procedure, which means the air interface between the UE and the mobile IAB-node may be maintained during the whole full migration procedure or IAB-DU migration procedure. Therefore, in this case, at least some lower layer configurations (RLC, MAC and PHY) can be reused before and after migration between source logical IAB-DU1 and target logical IAB-DU2.

Proposal 1: Lower layer configuration (RLC, MAC and PHY) can be shared between two logical IAB-DUs in case of full migration and IAB-DU migration.

Pre-configuration for IAB-DU migration

RAN3 117-e [2]: RAN3 to discuss whether a mobile IAB node may be configured with multiple configurations, each corresponding to a different target donor, that can be activated upon fulfillment of certain condition(s). The details of the configurations are FFS. RAN3 117bis-e [3]: RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed.

Pre-configuration at the mobile IAB-node may have some advantages to reduce the latency during IAB-DU migration or full migration, especially for the case of regular trajectory or predicable trajectory of mobile IAB-node. In addition, only the static paraments could be considered to be pre-configured at the mobile IAB-node, otherwise, it’s meaningless and the pre-configured parameters may be already outdated by the time they are used which led to extra overhead. As an example, at least the TNL address(es) of second logical IAB-DU and target IAB-donor-CU can be pre-stored at the mobile IAB-node to accelerate the F1 setup between second logical IAB-DU and target IAB-donor-CU.

Proposal 2: The TNL address(es) of second logical IAB-DU and target IAB-donor-CU can be pre-configured at the mobile IAB-node in case of determined trajectory.

In legacy F1 setup procedure, gNB-DU may obtain the TNL address of gNB-CU from OAM. Therefore, for the stationary IAB-node in R16 and R17, the IAB-DU selects the F1-terminating IAB-donor-CU to setup F1 interface based on the OAM configuration.

However, for the mobile IAB-node, the target IAB-donor-CU may be unpredictable due to the random mobility of mobile IAB-node, the legacy configuration from OAM for target TNL address may not be workable. And based on agreement from last meeting, it’s the IAB-donor-CU serving the IAB-DU (F1-termiating IAB-donor-CU) decides whether to execute IAB-DU migration or inter-donor F1 transport migration for the IAB-DU. How does the F1-termiating IAB-donor-CU and target logical IAB-DU to obtain the TNL address of target IAB-donor-CU needs to be further investigated.

Proposal 3: RAN3 to consider how F1-termiating IAB-donor-CU and target logical IAB-DU to obtain the TNL address of target IAB-donor-CU for mobile IAB.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: Lower layer configuration (RLC, MAC and PHY) can be shared between two logical IAB-DUs in case of full migration and IAB-DU migration.

Proposal 2: The TNL address(es) of second logical IAB-DU and target IAB-donor-CU can be pre-configured at the mobile IAB-node in case of determined trajectory.

Proposal 3: RAN3 to consider how F1-termiating IAB-donor-CU and target logical IAB-DU to obtain the TNL address of target IAB-donor-CU for mobile IAB.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 117-e meeting.

- Chairman notes of RAN3 117bis-e meeting.