3GPP TSG-RAN WG3 Meeting #118-e                                                      	R3-226516

Toulouse, Nov 14th – Nov 18th, 2022

Agenda item:	10.2.4

Source:	Qualcomm Incorporated

Title:	SON/MDT enhancements for Non-Public networks

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further how to support SON/MDT for Non-Public Networks (NPN) based on the open issues identified last meeting.

# Discussion

  - Area scope enhancements for PNI-NPN

Introduce a CAG list for MDT area scope

FFS how to introduce CAG list in existing MDT Configuration-NR IE, e.g., new separate Area Scope IE, or to extend existing area scope choice structure

Consider an example where a PLMN includes CAG cells - [1,2,3] and Non-CAG cells - [4,5,6]. The following cases are possible

| Case 1: OAM is interested in collecting from all cells under the PLMN i.e., PLMN wide (cells 1-6) | No enhancement needed (can be supported by including "PLMN wide" over NG and omitting AreaConfig over Uu) |
| --- | --- |
| Case 2: OAM is interested in collecting from a subset of CAG cells and a subset of non-CAG cells (e.g., cells 1, 2, 4, 5) | No enhancement needed (can be supported by providing a list of interested cells (both CAG and non-CAG) over NG and Uu) |
| Case 3: OAM is interested in collecting from all CAG cells ("PNI-NPN wide"), but not from non-CAG cells (cells 1,2,3) | "All CAG cells = TRUE" over NG. No need of providing list of cells |
| Case 4: OAM is interested in collecting from a subset of CAG cells, but not from non-CAG cells (e.g., cells 1,2) | " All CAG cells = FALSE" and a list of CAG cells over NG. |
| Case 5: OAM is interested in collecting from all non-CAG cells, but not from CAG cells (cells 4,5,6) | FFS whether we need a “non-CAG only indicator” |
| Case 6: OAM is interested in collecting from a subset of non-CAG cells, but not from CAG Cells (e.g., cells 4, 5) | FFS whether we need a “non- CAG only indicator” |

For case 3 and case 4, the following enhancement to MDT Configuration-NR is proposed:

9.3.1.169       MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  |
| CHOICE Area Scope of MDT | M |  |  |  |
| >Cell based |  |  |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  |
| >TA based |  |  |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |
| >PLMN wide |  |  | NULL |  |
| >CAG only |  |  |  |  |
| >> All CAG cells | M | BOOLEAN |  |  |
| >> List of CAG cells | O | 1.. <maxnoOfCAGcells> |  |  |

Proposal 1: OAM/AMF should be able to indicate to gNB that it is interested in collecting MDT from only CAG cells and optionally a subset of those CAG cells.

Proposal 2: Enhance Area Scope of MDT in MDT Configuration-NR by including a CHOICE “CAG only”, which can include a BOOLEAN IE “All CAG cells” and optionally a “List of CAG cells”

It is not clear whether case 5 and case 6 needs to be studied and hence the following proposal:

Proposal 3: RAN3 should discuss whether OAM should be able to indicate to AMF/gNB that it is interested in collecting MDT from only non-CAG cells and optionally a subset of those non-CAG cells.

  - Area scope enhancement for SNPN

Whether area scope enhancement for SNPN is needed

Similar to PLMN wide, RAN3 should discuss whether an SNPN wide indication is needed in Area Scope of MDT

Proposal 4: RAN3 should discuss whether an “SNPN wide indication” is needed in Area Scope of MDT

  - Handling SON/MDT reports upon mobility outside registered SNPN

- FFS whether UE performs MDT only in registered SNPN

- FFS whether UE stores or discards the collected SON/MDT measurements upon moving outside the registered SNPN

- Whether there is need to address the potential loss of SON/logged MDT reports upon mobility outside SNPN

We think the UE behavior can be same as it was in case of PLMN i.e., when to record, store, discard MDT measurements and we therefore have the following proposals:

Proposal 5: UE performs MDT only in registered SNPN i.e., UE can keep logging the measurements until the registered SNPN is part of the NPN identity list signaled to the UE. If UE is not in the logging area or registered SNPN is not part of the NPN identity list signaled to the UE, the logging is suspended.

Proposal 6: UE continues to store the collected SON/MDT measurements even upon moving outside the registered SNPN and discards it upon receiving MDT configuration in another PLMN/SNPN.

Proposal 7: There is need to address the potential loss of SON/logged MDT reports upon mobility outside SNPN by having a separate UE memory for SON/MDT reports collected in SNPN. A gNB can proactively retrieve SON/MDT reports in SNPN to avoid the potential loss of SON/MDT reports.

Proposal 8: Send LS to RAN2 based on the agreements to provide support for area scope enhancements and to confirm UE behavior upon mobility outside registered SNPN.

# Conclusion

Proposal 1: OAM/AMF should be able to indicate to gNB that it is interested in collecting MDT from only CAG cells and optionally a subset of those CAG cells.

Proposal 2: Enhance Area Scope of MDT in MDT Configuration-NR by including a CHOICE “CAG only”, which can include a BOOLEAN IE “All CAG cells” and optionally a “List of CAG cells”

Proposal 3: RAN3 should discuss whether OAM should be able to indicate to AMF/gNB that it is interested in collecting MDT from only non-CAG cells and optionally a subset of those non-CAG cells.

Proposal 4: RAN3 should discuss whether an “SNPN wide indication” is needed in Area Scope of MDT

Proposal 5: UE performs MDT only in registered SNPN i.e., UE can keep logging the measurements until the registered SNPN is part of the NPN identity list signaled to the UE. If UE is not in the logging area or registered SNPN is not part of the NPN identity list signaled to the UE, the logging is suspended.

Proposal 6: UE continues to store the collected SON/MDT measurements even upon moving outside the registered SNPN and discards it upon receiving MDT configuration in another PLMN/SNPN.

Proposal 7: There is need to address the potential loss of SON/logged MDT reports upon mobility outside SNPN by having a separate UE memory for SON/MDT reports collected in SNPN. A gNB can proactively retrieve SON/MDT reports in SNPN to avoid the potential loss of SON/MDT reports.

Proposal 8: Send LS to RAN2 based on the agreements to provide support for area scope enhancements and to confirm UE behavior upon mobility outside registered SNPN.

# 4. References