3GPP TSG-RAN WG3 #118																		R3-226557

14th – 18th Nov 2022

Toulouse, France

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for RV-QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#117 meeting, the NR QoE supporting in NR-DC was discussed. The RVQoE related agreements and open issues are captured in chair Note[1] as below:

Configuration of RVQoE

The MN can generate an RVQoE configuration for a UE.

The SN can generate an RVQoE configuration for a UE. FFS whether MN can modify the SN generated RVQoE configuration

The MN can send an RVQoE configuration to the UE.

FFS on the SN can send an RVQoE configuration to the UE.

The issue is acked, and continue the discussion on how to enable that node that provide(s) bearers associated to the RVQoE report(s) participate in RVQoE configuration.

The node that received the QoE configuration from the AMF/OAM can send to the other node the list of available RVQoE metrics.

Reporting of RVQoE

The MN can receive RVQoE reports directly from the UE.

The SN can receive RVQoE reports directly from the UE.

Turn the following WA into an agreement: “UE can send RVQoE report to the MN, the MN then forward the RVQoE report to the SN if needed, and vice versa”.

Agree to ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. FFS on how to ensure.

This contribution will further discuss the supporting on the RV QoE in NR-DC. And provide the view on the WA and open issues .

2		Discussion

2.1 Support for RAN-visible QoE configuration and reporting in NR-DC

In last meeting, we agreed both MN and SN can generate the RV QoE configuration. But some companies think the MN may modify the SN generated RVQoE configuration. Also FFS on the SN can send the RVQoE configuration to UE. There are two possible ways the SN sending its generated RVQoE configuration to UE. One is the SN send to UE directly. Another one is send to MN and MN forward to UE with or without modification. To have more flexible implementation, both ways should be supported. The SN should support sending the RVQoE configuration via SRB3

Proposal 1: SN can send an RVQoE configuration to the UE

If the SN sends the generated RVQoE configuration to UE via MN, whether the MN can modify this configuration should be further analysed. We may have sight on the current R17 RVQoE configuration specified in 38.331[2]. We may find only three parameters including in R17 configuration. For these parameters, we don’t think it is necessary for MN to modify the reporting metrics based on SN generated configuration. But the report periodicity may be change or not to align the two nodes if MN also configures the RVQoE. Also the MN may encode the SN generated configuration in the same RRC message with MN generated configuration.

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17    ENUMERATED {ms120, ms240, ms480, ms640, ms1024}   OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17    INTEGER (1..8)            				    OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                         		    OPTIONAL, -- Need M

...

}

Proposal 2: MN can modify the SN generated RVQoE configuration

Proposal 3: MN may encode RVQoE configuration generated by MN and SN in same RRC message.

After the MN configures the legacy QoE, the MN may configure the RV-QoE for MN. The MN also need send the available metrics and RRC ID to SN. Also if the SN receives the configuration from OAM, the SN should forward the available metrics to MN for the MN RV QoE configuration.

Proposal 4: The node that received the QoE configuration from the AMF/OAM can send to the other node the list of available RVQoE metrics.

As RAN2 agreement in last meeting, SN will configure the RV-QoE with same RRC ID with MN for same QoE reference. How the SN sends the RV-QoE configuration to UE may have several options: 1. Via SRB3; 2. Send to MN, then MN sends to UE. If the option2 is used, the configuration from SN should carry the indication of SN node.  So the UE may differentiate the RV-QoE configuration and report according the RRC ID or SN node indication.

Proposal 5: The MN need send the RRC ID to SN

In last meeting we agree the RV-QoE report can be separately sent to the configuration initialled node directly or send to another node and forwarding to the configuration initialled node. , we agreed that ensure that the RVQoE report is sent to the node(s) that provide the bearer(s) associated to the corresponding RVQoE measurement result in the RVQoE report. But FFS on how to ensure.

One solution is that the configuration and report should carry the indication of the node. And the UE app layer may be informed the configuration initialled node and the APP layer send the report to AS layer carrying the indication of the node

Proposal 6: UE may differentiate the RV-QoE configuration and report according node indication.

Another solution is the UE APP layer will sends the RV QoE report with PDU session ID plus QoS flow ID to AS layer and then the AS layer send the report to the node which the QoS flow is located.

Proposal 7: UE may send the RV-QoE report to corresponding node according QoS flow information

Regarding the open issue, the issue is ACKed, and continues the discussion on how to enable that node that provide(s) bearers associated to the RVQoE report(s) participate in RVQoE configuration. We state the issue as below. When the node configured the RVQoE, the node don’t know what kind application service is carried in its node. The node has no information about which QoS flow located in the node carry what kind application service. So then it cannot select the right metrics for its RVQoE configuration. If resolve this issue, the node should know the mapping between QoS flow and application service. To get this information, we may have two ways, one is from UE. Another is from core network. The UE app layer has the mapping between application service and QoS flow.  Whether the core network can provide this information should check with SA2

Proposal 8: The mapping between QoS flow and application service can be retrieval from UE

Proposal 8b: Check with SA2 whether the mapping between QoS flow and application service can be provided by core network

We may have another solution for the RVQoE configuration. The node may blindly configure the RVQoE with all available metrics. And then the report can be send the node according proposal 6 and proposal 7

Proposal 9: The node may blindly configure the RVQoE with all available metrics whether or not the application service carried in this node.

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: SN can send an RVQoE configuration to the UE

Proposal 2: MN can modify the SN generated RVQoE configuration

Proposal 3: MN may encode RVQoE configuration generated by MN and SN in same RRC message.

Proposal 4: The node that received the QoE configuration from the AMF/OAM can send to the other node the list of available RVQoE metrics.

Proposal 5: The MN need send the RRC ID to SN

Proposal 6: UE may differentiate the RV-QoE configuration and report according node indication.

Proposal 7: UE may send the RV-QoE report to corresponding node according QoS flow information

Proposal 8: The mapping between QoS flow and application service can be retrieval from UE

Proposal 8b: Check with SA2 whether the mapping between QoS flow and application service can be provided by core network

Proposal 9: The node may blindly configure the RVQoE with all available metrics whether or not the application service carried in this node.

4		Reference

- RAN3_117bis-e_agenda_20221018_EOM1

- TS38.331 NR; Radio Resource Control (RRC) protocol specification