3GPP TSG-RAN WG3 #118																				R3-226559

14th – 18th Nov 2022

Toulouse, France

Agenda item:		14.3

Source:				CATT

Title:	Discussion on CHO with SCG and multiple SCGs

Document for:		Discussion and Decision

1		Introduction

In RAN3 #117bis meeting, the CHO with SCG and CHO with multiple SCGs was discussed and the below agreements are captured in chair Notes[1].

There is a need to discuss the avoidance of unnecessary signaling between MN and target SN for CHO + MR-DC. FFS whether it is a correction from Rel-17 or an optimization for Rel-18.

Check with the wording in 2nd round “FFS where to discuss this issue”.

RAN3 discusses whether to avoid unnecessary signaling between MN and target SN for CHO + MR-DC in Rel-18 WI.

Early Data Forwarding optimizations with involvement of the target SCG(s) in Rel-18 will be supported.

CHO with one target SCG:

Focus on optimizing duplicated data forwarding scenario.

There is no issue to identify the same target candidate SN by the source in case direct data forwarding is used on all the forwarding paths/target MNs.

WA: both direct and indirect data forwarding will be supported.

CHO with multiple target SCGs:

Reuse the same design principle for CHO with one target SCG and with multiple target SCGs.

No consensus on whether the duplicated data forwarding issue in case of indirect data forwarding to the same target SN can be solved by some implementation way or any solution is needed.

This contribution will further discuss the supporting of CHO with SCG(s)

2		Discussion

2.1 Avoidance of unnecessary signaling

In last meeting, the avoidance of unnecessary signaling between MN and target SN for CHO + MR-DC was discussed. But only where and how to handle issue was discussed. Also during online session, some companies have concerns on whether the issue is valid. If the issue is confirmed, the technical discussion of this issue may be handled in R18 WI or TEI as it is optimization of the feature.

Proposal 1: Study whether this case is valid: the source SN updates something which does not impact the CPC configuration in the UE

Proposal 2: Discuss the issue on avoiding an unnecessary signaling between MN and target SN in R18

2.2 Optimizing duplicated data forwarding

In R17 CHO with SCG, the target SN already get the information whether the prepared candidate SCG is for same UE via source MN id and source MN UEAP id. So to avoid send/receive multiple same data, the prepared target SN may carry some information to source MN to indicate the source node send once data for the same purpose. RAB3 agree “Focus on optimizing duplicated data forwarding scenario”

The target SN may allocate the same TEID is one of feasible solution. Including this solution, some solutions for this issue can be considering.

Sol 1. The prepared target SN allocates the same TEID for all addition preparations and these TEIDs are passed to the source node for direct forwarding

Sol 2. Add indicator to indicate whether the Data Forwarding Info from target NG-RAN node is already provided and omit the Data Forwarding Info from target NG-RAN node IE for all addition preparations

The sol 1 is only adapted to the direct data forwarding. The sol 2 can be adapted both for the indirect data forwarding and direct data forwarding. So we prefer to select the sol 2.

Proposal 3: Add indicator to indicate whether the Data Forwarding Info from target NG-RAN node is already provided.

2.3 CHO with multiple SCGs

In R17 CHO with SCG, only one target SCG is configured and no execution condition is set for this SCG. The UE just evaluate the execution condition for target MN. In this situation, the radio quality of the SCG cannot be guaranteed. The justification of sub item is descripted as below in [2]:

However, this alone may not be sufficient to optimise MR-DC mobility, as the radio link quality of the conditionally-configured PSCell may not be good enough or may not be the best candidate PSCell when the UE accesses the target PCell, and this may impact the UE throughput. To mitigate this throughput impact, Rel-18 CHO+MRDC can consider CHO including target MCG and multiple candidate SCGs for CPC/CPA.

From above description about multiple candidate SCGs for CPC/CPA, the SCGs may be configured for CPC or CPA. We may derive the source set may be configured as MCG only or MCG+SCG. The target set includes one MCG and multiple SCGs. Also multiple target sets can be configured.

Proposal 4: RAN3 study the cases from source side of CHO: 1. Standard alone; 2. Source MCG+ SCG

There may be two understandings of the scenario on R18 CHO with target MCG and candidate SCGs in target side of CHO, as depicted in the Figure 1.

Figure 1: CHO with candidate MCG and candidate SCG procedure

Understanding 1: NW configures each candidate MCG which is associated with candidate SCGs, and UE will evaluate the candidate MCG and its associated candidate SCG simultaneously.

Understanding 2: NW configures CPAC within the CHO (i.e., NW configures the field conditionalReconfiguration for CPAC in the RRCReconfiguration message contained in condRRCReconfig for CHO), and UE should firstly perform CHO for PCell and then evaluate/execute CPAC for PScell.

Based on the above analysis, it is clear that the benefit of the R18 CHO with target MCG and candidate SCGs is to ensure UE to make the final decision on the target MCG as well as the associated target SCG to be accessed simultaneously, by pre-configuring multiple candidate SCGs associated with each candidate MCG. From this perspective, the R18 CHO with target MCG and candidate SCGs does not include the Understanding 2, i.e., UE firstly evaluate CHO and then to evaluate CPAC after UE successfully access the target PCell, similar like CPAC in CHO which are two independent procedures.

Proposal 5: Specify the below case for R18 CHO with target MCG and candidate SCGs: NW configures each candidate MCG which is associated with candidate SCGs; UE evaluates the candidate MCG and its associated candidate SCG simultaneously.

According the candidate handover principle, the UE should evaluate the execution condition when the condition handover performed. As we know, we didn’t set execution condition for the SCG in R17. In the current multiple SCGs configuration, we should set the execution condition for the multiple candidate SCGs. Then the UE choose the SCG with good radio quality according the execution condition when the handover performed. RAN2 may study which node is responsible to generate the execution condition and how to set execution condition. The execution condition may be set for each candidate SCG separately. Or one execution condition applies all candidate SCGs. E.g. choosing the best radio quality SCG of the candidate SCGs which are better than one threshold. .

Proposal 6: Set the execution condition for the multiple candidate SCGs

If we set the execution condition for the multiple candidate SCGs, the below scenarios maybe happened when the UE perform the CHO.

- MCG execution condition is satisfied but no any SCG execution condition is satisfied

- MCG execution condition is satisfied and one SCG execution condition is satisfied

- MCG execution condition is satisfied and more than one SCG execution condition is satisfied

Against the above scenarios, we should raise one question what the combine execution condition for the CHO with SCGs. We may have two choices.

Option1: Once the MCG execution condition is satisfied, the UE will perform the CHO

Option2: When the MCG execution condition is satisfied and at least one SCG execution condition is satisfied, the UE perform the CHO. As we proposed in proposal 2

Proposal 7: Support one or both options for combine execution condition for CHO with SCG:  1).Once the MCG execution condition is satisfied, the UE will perform the CHO; 2). When the MCG execution condition is satisfied and at least one SCG execution condition is satisfied, the UE perform the CHO.

If we choose option1, the UE has three options to handle the source SCG when the MCG handover to target MCG. 1) First one is removing the SCG and change to MCG only in target MN. 2). Second one is handover to one SCG with best radio quality among the candidate SCGs. 3).Third one is keeping the source SN as is and setup the DC between target MN and source SN. Then the UE perform the CPA or CPC once the execution condition of any one candidate SCG is satisfied.

Proposal 8: Study the options of SCG handling when only MN handover to target: 1).change to MCG; 2). Handover to one of candidate SCGs; 3). Keep source SN.

If we choose option2, it is relative simple. The UE can handover the MCG and SCG together to the target set.

But in the option2, the execution condition of MCG and SCG should not be satisfied at same time. The possible cases are: a). the MCG execution condition is satisfied firstly and one SCG execution condition is satisfied later. b) One or more SCG execution condition is satisfied firstly and the MCG execution condition is satisfied later. The case b) may not be valuable, so just consider the case a). In this case, one issue is how long the UE wait for after MCG execution condition is satisfied. The timer can be set for the waiting. When the timer is expired, the UE may just perform handover as option1. We may wait for RAN2 studying on this issue.

Proposal 9: Wait for RAN2 Studying on the handling of combine execution condition if MCG and SCG handover together

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: Study whether this case is valid: the source SN updates something which does not impact the CPC configuration in the UE

Proposal 2: Discuss the issue on avoiding an unnecessary signaling between MN and target SN in R18

Proposal 3: Add indicator to indicate whether the Data Forwarding Info from target NG-RAN node is already provided.

Proposal 4: RAN3 study the cases from source side of CHO: 1. Standard alone; 2. Source MCG+ SCG

Proposal 5: Specify the below case for R18 CHO with target MCG and candidate SCGs: NW configures each candidate MCG which is associated with candidate SCGs; UE evaluates the candidate MCG and its associated candidate SCG simultaneously.

Proposal 6: Set the execution condition for the multiple candidate SCGs

Proposal 7: Support one or both options for combine execution condition for CHO with SCG:  1).Once the MCG execution condition is satisfied, the UE will perform the CHO; 2). When the MCG execution condition is satisfied and at least one SCG execution condition is satisfied, the UE perform the CHO.

Proposal 8: Study the options of SCG handling when only MN handover to target: 1).change to MCG; 2). Handover to one of candidate SCGs; 3). Keep source SN.

Proposal 9: Wait for RAN2 Studying on the handling of combine execution condition if MCG and SCG handover together

4		Reference

- RAN3_117bis-e_agenda_20221018_EOM1

- R3-221799 Revised WID on Further NR mobility enhancements, MediaTek