3GPP TSG-RAN WG3 Meeting #118                                                                        R3-226710

14 – 18 November, 2022

Agenda Item:	12.2.2.1

Source:	CMCC

Title:	Remaining issues on predicted information

Document for:	Discussion and Decision

1	Introduction

In RAN3#117 e-meeting, it was agreed to define a new procedure over Xn to be used for AI/ML related predicted information. Then in last RAN3#117 e-meeting, related issue was further discussed and following are still ffs [1]:

It is FFS whether a node requesting a prediction includes timing information in order to indicate for which time a prediction is requested.

Whether there is a need for prediction accuracy at a receiving node is FFS.

FFS if also Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity are reported.

In this contribution, we provide considerations on above remaining issues on predicted information.

2	Discussion

2.1	Prediction timing information

The issue whether to indicate timing information when a node requesting a prediction was discussed in last RAN3#117b-e meeting, and the proponents think that predicted information without knowing when the prediction will be calculated cannot be used at the receiving node, especially when this information is used as an input to another ML Model. Furthermore, different timing options were discussed: a) start time and end time, b) time instant, or c) time duration.

Majority companies acknowledged the potential benefits of the timing information. For instance, some companies thought that requesting a prediction from a neighbour without any timing indication is not very useful in the sense that it also affects whether a node can provide the prediction or not. It also leads to different inference outputs (e.g., predicted load in the evening is expected to be less than predicted load on rush hours).

In our understanding, the timing information is helpful when the requesting node ask for the prediction for use in advance, e.g., the request node only needs the resource status related information for the midnight for AI-based load balancing. Another potential scenario is that the node requests the prediction information periodically, e.g., the predicted recommended cell activation/deactivation information at the midnight of everyday for AI-based energy saving.

As for the detailed information, it seems that option a could cover the information of option b+c, and companies did not have strong view on the choice.

Proposal 1: A node requesting a prediction could include timing information in order to indicate for which time a prediction is requested, and the timing information could be one of:

- start time and end time

- start time and duration

2.2	Prediction accuracy

The issue of prediction accuracy was also discussed in related CB [2]. By providing the prediction accuracy to the requesting NG-RAN node, the requesting node could have an understanding of “how good” these predictions are and make use of the prediction result in different ways, e.g., for low-accuracy, the requesting node may just regard it as reference information, and for high-accuracy, the node may take it into consideration when make the decision. Also, the request node may stop requesting the prediction information if the prediction accuracy is unacceptable since sometimes an inaccurate prediction could be even worse than no prediction.

As for the procedure, the receiving node could provide the accuracy information when requested by the requesting node.

Proposal 2: Prediction accuracy could be provided to the requesting node when requested.

2.3	Other Predicted information

It was agreed in last meeting that:

Predicted Resource Status Information reported in the new procedure for AI/ML Related Information can be predicted radio resources, predicted number of active UEs, and predicted number of RRC Connections.

FFS if also Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity are reported.

During the discussion in the CB [2], companies showed views on the following table, and majority companies could agree all of them. We think it could be easy to support them since they are existing measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Predicted Resource status information |  |  |  |  | – |  |
| >Predicted Radio Resource Status | O |  | 9.2.2.50 | Predicted value of the Radio Resource Status IE |  |  |
| >Predicted TNL Capacity Indicator | O |  | 9.2.2.49 | Predicted value of the TNL Capacity Indicator IE |  |  |
| >Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Predicted value of the Composite Available Capacity Group IE |  |  |
| >Predicted Slice Available Capacity | O |  | 9.2.2.55 | Predicted value of the Slice Available Capacity IE |  |  |
| >Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the Number of Active Ues IE |  |  |
| >Predicted RRC Connections | O |  | 9.2.2.56 | Predicted value of the RRC Connections IE |  |  |

Proposal 3: Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity are supported.

3	Summary

In this contribution, we provide considerations on remaining issues on predicted information. Following proposals are made:

Proposal 1: A node requesting a prediction could include timing information in order to indicate for which time a prediction is requested, and the timing information could be one of:

- start time and end time

- start time and duration

Proposal 2: Prediction accuracy could be provided to the requesting node when requested.

Proposal 3: Predicted TNL Capacity Indicator, Predicted Composite Available Capacity Group and Predicted Slice Available Capacity are supported.

4	Reference

[1] 	RAN3_117bis-e_agenda_20221018_EOM1

[2]	R3-225917, Summary of email Discussion on CB: # AIRAN2_XnImpact, Nokia