3GPP TSG-RAN WG3 Meeting #118	R3-226758

Online, 14-18 November 2022

Agenda Item:	23.2

Source:	Ericsson

Title:		Overview of the received Positioning LSes on Rel-18 Positioning

Document for:	Discussion

# Introduction

RAN3 has made the following agreements in last meeting regarding the SL Positioning architecture in RAN3#117-e:

- From RAN3’s perspective, the current NG-RAN positioning architecture can in principle be re-used to support Sidelink Positioning in in-coverage and partial coverage scenarios.

- Whether and how to support SL Positioning and Ranging Service Authorizations signalling to NG-RAN can be investigated by RAN3 during the WI phase, taking into account SA2 decisions on this aspect.

- The potential impacts of SL resource pools, SL positioning measurements, UL CPP measurements, LPHAP, RedCap positioning and positioning Integrity on the RAN3 specifications can be examined during the WI phase, taking into account RAN1/RAN2 decisions.

In RAN3#117bis-e, RAN3 considered that the SI is finished in RAN3 and the following was captured:

- RAN3 will align with RAN1/RAN2 decisions on terminologies for Ranging/Sidelink positioning.

- Currently, there are no known open issues that require RAN3 action during the study phase. Therefore, TUs may not be needed at RAN3#118 (unless there is incoming LS requesting RAN3 action).

RAN3 has received several LSes from other working groups on Positioning. in the following, we provide an overview of the RAN3 aspects from those LSes.

# Discussion

| LS Tdoc | Title | WG | Action | Proposed RAN3 action |
| --- | --- | --- | --- | --- |
| R3-226180 | LS on SRS in multiple cells | RAN2 | RAN2 kindly requests RAN1 to take the above information into account during the continued work for LPHAP and analyze whether SRS positioning configuration for LPHAP across multiple cells is feasible from RAN1’s perspective. | The potential impacts of … LPHAP … on the RAN3 specifications can be examined during the WI phase, taking into account RAN1/RAN2 decisions. No action is needed |
| R3-226183 | Reply LS on Terminology Alignment for Ranging/Sidelink Positioning | RAN2 | For the terminology of Network assisted Operation, the use of ‘assisted’ by SA2 is not aligned with TS38.305, RAN2 asks SA2 to take the above into account | RAN3 will align with RAN1/RAN2 decisions on terminologies for Ranging/Sidelink positioning. No action is needed |
| R3-226164 | LS Out on Positioning Reference Units | SA2 | Question#1: Whether there is additional information that needs to be stored in the core network? If yes, what is the information? Question#2: Whether a PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5? | Q#1: PRU mobility is out of scope, PRU is a manually configured UE with fixed location. PRU support is implementation based and out of 3GPP scope. Q#2: From the RAN3 perspective, a PRU is a UE that may measure downlink PRS or SL PRS. We assume RAN2 will discuss these questions in more details, RAN3 can also feedback with respect to Q#1 that PRU mobility is not in the scope and setting up the PRU is up to OAM/UE implementation. |
| R3-226165 | LS on LPHAP information delivery to RAN | SA2 | SA2 kindly asks RAN WG, is it necessary to provide LPHAP indication to RAN at an earlier time, before positioning procedure is triggered? | Considering that LPHAP is a new use case for Rel-18 Positioning, whether NG-RAN should be made aware of the expected UE behaviour for a UE targeted for LPHAP operations depends on the capability discussion in RAN2. RAN3 can discuss if any NGAP impacts are needed during WI phase. No action is needed |
| R3-226171 | LS on RAN dependency for Ranging/Sidelink Positioning | SA2 | SA2 asks RAN to evaluate the SL/Positioning/Ranging issues and provide feedback. | We note that some aspects may depend on Relay UE U2N framework. RAN3 to provide feedback if any. |
| R3-226175 | Reply LS on Terminology Alignment for Ranging/Sidelink Positioning | RAN1 | From RAN1 perspective, there is no difference between Anchor UE and SL reference UE, and RAN1 assumes that any distinction between Assistant UE and SL reference UE is transparent to RAN1. | RAN3 will align with RAN1/RAN2 decisions on terminologies for Ranging/Sidelink positioning. No action is needed |

Proposal: RAN3 to consider the above overview wen discussing the receiving RAN3 LSes on Rel-18 positioning.

it is proposed to agree on the TP in the annex of this document.

# Text Proposal to TR 38.859

/////////////////////////////////////////////////START OF CHANGES/////////////////////////////////////////////

### 5.2.2	Potential Architecture and Signalling Procedures for Sidelink Positioning

RAN3 considers that current NG-RAN positioning architecture can in principle be re-used to support Sidelink Positioning in in-coverage and partial coverage scenarios.

////////////////////////////////////////////////END OF CHANGES//////////////////////////////////////////////