3GPP TSG-RAN3 Meeting #119	R3-230878

Athens, Greece, 27th February - 3rd March 2023

Agenda Item:	9.2.5

Source:	ZTE (Moderator)

Title:	Summary of Offline Discussion on CB: # 23_UEIdentityIndex

Document for:	Approval

# Introduction

CB: # 23_UEIdentityIndex

- Check the issues raised in above CRs, and merge CRs if needed

- Provide CRs if agreeable

(ZTE - moderator)

Summary of offline disc R3-230878

Please provide your view in draft SoD before Friday.

# For the Chairman’s Notes

<TBD>

# Discussion

R3-230118[1] provides the following observations:

Observation 1:  In current TS 38.304 spec in Rel-17, if eDRX is used for UE in RRC INACTIVE or RRC IDLE, 12 bits of UE identity index (5G-S-TMSI mod 4096) shall be used for PF and PO calculation. However, only 10 bits UE identity index is defined in current F1AP paging message, XnAP RAN paging message, and Core Network Assistance Information for RRC INACTIVE IE over NGAP.

Observation 2:  Anchor gNB does not know whether a UE configured with eDRX will really use eDRX in the paging area (cell also need to broadcast the support for eDRX in System Information). Therefore, both the 10bits and 12bits UE identity index shall be provided together to the paging cell.

R3-230498[5] provides the following observation:

Observation 2: For UE subgroup paging as per TS 38.304 needs 15 bits for eDRX and 13 bits otherwise. But currently only 10 bits are supported over NG, F1 and XN interfaces.

According to the eDRX and UE subgrouping configuration, we have following cases which different bits for UE identity index is used.

Case 1:  DRX is used for paging UE, UE identity index =10bits

Case 2:  eDRX is used for paging UE, UE identity index =12bits

Case 3:  UE_ID based subgrouping and eDRX are used for paging UE, UE identity index =15bits

Case 4:  UE_ID based subgrouping and DRX are used for paging UE, UE identity index =13bits

Based on current TS38.304, seen below.

The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting multicast session activation notification) can monitor PEI using PEI parameters in system information according to the procedure described below

So, Moderator thinks the anchor gNB does not know whether a UE configured with eDRX or supporting PEI paging will really use eDRX and/or UE_ID based subgrouping in the paging area (based on System Information). Therefore, all bits (10bits, 12bits, 13bits, 15bits) information for UE identity index shall be provided together to paging cell.

Q1: Does company agree that all bits (10bits, 12bits, 13bits, 15bits) information for UE identity index shall be provided together to paging cell?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Ericsson | Yes | We agree to follow the same approach as for eMTC, sending all possible bits that will be used for Paging. Therefore [8] cannot work properly. |
| CATT | 10bits and 16bits | The NG-RAN should receive both 10bits (UE identity index) and 16bits (Extended UE Identity Index Value IE). We think gNB is able to “cut” the 16bits UE ID based on the service. For example, a RRC inactive UE configured with CN based subgrouping and eDRX, the gNB which UE located at (paging cell) received 16bits UEID can use the leftmost 12bit for PO/PF calculation, and use the leftmost 15bits for Subgroup ID calculation. We do not need to send all these four bits. Please correct me if misunderstood something. |

Below, different solutions (option1, option2, option3) are provided to extend UE identity index for eDRX and UE_ID based subgrouping.

In R3-230118[1], to extend UE index for eDRX, the following proposal is provided:

- Reuse the existing Extended UE Identity Index Value IE (for eMTC) in NGAP/XnAP for NR (in Core Network Assistance Information for RRC INACTIVE IE and RAN paging message).

- Introduce the new Extended UE Identity Index Value IE (12 bits) in F1 paging message.

Furthermore, the following PEIPS Assistance Information IE exists over current NGAP/F1AP/XnAP for PEI paging.

#### 9.3.1.269	PEIPS Assistance Information

This IE provides the information related to CN paging subgrouping for a particular UE, as specified in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..7, …) |  |

Since moderator thinks Q1 is yes, in order to support extended UE identity index for subgrouping, we can agree the above proposal in In R3-230118[1], and then introduce the new additional PEIPS Assistance Information over NG/Xn/F1 to include specific UE  Identity Index (13bits, and 15bits) for UE_ID based subgrouping as following.

#### x.x.x.x 	Additional PEIPS Assistance Information

This IE provides the information related to UE ID based paging subgrouping for a particular UE, as specified in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | BIT STRING (SIZE(13)) |  |
| UE Identity Index Value for eDRX | O |  | BIT STRING (SIZE(15)) | if eDRX is used for the UE |

(the existing PEIPS Assistance Information is only present in the case of CN subgroup ID is provided by CN, and the CN Subgroup ID IE is mandatory. If the above additional UE Index information needs to be added in existing PEIPS Assistance Information IE, then the CN Subgroup ID IE should also be changed to optional, this is a NBC correction).

Option 1:

- For eDRX, to reuse the existing Extended UE Identity Index Value IE (12bits) over NG/Xn, and introduce a new Extended UE Identity Index Value IE (12 bits) over F1;

- For UE_ID based subgrouping, introduce the new Additional PEIPS Assistance Information IE over NG/Xn/F1 to include all specific UE Identity Index (13bits, and 15bits) for UE_ID based subgrouping.

In R3-230559, R3-230560, R3-230561 [6,7,8], the existing Extended UE Identity Index Value IE is reused for UE_ID based subgrouping over NG/Xn, and a new UE Identity Index Value for PEI Subgrouping IE (15bits) is introduced over F1. During the online discussion, some companies think [6, 7, 8] can be merged into [2, 3, 4].  Then the following option2 may be a merged solution.

Option 2:

- For eDRX and UE_ID based subgrouping, to reuse the existing Extended UE Identity Index Value IE over NG/Xn (12, 13, 15 bits).

- For eDRX,  introduce a new Extended UE Identity Index Value IE (12 bits) over F1;

- For UE_ID based subgrouping, introduce the new UE Identity Index Value for PEI Subgrouping IE (13bits, and 15bits) over F1.

However, if Q1 is answered yes, option2 would not actually work, because reusing just one IE is not enough for the need for transferring multiple UE indexes.

In R3-230498 [5], the following proposal is provided:

Proposal 1: Provide 5G-S-TMSI as an optional IE over NG, F1 and XN interfaces for Paging calculation for eDRX and UE subgroup paging.

Option 3: Provide 5G-S-TMSI as an optional IE over NG, F1 and XN interfaces for Paging calculation for eDRX and UE subgroup paging.

Note that op3 is not consistent with our previous principle of avoiding to transfer 5G-S-TMSI over the Xn/F1 interface, it leads to unnecessary work load for UE index calculations in the paging cell, and in addition, it also makes the existing defined UE index IE useless.

Q2: Please company provide view on the preference of the solutions. If the company has other solutions, please provide them.

| Companies | OP1/OP2/OP3 | Comments |
| --- | --- | --- |
| ZTE | OP1 | We suggest to decouple these two issues from different two WID, and resolve two issues one by one. In this meeting, we can firstly resolve the eDRX issue from redcap WID, and leave time to consider how to resolve UE_ID based subgrouping. |
| Ericsson | OP1 FFS | We are fine progressing with the UE ID index for eDRX and postponing the UE_ID based subgrouping issue for next meeting to avoid colliding CRs. |
| CATT | Option 3 | Extend UEID is used in both redcap and power saving. #### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame as specified in TS 36.304 [29], or calculate Paging occasion for eDRX and UE_ID based subgrouping ID as specified in TS38.304 [12]. |

If the answer of Q2 is Option1,

Q3: Does company agree the CRs provided in [2, 3, and 4] only for eDRX, and agree to postpone UE subgroupping configuration to next meeting?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | We need time to consider how to enhance the existing PEIPS Assistance Information IE to support UE subgrouping configuration. |
| Ericsson | Yes | Agree with moderator’s proposed way forward |
| CATT | See comments above |  |

# References

- R3-230118 Consideration on the UE identity index for paging RedCap UE (ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Huawei)

- R3-230119 Correction on the UE identity index for paging RedCap UE to TS38.413 (ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Huawei)

- R3-230120 Correction on the UE identity index for paging RedCap UE to TS38.423 (ZTE, Nokia, Nokia Shanghai Bell, Ericsson, Huawei)

- R3-230292 Correction on the UE identity index for paging RedCap UE to TS38.473 (Huawei, ZTE, Nokia, Nokia Shanghai Bell, Ericsson)

- R3-230498 UE_ID Index Values for eDRX (Qualcomm Incorporated)

- R3-230559 Correction on UE_ID based subgrouping for UE power saving (Huawei, Deutsche Telekom, CATT)

- R3-230560 Correction on UE_ID based subgrouping for UE power saving (Huawei, Deutsche Telekom, CATT)

- R3-230561 Correction on UE_ID based subgrouping for UE power saving (Huawei, Deutsche Telekom, CATT)