3GPP TSG-RAN WG3 Meeting #119bis-e                                                      	R3-231348

E-meeting, April 17th – April 26th, 2023

Agenda item:	24.2

Source:	Qualcomm Incorporated

Title:	Paging enhancements and beam level (de)activation

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss further details related to the following objective in the Rel-18 NES WID (RP-223450) based on the discussion in the last meeting:

Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3].

# Discussion

## Paging enhancements

### Benefits of paging in a restricted area (e.g., certain SSBs)

In the last meeting, some companies were not convinced on the benefits of paging in a restricted area (e.g., certain SSBs) for network energy saving purposes. The claim was that the benefits are small, and it could only save up to “2 bytes per beam per UE”.

Firstly, we don’t think that the paging message is only 2 bytes. This is the structure of Paging message as per 38.331

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700                                                  OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17                                                     OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPageGroup-r17                    INTEGER ::= 32      -- Maximum number of paging groups in a paging message

NG-5G-S-TMSI ::=                         BIT STRING (SIZE (48))

I-RNTI-Value ::=                        BIT STRING (SIZE(40))

Even without considering the Rel-17 feature of paging subgrouping, the paging message takes 5-6 bytes per UE (depending on whether NG-5G-S-TMSI or I-RNTI is used as the PagingUE-Identity).

Observation 1: Paging message takes at least 5-6 bytes per UE (depending on whether NG-5G-S-TMSI or I-RNTI is used as the PagingUE-Identity)

Further, we think that the network energy savings by paging in a restricted area (e.g., certain SSBs) (instead of paging blindly over the whole cell) comes from the fact that the paging channel need not be sent in those SSBs where there is a low probability of a UE being present. Currently this can be done via implementation e.g., a gNB can choose to activate only certain SSBs in a cell and send paging channel in only those SSBs.

Observation 2: The network energy savings by paging in a restricted area (e.g., certain SSBs) instead of paging blindly over the whole cell comes from the fact that the paging channel need not be sent in those SSBs where there is a low probability of a UE being present

Also, we would like to remind that the focus of Rel-18 NES WI is for low/medium loads and not high loads. If the cell is already quite active (e.g., high loads), it might be true that sending the paging control channel in every beam for every UE might have negligible impact on the power consumption. If the cell is underutilized (e.g., only sending SSBs/SIB/paging and monitoring for RACH), then dropping the paging control channel for certain beams could be rewarding because it may lead to longer/deeper sleep.

Observation 3: If the cell is already quite active (e.g., high loads), it might be true that sending the paging channel in every beam for every UE might have negligible impact on the power consumption

Observation 4: The focus of Rel-18 NES WI is for low/medium loads and not high loads.

Observation 5: If the cell is underutilized (e.g., only sending SSBs/SIB/paging and monitoring for RACH), then dropping the paging control channel for certain beams could be rewarding because it may lead to longer/deeper sleep.

There was also an offline discussion last time mentioning that it is a requirement to send the paging channel in the entire coverage of the cell and we confirm the same from the following clause from TS 38.300:

Paging Channel (PCH) characterized by:

-    requirement to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances;

What the above clause means that the paging channel should have the same coverage as SSBs. For example, a gNB (via implementation) today can choose to activate a subset of SSBs within the cell and therefore need to send the paging channel in only those SSBs.

Observation 6: There is a requirement in TS 38.300 that the “Paging channel needs to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances”. This means that the paging channel should have the same coverage as SSBs.

As mentioned before, this can be done today via implementation e.g., the gNB can remember the last served SSB(s), activate only those SSB and send paging only in those SSBs. In Rel-18, we can provide standardized mechanisms (e.g., providing last served SSB(s) as assistance) to enable this network energy saving for low/medium loads

Proposal 1: RAN3 should support mechanisms (e.g., assistance information over F1/NG/Xn) to enable paging in a restricted area (e.g., certain SSBs) for network energy savings in low/medium loads

### How the gNB/AMF knows about the UE mobility

There was a discussion last time on how the gNB/AMF knows about the UE mobility (stationary/mobile or exact speed information) and thereby can decide which UEs to target for applying this mechanism of restricted paging in certain SSBs. We discuss below different mechanisms by which the gNB/AMF is aware of the UE mobility information.

Way 1: gNB can know the exact UE speed if sensor measurements (measUeSpeed) are configured as part of immediate MDT measurements

Sensor-NameList-r16 ::= SEQUENCE {

measUncomBarPre-r16     ENUMERATED {true}            OPTIONAL,  -- Need R

measUeSpeed             ENUMERATED {true}            OPTIONAL,  -- Need R

measUeOrientation       ENUMERATED {true}            OPTIONAL   -- Need R

}

Sensor-NameList field descriptions

measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined in TS 37.355 [49].

measUeSpeed If configured, the UE reports the UE speed measurement as defined in TS 37.355 [49].

measUeOrientation If configured, the UE reports the UE orientation information as defined in TS 37.355 [49].

Observation 7: A gNB can know the exact UE speed if sensor measurements (measUeSpeed) are configured as part of immediate MDT measurements

Way 2: AMF/gNB can know whether the UE is expected to be stationary or mobile via the "Expected UE behavior" parameters provisioned from AF (via NEF) or stored at UDM as part of the UE's subscription

9.3.1.93          Expected UE Behaviour

This IE indicates the behaviour of a UE with predictable activity and/or mobility behaviour, to assist the NG-RAN node in e.g. determining the optimum RRC connection time or helping with the RRC_INACTIVE state transition and RNA configuration (e.g. size and shape of the RNA).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected UE Activity Behaviour | O |  | 9.3.1.94 |  |
| Expected HO Interval | O |  | ENUMERATED (sec15, sec30, sec60, sec90, sec120, sec180, long-time, …) | Indicates the expected time interval between inter NG-RAN node handovers. If "long-time" is included, the interval between inter NG-RAN node handovers is expected to be longer than 180 seconds. |
| Expected UE Mobility | O |  | ENUMERATED (stationary, mobile, ...) | Indicates whether the UE is expected to be stationary or mobile. |
| Expected UE Moving Trajectory |  | 0..1 |  | Indicates the UE's expected geographical movement. |
| >Expected UE Moving Trajectory Item |  | 1..<maxnoofCellsUEMovingTrajectory> |  | Includes list of visited and non-visited cells, where visited cells are listed in the order the UE visited them with the most recent cell being the first in the list. Non-visited cells are included immediately after the visited cell they are associated with. |
| >>NG-RAN CGI | M |  | 9.3.1.73 |  |
| >>Time Stayed in Cell | O |  | INTEGER (0..4095) | Included for visited cells and indicates the time a UE stayed in a cell in seconds. If the UE stays in a cell more than 4095 seconds, this IE is set to 4095. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsUEMovingTrajectory | Maximum no. of cells of UE moving trajectory. Value is 16. |

Observation 8: AMF knows whether the UE is expected to be stationary via the "Expected UE behavior" parameters provisioned from AF (via NEF) or stored at UDM as part of the UE's subscription

Observation 9: A gNB can also know whether the UE is expected to be stationary via the "Expected UE behavior" IE indicated from AMF in "Core Network Assistance Information for RRC INACTIVE" or "CN Assisted RAN Parameters Tuning"

Proposal 2: There are existing mechanisms for the gNB/AMF to be aware of UE mobility (i.e., whether UE is expected to be stationary/mobile or the exact UE speed) and there are no further enhancements needed to help the AMF/gNB to identify UE mobility for the purpose of restricted paging in certain SSBs

### Paging enhancements for UE in RRC_INACTIVE

In order to restrict the paging to certain SSBs and not page over the entire cell/RNA, some kind of assistance would be needed at the gNB-CU for RAN paging so that they know where the UE was last located (at beam level granularity) before it went into RRC_INACTIVE.

One idea could be that the gNB-DU can indicate the SSB(s) that served the UE before it went into RRC_INACTIVE to gNB-CU. Once the gNB-CU decides to page the UE back to RRC_CONNECTED, it can decide to activate only those SSB(s) and send paging channel in only those SSB(s).

There was also a discussion last time on whether to support this restricted paging mechanism to only stationary UEs or can it also apply to limited mobility UEs.

The motivation to start with stationary UEs was due to the fact that the last serving SSB might not change much for a stationary UE and the gNB-DU would just need to provide the last served SSB as assistance and not a list of last few served SSBs. But we would like to highlight that even a stationary UE can also be served by different beams depending on the UE's orientation. Therefore, only indicating the last served SSB before the UE went into RRC_INACTIVE might not be always useful for a stationary UE.

Therefore, we think that there are benefits in indicating a list of last few served SSBs and not just the last served SSB as assistance before going to RRC_INACTIVE.

Observation 10: A stationary UE can also be served by different beams depending on the UE's orientation. Therefore, only indicating the last served SSB before a stationary UE went into RRC_INACTIVE might not be always useful

Proposal 3: When the UE is released to RRC_INACTIVE, the gNB-DU can indicate a list of last few served SSBs as assistance to the gNB-CU in UE CONTEXT RELEASE COMPLETE

Proposal 4: Based on the assistance received from gNB-DU, gNB-CU can decide to activate only those SSBs and send paging channel in only those SSBs to enable network energy savings

If we agree to provide a list of last served SSBs as assistance, this can work for both stationary UEs (which can still be served by different beams due to different orientations) or for low mobility UEs. We therefore have the following proposal:

Proposal 5: RAN3 should support paging enhancement mechanisms for both stationary UEs and limited mobility UEs

### Paging enhancements for UEs in RRC_IDLE

We think that a similar assistance mechanism described in section 2.3 can also work for UEs in RRC_IDLE (i.e., CN-based paging). We therefore make the following proposal:

Proposal 6: RAN3 should support paging enhancement mechanisms for UEs in both RRC_INACTIVE and RRC_IDLE

gNB already provides “Information on Recommended Cells and RAN Nodes” to AMF in UE Context Release Complete as assistance for idle mode paging as shown below

9.3.1.100       Information on Recommended Cells and RAN Nodes for Paging

This IE provides information on recommended cells and NG-RAN nodes for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Cells for Paging | M |  | 9.3.1.71 |  |
| Recommended RAN Nodes for Paging | M |  | 9.3.1.101 |  |

Observation 11: gNB already provides “Information on Recommended Cells and RAN Nodes” to AMF in UE Context Release Complete as assistance for idle mode paging.

gNB can further indicate a list of last serving SSBs (as a transparent container) to AMF in UE context Release complete which can be used as further assistance for idle mode paging. During idle mode paging, AMF sends the list of SSBs (provided during UE context release) back to gNB-CU. Based on the assistance received from AMF, gNB-CU can decide to activate only those SSBs and send paging channel in only those SSBs to enable network energy savings

Proposal 7: gNB can indicate a list of last serving SSBs (as a transparent container) to AMF in UE CONTEXT RELEASE COMPLETE message over Xn

Proposal 8: During idle mode paging, AMF sends the list of SSBs (provided by old gNB in UE CONTEXT RELEASE COMPLETE) back to gNB-CU. Based on the assistance received from AMF, gNB-CU can decide to activate only those SSBs and send paging channel in only those SSBs to enable network energy savings

### Xn Paging enhancements

A gNB which receives a Paging message from neighboring gNB over Xn has to blindly page the entire RAN Paging area (can be up to 32 cells) which could waste network energy

9.2.3.38          RAN Paging Area

The RAN Paging Area IE defines the paging area within a PLMN for RAN paging a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.4 |  |
| CHOICE RAN Paging Area Choice | M |  |  |  |
| >Cell List |  |  |  |  |
| >>Cell List Item |  | 1 .. < maxnoofCellsinRNA> |  |  |
| >>>NG-RAN Cell Identity | M |  | 9.2.2.9 | In this version of the specification, the RAN paging area should contain NG-RAN cells of the same RAT type. |
| >RAN Area ID List |  |  |  |  |
| >>RAN Area ID List Item |  | 1 .. <maxnoofRanAreasinRNA> |  |  |
| >>>RAN Area ID | M |  | 9.2.3.39 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinRNA | Maximum no. of cells in a RAN notification area. Value is 32. |
| maxnoofRanAreasinRNA | Maximum no. of RAN area IDs in a RAN notification area. Value is 16. |

Observation 12: A gNB which receives a Paging message from neighboring gNB over Xn has to blindly page the entire RAN Paging area (can be up to 32 cells) which could waste network energy

If a gNB can provide a list of recommended cells (belonging to neighbor gNB) for future RAN-paging, then it can help the neighboring gNB to page in a restricted set of cells instead of paging over the whole RAN paging area. The gNB can come up with this recommendation based on RRM of neighbor cells or other implementation-based mechanism. We therefore think that a gNB can send a list of recommended cells to its neighboring gNBs, so that a more efficient/sequential RAN Paging can be done, thereby saving network energy

Observation 13: If a gNB can provide a list of recommended cells (belonging to neighbor gNB) for future RAN-paging, then it can help the neighboring gNB to page in a restricted set of cells instead of paging over the whole RAN paging area

Proposal 9:  To assist in RAN Paging, a gNB can send a list of recommended cells to its neighboring gNBs so that a more efficient/sequential RAN Paging can be done, thereby saving network energy

To provide further granularity, a gNB can also provide a list of recommended beams (belonging to the neighbor gNB) as assistance for RAN Paging

Proposal 10: To assist in RAN Paging, a gNB can further send a list of recommended beams (belonging to neighbor gNB) to restrict the RAN Paging to specific SSBs within a cell

## Inter-node beam activation

The TR 38.864 captures the following text on inter-node beam activation:

This mechanism allows an NG-RAN node to request a neighboring NG-RAN node to switch on certain SSB beams which have been deactivated e.g., in the capacity layer. With this mechanism, the NG-RAN node (e.g., in the coverage-layer) can request the re-activation of SSB beams that are deactivated before.

The motivation behind beam level activation for network energy savings is that an NG-RAN node can request the activation of some specific SSBs of one cell instead of the activating the whole cell. The SSBs to be activated can be determined based on OAM configuration or via NG-RAN node implementation, e.g., by considering the SSB beam measurement reports from UE and SSB load etc.

Observation 14: Currently an NG-RAN node can only achieve cell level activation and there is no mechanism to activate only specific beams of one cell

RAN3 therefore made the following agreement last meeting:

Support beam level activation over Xn and F1.

Last meeting, a majority of companies thought that the Cell Activation procedure can be reused for beam level activation as well. We therefore propose the following:

Proposal 11: Enhance the existing cell activation procedures in Xn and F1 for beam level activation i.e., there is no need to define a new Xn/F1 message

Proposal 12: For each NR cell to be activated in CELL ACTIVATION REQUEST, a gNB1 should be able to indicate to gNB2 whether to activate the whole NR cell or indicate the list of SSBs to be activated within the NR cell without activating the whole NR cell

Proposal 13: In CELL ACTIVATION RESPONSE, the gNB2 should respond to gNB1 whether the list of SSBs that were requested to be activated got activated

Proposal 14: In CELL ACTIVATION FAILURE, the gNB2 should respond to gNB1 whether the list of SSBs that were requested to be activated didn’t get activated along with an appropriate cause.

# Conclusion

Paging enhancements

Benefits of paging in a restricted area (e.g., certain SSBs)

Observation 1: Paging message takes at least 5-6 bytes per UE (depending on whether NG-5G-S-TMSI or I-RNTI is used as the PagingUE-Identity)

Observation 2: The network energy savings by paging in a restricted area (e.g., certain SSBs) instead of paging blindly over the whole cell comes from the fact that the paging channel need not be sent in those SSBs where there is a low probability of a UE being present

Observation 3: If the cell is already quite active (e.g., high loads), it might be true that sending the paging channel in every beam for every UE might have negligible impact on the power consumption

Observation 4: The focus of Rel-18 NES WI is for low/medium loads and not high loads.

Observation 5: If the cell is underutilized (e.g., only sending SSBs/SIB/paging and monitoring for RACH), then dropping the paging channel for certain beams could be rewarding because it may lead to longer/deeper sleep.

Observation 6: There is a requirement in TS 38.300 that the “Paging channel needs to be broadcast in the entire coverage area of the cell, either as a single message or by beamforming different BCH instances”. This just means that the paging channel should have the same coverage as SSBs.

Proposal 1: RAN3 should support mechanisms (e.g., assistance information over F1/NG/Xn) to enable paging in a restricted area (e.g., certain SSBs) for network energy savings in low/medium loads

How the gNB/AMF knows about the UE mobility

Observation 7: A gNB can know the exact UE speed if sensor measurements (measUeSpeed) are configured as part of immediate MDT measurements

Observation 8: AMF knows whether the UE is expected to be stationary via the "Expected UE behavior" parameters provisioned from AF (via NEF) or stored at UDM as part of the UE's subscription

Observation 9: A gNB can also know whether the UE is expected to be stationary via the "Expected UE behavior" IE indicated from AMF in "Core Network Assistance Information for RRC INACTIVE" or "CN Assisted RAN Parameters Tuning"

Proposal 2: There are existing mechanisms for the gNB/AMF to be aware of UE mobility (i.e., whether UE is expected to be stationary/mobile or the exact UE speed) and there are no further enhancements needed to help the AMF/gNB to identify UE mobility for the purpose of restricted paging in certain SSBs

Paging enhancements for UE in RRC_INACTIVE

Observation 10: A stationary UE can also be served by different beams depending on the UE's orientation. Therefore, only indicating the last served SSB before a stationary UE went into RRC_INACTIVE might not be always useful

Proposal 3: When the UE is released to RRC_INACTIVE, the gNB-DU can indicate a list of last few served SSBs as assistance to the gNB-CU in UE CONTEXT RELEASE COMPLETE

Proposal 4: Based on the assistance received from gNB-DU, gNB-CU can decide to activate only those SSBs and send paging channel in only those SSBs to enable network energy savings

Proposal 5: RAN3 should support paging enhancement mechanisms for both stationary UEs and limited mobility UEs

Paging enhancements for UEs in RRC_IDLE

Proposal 6: RAN3 should support paging enhancement mechanisms for UEs in both RRC_INACTIVE and RRC_IDLE

Observation 11: gNB already provides “Information on Recommended Cells and RAN Nodes” to AMF in UE Context Release Complete as assistance for idle mode paging.

Proposal 7: gNB can indicate a list of last serving SSBs (as a transparent container) to AMF in UE CONTEXT RELEASE COMPLETE message over Xn

Proposal 8: During idle mode paging, AMF sends the list of SSBs (provided by old gNB in UE CONTEXT RELEASE COMPLETE) back to gNB-CU. Based on the assistance received from AMF, gNB-CU can decide to activate only those SSBs and send paging channel in only those SSBs to enable network energy savings

Xn Paging enhancements

Observation 12: A gNB which receives a Paging message from neighboring gNB over Xn has to blindly page the entire RAN Paging area (can be up to 32 cells) which could waste network energy

Observation 13: If a gNB can provide a list of recommended cells (belonging to neighbor gNB) for future RAN-paging, then it can help the neighboring gNB to page in a restricted set of cells instead of paging over the whole RAN paging area

Proposal 9:  To assist in RAN Paging, a gNB can send a list of recommended cells to its neighboring gNBs so that a more efficient/sequential RAN Paging can be done, thereby saving network energy

Proposal 10: To assist in RAN Paging, a gNB can further send a list of recommended beams (belonging to neighbor gNB) to restrict the RAN Paging to specific SSBs within a cell

Inter-node beam activation

Observation 14: Currently an NG-RAN node can only achieve cell level activation and there is no mechanism to activate only specific beams of one cell

Proposal 11: Enhance the existing cell activation procedures in Xn and F1 for beam level activation i.e., there is no need to define a new Xn/F1 message

Proposal 12: For each NR cell to be activated in CELL ACTIVATION REQUEST, a gNB1 should be able to indicate to gNB2 whether to activate the whole NR cell or indicate the list of SSBs to be activated within the NR cell without activating the whole NR cell

Proposal 13: In CELL ACTIVATION RESPONSE, the gNB2 should respond to gNB1 whether the list of SSBs that were requested to be activated got activated

Proposal 14: In CELL ACTIVATION FAILURE, the gNB2 should respond to gNB1 whether the list of SSBs that were requested to be activated didn’t get activated along with an appropriate cause.

# 4. References