3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231499

# Online, 17th – 26th April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0035 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on support UP based QoS notification control for XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To support UP based QoS notification control for XRM, PDU session user plane protocol shall support carrying the notification indicating that "GFBR can no longer (or can again) be guaranteed" via GTP-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use a New IE Flag to extend UL PDU SESSION INFORMATION frame to carry the notification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UP based QoS notification control is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.2, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE STARTS

5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  |  |  | QoS Notification Cause Ind | D1 UL PDCPDelay Result Ind | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0)

Bit 1 of New IE flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the QoS Notofication Cause Ind is present (1) or not (0).

NEXT CHANGE

5.5.3	Coding of information elements in frames

5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

5.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

5.5.3.3	QoS Flow Identifier (QFI)

Description: When present this parameter indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

5.5.3.4	Reflective QoS Indicator (RQI)

Description: This parameter indicates activation of the reflective QoS towards the UE for the transferred packet as described in clause 5.4.1.1. It is used only in the downlink direction. If RQA (Reflective QoS Activation) has not been configured for the involved QoS flow, the RQI shall be ignored by the NG-RAN node.

Value range: {0= Reflective QoS activation not triggered, 1= Reflective QoS activation triggered}.

Field length: 1 bit.

5.5.3.5	Padding

Description: The padding is included at the end of the frame to ensure that the PDU Session user plane protocol PDU length (including padding and the future extension) is (n*4– 2) octets, where n is a positive integer. If there is any future extension, the padding should be added after the future extensions.

Field Length: 0–3 octets.

5.5.3.6	Paging Policy Presence (PPP)

Description: This parameter indicates the presence of the Paging Policy Indicator (PPI).

Value range: {0= Paging Policy Indicator not present, 1= Paging Policy Indicator present}.

Field length: 1 bit.

5.5.3.7	Paging Policy Indicator (PPI)

Description: When present, the Paging Policy Indicator is used for paging policy differentiation (see details in 3GPP TS 23.501 [5]). This field applies to PDU sessions of IP type.

Value range: {0..23-1}.

Field length: 3 bits.

5.5.3.8	QoS Monitoring Packet (QMP)

Description: This parameter indicates that the transferred packet is used for QoS monitoring as described in clause 5.4.1.1 and clause 5.4.2.1. This parameter also indicates the presence of the DL Sending Time Stamp in the DL PDU Session Information frame and the presence of the DL Sending Time Stamp Repeated, the DL Receiving Time Stamp, the UL Sending Time Stamp in the UL PDU Session Information frame. If QoS monitoring has not been configured for the involved QoS flow, the QMP shall be ignored by the NG-RAN node.

Value range: {0= not used for QoS monitoring, 1= used for QoS monitoring}.

Field length: 1 bit.

5.5.3.9	DL Sending Time Stamp

Description: This field indicates the time when the UPF sends the DL PDU Session Information frame with the QMP field set to 1. It is used only in the downlink direction and encoded in the same format as the 64-bit timestamp format as defined in Section 6 of IETF RFC 5905 [6].

Value range: {0..264-1}.

Field length: 8 octets.

5.5.3.10	DL Sending Time Stamp Repeated

Description: This field indicates the value of the DL Sending Time Stamp field that the NG-RAN has received in the DL PDU Session Information frame with the QMP field set to 1 for the involved QoS flow. It is used only in the uplink direction and encoded in the same format as the 64-bit timestamp format as defined in Section 6 of IETF RFC 5905 [6]. The UPF shall, if supported, use this information to calculate DL or RTT delay between the NG-RAN and the UPF as specified in TS 23.501 [5].

Value range: {0..264-1}.

Field length: 8 octets.

5.5.3.11	DL Received Time Stamp

Description: This field indicates the time when the NG-RAN node receives the DL PDU Session Information frame with the QMP field set to 1 for the involved QoS flow. It is used only in the uplink direction and encoded in the same format as the 64-bit timestamp format as defined in Section 6 of IETF RFC 5905 [6]. The UPF shall, if supported, use this information to calculate DL or RTT delay between the NG-RAN and the UPF as specified in TS 23.501 [5].

Value range: {0..264-1}.

Field length: 8 octets.

5.5.3.12	UL Sending Time Stamp

Description: This field indicates the time when the NG-RAN node sends this UL PDU Session Information frame. It is used only in the uplink direction and encoded in the same format as the 64-bit timestamp format as defined in Section 6 of IETF RFC 5905 [6]. The UPF shall, if supported, use this information to calculate UL or RTT delay between the NG-RAN and the UPF as specified in TS 23.501 [5].

Value range: {0..264-1}.

Field length: 8 octets.

5.5.3.13	DL Delay Ind.

Description: This parameter indicates the presence of DL Delay Result.

Value range: {0= DL Delay Result not present, 1= DL Delay Result present}.

Field length: 1 bit.

5.5.3.14	DL Delay Result

Description: This field indicates the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface in milliseconds for the involved QoS flow. It is used only in the uplink direction and encoded as an Unsigned32 binary integer value. The UPF shall, if supported, use this information to calculate DL or RTT delay as specified in TS 23.501 [5].

Value range: {0..232-1}.

Field length: 4 octets.

5.5.3.15	UL Delay Ind.

Description: This parameter indicates the presence of UL Delay Result.

Value range: {0= UL Delay Result not present, 1= UL Delay Result present}.

Field length: 1 bit.

5.5.3.16	UL Delay Result

Description: This field indicates the uplnk delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU), the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow. It is used only in the uplink direction and encoded as an Unsigned32 binary integer value. The UPF shall, if supported, use this information to calculate UL or RTT delay as specified in TS 23.501 [5].

Value range: {0..232-1}.

Field length: 4 octets.

5.5.3.17	Sequence Number Presence (SNP)

Description: This parameter indicates the presence of the DL QFI Sequence Number in the DL PDU Session Information frame or the presence of the UL QFI Sequence Number in the UL PDU Session Information frame.

Value range: {0= DL/UL QFI Sequence Number not present, 1= DL/UL QFI Sequence Number present}.

Field length: 1 bit.

5.5.3.18	DL QFI Sequence Number

Description: This parameter indicates the sequence number as assigned by the UPF associated with a given QoS Flow.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.19	UL QFI Sequence Number

Description: This parameter indicates the sequence number as assigned by the NG-RAN node associated with a given QoS flow.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.20	N3/N9 Delay Ind.

Description: This parameter indicates the presence of N3/N9 Delay Result.

Value range: {0= N3/N9 Delay Result not present, 1= N3/N9 Delay Result present}.

Field length: 1 bit.

5.5.3.21	N3/N9 Delay Result

Description: This field indicates the accumulated N3 and N9 packet delay as specified in [5], and is reported by the I-UPF. The reported value is expressed in milliseconds and encoded as an Unsigned32 binary integer value.

Value range: {0..232-1}.

Field length: 4 octets.

5.5.3.22	D1 UL PDCP Delay Result Ind

Description: This parameter indicates if the UL Delay Result includes or not includes the D1 measurement (UL PDCP Packet Average Delay). This parameter shall be ignored if the UL Delay Ind is set to “0”.

Value range: {0= D1 UL PDCP Packet Average Delay measurement is not included, 1= D1 UL PDCP Packet Average Delay measurement is included}.

Field length: 1 bit.

5.5.3.23	MBS Sequence Number Presence (MSNP)

Description: This parameter indicates the presence of the DL MBS QFI Sequence Number in the DL PDU Session Information frame.

Value range: {0= DL MBS QFI Sequence Number not present, 1= DL MBS QFI Sequence Number present}.

Field length: 1 bit.

5.5.3.24	DL MBS QFI Sequence Number

Description: This parameter indicates the sequence number as assigned by the MB-UPF associated with a given MBS QoS Flow.

Value range: {0..232-1}.

Field length: 4 octets.

5.5.3.x	QoS Notification Cause Ind

Description: This parameter indicates that the QoS of the QoS flow associated with notification control is not fulfilled any longer or it is fulfilled again.

Value range: {0= GFBR can no longer be fulfilled, 1= GFBR can be fulfilled again}.

Field length: 1 bit.

CHANGE ENDS