3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231790

Online, April 17–26, 2023

Agenda Item:	10.2.2

Source:	CMCC

Title:	MRO for CPC and CPA

Document for:	Discussion and Decision

# 1 Introduction

In the past few RAN3 meetings, some agreements for MRO for CPC and CPA were achieved as shown below.

MRO for CPC and CPA:

For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

For MRO for CPAC, deprioritize Case i/ii/iii/iv:

-	Case i: mixed scenarios of legacy PA and CPA, i.e. UE receives CPA configuration, a legacy PSCell addition is performed but fails, or a legacy PSCell addition is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell addition.

-	Case ii: mixed scenarios of legacy PC and CPC, i.e. UE receives CPC configuration, a legacy PSCell change is performed but fails, or a legacy PSCell change is performed and succeeds but an SCG failure occurs shortly after the successful legacy PSCell change.

-	Case iii: MCG RLF or handover failure or CHO execution failure before CPA/CPC execution.

-	Case iv: CHO-CPC coexistence scenarios with low priority.

We can notice both RAN2 and RAN3 have further discussion on enhancements on SCGFailureInformation for CPC and CPA. In this contribution, we would provide some proposals on enhancements on SCGFailureInformation for the case that there is a SCG failure before CPC execution and further discuss on the case of MCG RLF before CPA/CPC execution.

# 2 Discussion

2.1 SCG failure before CPC execution

In the procedures of CPA and CPC, the MN sends RRCreconfiguration message including the CPA/CPC configuration to UE. The CPA/CPC configuration contains the configuration of CPA/CPC candidate PSCell(s) and the execution condition(s). After receiving the CPA/CPC configuration, UE stores the CPA/CPC configuration and then starts evaluating the associated execution condition(s). If the execution condition of one candidate PSCell is satisfied, the UE performs the procedure of CPA/CPC execution.

In the case of CPC, inappropriate CPC configuration may cause that there is a SCG failure before CPC execution. That means the CPC candidate PSCell(s) and the execution condition(s) included in CPC configuration may be not configured properly or the CPC configuration may be sent too late to UE. So in the case of SCG failure occurs before CPC execution, we think it is beneficial to include the time elapsed between the SCG failure in source SCG and the latest CPC configuration is received and the information of the CPC event trigger condition in the SCGFailureInformation message reported by UE.

Proposal 1: For MRO for CPC, if there is a SCG failure before CPC execution, the UE reports the following information in the SCGFailureInformation message

- The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

- The information of the CPC event trigger condition, e.g. no configured CPC event triggered, and only one of the CPC events triggered when there are multiple events configured for CPC. The detail can follow the agreement that the UE reports the first triggered CPAC event for MRO for CPC and CPA achieved in RAN3#119.

2.2 MCG RLF before CPA/CPC execution

- At RAN3#117bis-e meeting, RAN3 had agreements to deprioritize Case i/ii/iii/iv for MRO for CPAC.

-	Case iii: MCG RLF or handover failure or CHO execution failure before CPA/CPC execution.

- Among the three sub-cases of Case iii, we think MCG RLF before CPA/CPC execution is a more general scenario and the cooperation between MCG and SCG for MR-DC is need to be considered. In this case, when the MCG RLF occurs there is no configured CPA/CPC event triggered or only one of the CPA/CPC events triggered when multiple events configured for CPA/CPC. That means there is no need to configure CPA/CPC to UE, since the evaluation of the associated execution condition(s) also caused unnecessary UE power consumption. Additional, before the CPA/CPC configuration the MN has unnecessary coordination with each candidate SN(s) to allocate resources for the UE.

- Proposal 2: The case that there is a MCG RLF before CPA/CPC execution is considered for MRO for CPC and CPA.

- In the case that there is a MCG RLF before CPA/CPC execution, we think the information of the CPA/CPC event trigger condition reported by UE is helpful for network to optimize CPA/CPC configuration or to decide whether there is need to configure CPA/CPC to UE. Since UE generates RLF report when MCG RLF occurs, the enhancements on RLF report is needed for the case that there is a MCG RLF before CPA/CPC execution.

- Proposal 3: The enhancements on RLF report is needed for the case that there is a MCG RLF before CPA/CPC execution.

- Proposal 4: In the case that there is a MCG RLF before CPA/CPC execution, UE reports the information of the CPA/CPC event trigger condition, e.g. no configured CPA/CPC event triggered, and only one of the CPA/CPC events triggered when there are multiple events configured for CPA/CPC in RLF report.

# 3 Conclusion

In this paper, we discuss on MRO for CPC and CPA, and we have the following proposals:

Proposal 1: For MRO for CPC, if there is a SCG failure before CPC execution, the UE reports the following information in the SCGFailureInformation message

- The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

- The information of the CPC event trigger condition, e.g. no configured CPC event triggered, and only one of the CPC events triggered when there are multiple events configured for CPC. The detail can follow the agreement that the UE reports the first triggered CPAC event for MRO for CPC and CPA achieved in RAN3#119.

- Proposal 2: The case that there is a MCG RLF before CPA/CPC execution is considered for MRO for CPC and CPA.

- Proposal 3: The enhancements on RLF report is needed for the case that there is a MCG RLF before CPA/CPC execution.

- Proposal 4: In the case that there is a MCG RLF before CPA/CPC execution, UE reports the information of the CPA/CPC event trigger condition, e.g. no configured CPA/CPC event triggered, and only one of the CPA/CPC events triggered when there are multiple events configured for CPA/CPC in RLF report.

# Reference

- Chairman notes for RAN3#119 meeting, March 2023

- Chairman notes for RAN3#117bis-e meeting, October 2022