3GPP TSG-RAN WG3 Meeting #119-bis electronic	R3-231832

e-Meeting, 17th April – 26th April, 2023

Source:	vivo

Title:          Discussion on solution of LPHAP

Agenda Item:	23.2

Document for:	Discussion and Decision

- Introduction

R18 positioning WI in [1] was approved at RAN plenary#98-e. For LPHAP, the following objectives are listed in the WID:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we will discuss the following solutions for LPHAP from RAN3’s perspective.

-	SRS configuration enhancement

-	Alignment between PRS and eDRX

- Discussion

2.1	SRS configuration enhancement

2.1.1	SRS for positioning configurations in multiple cells

The potential procedure of SRS for positioning configurations in multiple cells is as follows.

Figure 2.1.1-1: The potential procedure of SRS for positioning configurations in multiple cells

Regarding the entity determining the SRS positioning validity area and requesting neighbor cells to reserve radio resources for SRS configuration, considering the existing UL positioning procedure, the SRS configuration is forwarded from the LMF to the TRPs via the NRPPa procedure. To our understanding, in line with the current operation, LMF should be the entity determining the SRS positioning validity area and requesting neighbor cells to reserve radio resources for SRS configuration.

After deciding to adopt the SRS configuration enhancement for LPHAP and the expected validity area, LMF should request the serving cell to configure SRS configuration. Then LMF should forward SRS configuration received from serving cell to neighbor cells belonging to the expected validity area and request neighbor cells to reserve radio resources for SRS configuration. The final validity area consists of neighbor cells that can reserve radio resources for SRS configuration.

Currently, the serving cell may release the UE to RRC_INACTIVE after determining the SRS configuration. As the LMF needs to negotiate with the neighbor cells and decides the final validity area, the serving cell should not release UE to RRC_INACTIVE before receiving the validity area from LMF. Therefore, an indication along with the SRS configuration request is needed. With the indication, the serving cell will know the SRS configuration is for low power high accuracy positioning and will wait for the validity area of the SRS configuration from LMF before releasing UE to the RRC_INACTIVE state. And then, the agreed SRS configuration associated with the validity area can be sent from the serving cell to the UE in RRCRelease with SuspendConfig.

Proposal 1: The SRS positioning validity area is determined by LMF.

Proposal 2: LMF may send an indication to the serving cell when requesting the serving cell to configure SRS configuration. After receiving the indication, the serving cell should wait for the validity area before releasing the target UE to the RRC_INACTIVE state.

Proposal 3: The LMF should forward the SRS configuration received from the serving cell to neighbor cells belonging to the expected validity area and request neighbor cells to reserve radio resources for SRS transmission. And LMF can decide the final validity area based on the feedback from the neighbor cells.

Proposal 4: The LMF should send the validity area to the serving cell so that the serving cell can forward it to the target UE via RRCRelease with SuspendConfig.

2.1.2 SRS configuration request

The potential procedure for SRS configuration request is as follows.

Figure 2.1.2-1: The potential procedure for SRS configuration request

When the UE reselects a new cell that is out of the validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. Then, the following two issues should be addressed:

- Issue 1: how to send this RRC message

- Issue 2: what is the RRC message

For the first issue, in our understanding, if there is an ongoing SDT, this RRC message can be sent by the ongoing SDT. If there isn’t ongoing SDT, two options can be considered. One is that UE sends the RRC message after entering the RRC_CONNECTED state, and the other is that UE sends the RRC message by triggering SDT without going to the RRC_CONNECTED state. Note that the current MO-SDT can only be triggered by upper layers, i.e., NAS or application layer.

For the second issue, either reusing the existing RRC message (e.g., UE Assistance Information) or introducing a new RRC message is feasible.

After receiving the RRC message for SRS configuration request, the new serving cell will know UE’s requirement for new SRS configuration. Then, the new serving cell will update the SRS configuration according to the UE context. In addition to the SRS characteristics, the new indications should also be included in the UE context, so the new serving cell will release UE to the RRC_INACTIVE state after receiving a validity area of SRS configuration from LMF.

Proposal 5: The last serving cell should include the indication received from LMF in the UE context. With it, the new cell will wait for the validity area from LMF before releasing the target UE to the RRC_INACTIVE state.

2.2	Alignment between PRS and eDRX

2.2.1 Align PRS with fixed eDRX

The state transition energy is a key component of power consumption during positioning. Thus it is beneficial to minimize the gap between PRS measurement and paging monitoring in the time domain to reduce the number of sleep-on transitions. The potential procedure for aligning PRS with fixed eDRX is as follows.

Figure 2.2.1-1: The potential procedure for aligning PRS with fixed eDRX

How to align the positioning and paging monitoring if eDRX configured needs to be studied. The issues include:

- The eDRX configuration is controlled by the gNB and the AMF, which is transparent to the LMF.

- Different UEs have different POs. That is, eDRX is per UE while PRS is per cell.

- Cell reselection may happen due to UE mobility and some of the parameters for UE to monitor paging are cell-specific, e.g., paging frame offset. That is, the PO in the time domain will change upon cell reselection.

For the first issue, the LMF should be informed about the eDRX, DRX and default paging configuration in advance, and then LMF may take the information into account when setting the periodicity and/or response time of deferred MT-LR.

Proposal 6: LMF should be informed about the eDRX, DRX and default paging configuration in advance, and then it may take the information into account when setting the periodicity and/or response time of deferred MT-LR.

For the second and third issues, the LMF shall indicate some assistance information to the serving gNB and neighbor gNB to configure suitable PRS. As mentioned before, the PRS measurement should be configured to be close to the paging occasion, which can reduce the transition power of the UE to receive PRS when it wakes up from deep sleep. One straightforward way is to configure a relatively short PRS periodicity by on-Demand PRS, making sure that all target UEs can detect PRS around their respective PO. Another way is to request UE-specific PRS for the target UEs with LPHAP requirements and the gNB may stop the PRS transmission if no target UE will monitor paging around the PRS.

Proposal 7: To align the PRS configuration with fixed eDRX, LMF should negotiate PRS configuration with selected TRP(s) to ensure there is valid PRS around PO within PTW.

2.2.2 Align eDRX with fixed PRS

The paging occasion position is determined by the UE_ID, so it is infeasible to align the PO of multiple UEs with fixed PRS. In addition, according to the justification R14 eDRX’ WID[2] below, the eDRX cycle is configured based on the data delay tolerance and power saving requirements. Therefore, the eDRX cycle can not be adjusted significantly for positioning purposes.

“Hence, DRX cycle extension is required in order to enable significant battery savings for such UEs. Furthermore, the DRX cycle can be set depending on the data delay tolerance and power saving requirements, thus providing a flexible solution for achieving significant UE battery savings.”

Based on the above analysis, we propose:

Proposal 8: Considering the following aspects, the discussion on aligning eDRX with fixed PRS can be de-deprioritized in Rel-18.

- It is not feasible to align the eDRX of multiple target UEs with fixed PRS considering that different UEs have different PO.

- The DRX cycle can be set depending on the data delay tolerance and power-saving requirements. In this sense, the eDRX cannot be adjusted significantly for positioning purposes.

- Conclusion

SRS configuration enhancement

Proposal 1: The SRS positioning validity area is determined by LMF.

Proposal 2: LMF may send an indication to the serving cell when requesting the serving cell to configure SRS configuration. After receiving the indication, the serving cell should wait for the validity area before releasing the target UE to the RRC_INACTIVE state.

Proposal 3: The LMF should forward the SRS configuration received from the serving cell to neighbor cells belonging to the expected validity area and request neighbor cells to reserve radio resources for SRS transmission. And LMF can decide the final validity area based on the feedback from the neighbor cells.

Proposal 4: The LMF should send the validity area to the serving cell so that the serving cell can forward it to the target UE via RRCRelease with SuspendConfig.

Proposal 5: The last serving cell should include the indication received from LMF in the UE context. With it, the new cell will wait for the validity area from LMF before releasing the target UE to the RRC_INACTIVE state.

Alignment between PRS and eDRX

Proposal 6: LMF should be informed about the eDRX, DRX and default paging configuration in advance, and then it may take the information into account when setting the periodicity and/or response time of deferred MT-LR.

Proposal 7: To align the PRS configuration with fixed eDRX, LMF should negotiate PRS configuration with selected TRP(s) to ensure there is valid PRS around PO within PTW.

Proposal 8: Considering the following aspects, the discussion on aligning eDRX with fixed PRS can be de-deprioritized in Rel-18.

- It is not feasible to align the eDRX of multiple target UEs with fixed PRS considering that different UEs have different PO.

- The DRX cycle can be set depending on the data delay tolerance and power-saving requirements. In this sense, the eDRX cannot be adjusted significantly for positioning purposes.

- Reference

- RP-223549, New WID on Expanded and Improved NR Positioning

- RP-151339, Updated WI Proposal: RAN enhancements for extended DRX in LTE