3GPP TSG-RAN WG3 #119bis-e		                                                                 R3-2xxxxx

Online 17th – 26th Apr 2023

Title:	[draft] Reply LS on Mapping of F1-C IP addresses in the IAB inter-CU topology adaptation and backhaul RLF recovery procedures

Response to:

Release:	Rel-17

Work Item:	TEI17

Source:	ZTE [to be: RAN3]

To:	SA3

Cc:	RAN2

Contact Person:	Ying Huang

huang.ying11@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA3 for their LS on Mapping of F1-C IP addresses in the IAB inter-CU topology adaptation and backhaul RLF recovery procedures. And RAN3 provides the following feedback on the issue raised in the LS:

SA3 Question: SA3 is currently trying to specify the security handling of IAB inter-CU topology adaptation and backhaul RLF recovery procedures.

In this work, SA3 would need more information for defining one aspect in a way sympathetic to the current flows in TS 38.401, when using dynamic PSK. SA3 believe that the Source/Initial Donor IAB-node needs to know the mapping between the old IP address of the F1-C interface and the new IP address. This mapping is needed to identify the security credentials that will be used to re-establish (using IKE) the IPsec connection used to protect the FI-C interface. In particular this needs to be done for IPsec transport mode and also when there are one or more old/new IP addresses.

SA3 requests RAN3 to provide a suitable method for Source/Initial Donor-IAB node to know the mapping between these  IP addresses in order for SA3 to progress with its security work.

RAN3 feedback: RAN3 has discussed the issue, and no agreement to support it in Rel-17.

# 2	Actions

To SA3 group.

ACTION: 	RAN3 respectfully ask SA3 take RAN3 input into consideration.

# 3. Date of Next RAN3 Meetings:

RAN3#120	22 th – 26 th May, 2023	Incheon, Korea

RAN3#121	21th – 25th  August, 2023	Toulouse, France