3GPP TSG-RAN WG3 Meeting #119bis-e	TDoc R3-232061

Electronic meeting, Apr. 17th – Apr. 26th, 2023

Title:	LS on intra-system inter-RAT SHR and SPR

Response to:

Release:	Rel18

Work Item:	Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI (NR_ENDC_SON_MDT_enh2-Core)

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Henrik Olofsson

henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

Inter-RAT SHR

RAN3 has agreed to support collection of SHR during successful inter-RAT HO (LTE to NR) but only if the impact on LTE can be minimized. With this in mind, RAN3 agreed to only consider the T304 trigger. The decision further relied on the following assumptions which needs to be confirmed by RAN2:

- Target gNB can send SHR configuration (T304 trigger) to UE via NR container (targetRAT-MessageContainer) in MobilityFromEUTRACommand

- UE stores this SHR configuration in NR format

- If T304 trigger is met, UE records SHR in NR format

- UE reports this SHR to only an gNB (either the target gNB or another gNB)

In case the above scenario is also considered feasible for RAN2, RAN3 has identified that at least the following information is beneficial to be reported from the UE:

- Source LTE cell

- Target NR cell

SPR

RAN3 did not identify any further requirements regarding the representation of triggers for SPR compared to what is used in Rel17 for SHR, i.e. triggers may be represented as percentage values.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration and provide feedback on the feasibility of the scenario of SHR during successful inter-RAT HO (LTE to NR)

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3 #120		22 – 26 May 2023		Incheon, KR

RAN3#121		21 – 25 Aug 2023		Toulouse , FR