3GPP TSG-RAN WG3 #120							R3-233357

22th – 26th May 2023

Incheon, Korea

Agenda Item:	14.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion: CB: # MobilityEnh1_L1L2Mobility

Document for:	Discussion

# Introduction

This is the offline summary for the following comeback:

CB: # MobilityEnh1_L1L2Mobility

- Continues discuss on WF or agreement the selection between option 1 (class 2) and option 2.

- Check E1 stage-2 description, and details in blue texts above.

- Further offline discussion on TA acquisition, HO collision btw LTM and L3 mobility, DDDS. Capture the agreements and open issues.

- Check F1 stage-3 and draft the TPs.

Offline Summary in R3-233357

(Moderator - HW)

# For the Chairman’s Notes

# Discussion

- Notification of LTM triggering to gNB-CUs

Option 1: New F1 message, class 1 or class 2, such as “LTM CELL CHANGE NOTIFICATION”.

Option 2: Legacy message, i.e., “UE CONTEXT MODIFICATION REQUIRED” message.

Option1: Nok, QC, Lenovo,CATT, CMCC, HW

Option2: Samsung, ZTE, LG

If go for Option 1, there is no need for response message.

Offline discussion :

- E1 aspects for inter-DU LTM

Further check on wording P3.6-5 and P3.6-3 for stage-2 description.

Proposal 3.6-3: For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP.

Proposal 3.6-5: For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure is performed towards the core network.

Need further discussion on :

Proposal 3.6-4: In case of gNB-CU-UP change, the gNB-CU triggers the source gNB-CU-UP to start data forwarding after receiving LTM cells switch signalling from DU.

Different proposal from R3-232671:

Proposal 11: Data forwarding can be triggered before LTM CELL CHANGE NOTIFICATION message and is left up to the implementation.

Offline discussion :

- TA acquisition

- Issue 1: Request of RACH resources for TA acquisition

- CU can send a request for TA establishment together with the request to configure an LTM candidate cell is requested to a Candidate DU in the UE Context Setup Request and UE Context Modification Request messages.

- If the Candidate DU accepts the request for TA establishment, Candidate DU responds the CU with a RACH configuration for TA establishment in the UE Context Setup Response and UE Context Modification Response messages.

- TA info acquisition

TA info acquisition Method:

Proposal : The TA acquisition method is configured during LTM preparation phase to the (Target) gNB-DU via either the UE Context Setup or the UE Context Modification procedure.

Proposal 4: Add new RACH Configuration IE indicating whether RAR needs to be received or not (e.g., RACH-ConfigDedicated).

Which node decide the method:

Option 1: gNB-CU

Option 2: candidate gNB-DU

Option 3: serving gNB-DU

- Issue 2: Configure the RACH resources to source DU

- During LTM configuration phase in inter-DU LTM, the gNB-CU provides the RACH resource for TA acquisition received from the candidate gNB-DU via UE Context Modification procedure.

- TA information transfer from candidate DU to source DU

- When TA establishment is triggered and a “RAR” is not configured, the C-DU receives the preamble, calculates the TA value and provides it to the CU (which forwards it to the S-DU).

- When a TA establishment is triggered and a “RAR” is configured, the information required to be transferred over F1 should be the same as without RAR case. Details are pending in RAN2.

F1 Signalings for TA info transfer:

Proposal 1: To introduce new class 2 non-UE associated F1 signalings to deliver the TA information from the candidate gNB-DU to the source gNB-DU via gNB-CU for both without RAR and with RAR cases.

- Regarding the handling of the TA value we are open to both the use of legacy procedures (i.e. UE Context Setup and UE Context Modification procedure) and the introduction of a new non-UE associated procedure.

Offline discussion :

- HO collision btw LTM and L3 mobility

Inputs from last meeting:

- Option 1: OAM configured priority.

- Option 2: Network decides the priority based on scenario (intra-gNB-CU or inter-gNB-CU) and some assistance information (the measurement results, candidate target cells).

- Option 3: Flexible priority. The handover triggered first take the high priority.

For option 3, The detailed description would be:

- Case 1, L3 handover triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority.

- Case 2, LTM triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority,

- Case 3, LTM and L3 handover are triggered almost simultaneously (cross signallings on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority. This is to avoid to cancel the LTM command already sent to the UE which seems complex from moderator point of view.

RAN2 agreements:

- While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

Offline discussion :

- DDDS

Status of last meeting:

Proposal 3.2-1: For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs RLC is not re-established.

For intra-DU LTM, the gNB-DU sends a DDDS frame about unsuccessfully transmitted downlink data to the gNB-CU after LTM cell switch if RLC reestablishment is configured.

For inter-DU LTM, the DDDS should be sent from source gNB-DU to CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU.

Proposal 3.2-4: For both intra-DU and intra-CU inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access.

Proposals at this meeting:

Proposal 6: For both intra-gNB-DU and inter-gNB-DU LTM, the target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access.

Proposal 13. Upon detecting that the UE has accessed the candidate target cell, the target DU transmits an initial DDDS to the candidate CU-UP.

Proposal 3: Capture the agreement about intra-DU LTM DDDS until RLC reestablishment is confirmed to be a valid case in intra-DU LTM.

Proposal 4: For intra-DU case, if new UL TEID if assigned by the CU(usually it is for CP change case), the DU sends initial DDDS using the new UL TEID to CU-UP after DU detects the UE access.

Proposal 5: For intra-DU case, if there is no new UL TEID assigned by the CU, the DU is no need to send the initial DDDS message.

Proposal 4: It is not needed to capture “For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs for which RLC is not re-established.” in the spec.

Offline discussion :

# Check F1 stage-3 and draft the TPs (2nd round)