3GPP TSG-RAN WG3 #120	draft R3-233437

Incheon, South Korea, 22-26 May 2023

Agenda Item:	17.2

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on NR NTN Service Continuity

Document for:	Discussion

# Introduction

CB: # NRNTN1_ServiceContinuity

- Check the details of TPs and capture the agreements above on early data forwarding and time conditional NG HO

- LS to SA2?

(moderator - E///)

Summary of offline R3-233437

# For the Chair’s Notes

Proposals to be added

# Discussion

According to the online discussion, the following items are proposed to be discussed/captured:

- Time condition in NG HO – aiming to capture the existing agreement (RAN3 #119): Confirm to add the handover window start and duration IEs to the NGAP Source NG-RAN Node to Target NG-RAN Node Transparent Container IE.

  - Stage 2 TP in R3-232947 (need to choose 1 agreeable option)

  - NGAP BL CR in R3-233309

- Early data forwarding / discarding – aiming to capture the existing agreement (RAN3 #119): Confirm to enhance the early data forwarding with data discarding for NG HO. FFS on details, e.g. Introduce a DL discarding related IE in Early Status Transfer Transparent Container IE.

  - Stage 2 TP in R3-232930

  - NGAP TP in R3-232931

  - NGAP TP contained in R3-232798

- Proposed correction to current XnAP BL CR

  - XnAP TP contained in R3-232798

- LS to SA2 – asking SA2 to take the RAN3 agreements into account and update their specifications if needed.

## Time Condition in NG HO

### Stage 2 (TS 38.300)

Currently 2 options are on the table: Option 1 from R3-232947 and Option 2 from the same document.

From current status of discussions, it seems 1 company supports Option 1, and 9 companies support Option 2. Considering the above, the Moderator proposes to put up Option 2 for agreement.

Any objection/comment to agreeing Option 2 as the Stage 2 TP? (R3-232947 will need to be revised to only include the TP for the agreeable option)

| Company | Objection (if any) / Comment for further revision (if any) |
| --- | --- |
| Ericsson | No objection. Option 2 is agreeable. Considering that a) the NG HO signaling flow is not captured in TS 38.300 and that b) we are not changing the signaling flow but only adding IEs in the transparent containers, Option 2 seems to be adequate. |

### NGAP

If the contents are agreeable, R3-233309 will become the NGAP BL CR for this WI.

Any objection/comment to endorsing R3-233309 as the NGAP BL CR?

| Company | Objection (if any) / Comment for further revision (if any) |
| --- | --- |
| Ericsson | No objection. 3309 can be endorsed as the NGAP BL CR. |

## Early Data Forwarding / Discarding

### Stage 2 (TS 38.300)

R3-232930 provides a stage 2 TP for this. A Tdoc # (R3-233454) has been allocated for a revision of R3-232930.

Any objection/comment to agreeing the TP included in R3-232930 ? (To be revised if needed in R3-233454)

| Company | Objection (if any) / Comment for further revision (if any) |
| --- | --- |
| Ericsson | No objection. 3454 should contain only the TP and can be agreed. |

### NGAP

R3-232931 provides a stage 3 TP for this. A Tdoc # (R3-233455) has been allocated for a revision of R3-232931.

Any objection/comment to agreeing the TP included in R3-232931? (To be revised if needed in R3-233455)

| Company | Objection (if any) / Comment for further revision (if any) |
| --- | --- |
| Ericsson | No objection. 3455 should contain only the TP and can be agreed. |

R3-232798 includes an alternative realization for the stage 3 TP for this. The Moderator proposes to check whether any “delta” from R3-232798 needs to be captured, in which case a revision of R3-232798 will have to be produced and put up for agreement.

Any “delta” from the NGAP TP in R3-232798 that needs to be captured? (A new Tdoc # with such “delta” will have to be produced and put up for agreement)

| Company | Comment |
| --- | --- |
| Ericsson | No “delta” needs to be captured, as the difference between 2798 and 2931 is just aesthetics: the proposed new DL Discarding IE is defined as a standalone IE in 2798 whereas in 3455 it is included in an existing IE. 2798 can be noted. |

## Correction to Current XnAP BL CR

R3-232798 also includes an XnAP TP proposing to remove part of the currently endorsed text for Sec. 8.2.1.2 of TS 38.423 as follows:

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use this information to allocate necessary resources for the incoming CHO.

Any objection to agreeing the XnAP TP in R3-232798? (A new Tdoc # with such TP will have to be produced and put up for agreement)

| Company | Objection (if any) / Comment for further revision (if any) |
| --- | --- |
| Ericsson | No objection: we have a slight preference for removing the text as proposed in 2798, although we would also be OK with leaving things as they are. |

## LS to SA2

Once the TPs are stable, it is proposed to liaise SA2 (cc RAN2) to notify them of the RAN3 agreements and to ask SA2 to update its specifications if needed.

A draft LS is provided in the folder for this CB. Companies are kindly asked to check the draft LS and provide their comments directly on it.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-232947	NGAP Support for Time-Based HO in NTN, Ericsson, Thales, Intelsat, Lockheed Martin, Hughes Network Systems, CATT, Nokia, Nokia Shanghai Bell.

- R3-233309	Time-Based HO for NTN – NGAP Impacts, Ericsson, Thales, ZTE, Omnispace, TTP, CATT, Hughes Network Systems, Huawei, Lockheed Martin, Intelsat, ESA.

- R3-232930	(TP for NR NTN BL CR TS38.300) Discussion on the time-based trigger condition in NR NTN, Nokia, Nokia Shanghai Bell.

- R3-232931	(TP for NR NTN BL CR TS38.413) Support time-based trigger condition in NR NTN NG-HO, Nokia, Nokia Shanghai Bell.

- R3-232798	(TP for BL CR to TS 38.413/38.423) On Mobility and Service Continuity Enhancements for NTN, CATT.