3GPP TSG-RAN WG3 #120                                            R3-233282

22 - 26 May 2023

Incheon, Korea

Agenda item:	11.3

Source: 	ZTE

Title: 	(TP to BL CR of 38.423) QoE in NR-DC

Document for:   Discussion and Approval

# Introduction

This is  a TP for IE design the MN-SN QMC coordination.

# 5. TP to BL CR of 38.423

### 9.2.3	General IE definitions

- <unchanged omitted>

- 9.2.3.x	S-NODE Initiated QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

Editor’s Note: The messages used to convey this IE are FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Configuration List |  |  |  |  |
| >QMC Configuration List Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Configuration Sending Option [FFS] | O |  |  |  |
| >>QoE Reporting Option Preference [FFS] | O |  |  |  |
| >>RVQoE Reporting Option Preference [FFS] | O |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

- 9.2.3.y	S-NODE Initiated QMC Coordination Response

Editor’s Note: The messages used to convey this IE are FFS.

This IE contains the information that the M-NG-RAN node needs to provide in response to the S-NG-RAN node-initiated QMC coordination request.

| IE/Group Name | Presence |  | Range |  | IE type and reference |  | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QMC Response List |  |  |  |  |  |  |  |  |  |
| >QMC configuration List Item |  |  |  |  | 1..<maxnoofUEAppLayerMeas> |  |  |  |  |
| >>QoE Reference | M |  |  |  | OCTET STRING (SIZE(6)) |  |  |  |  |
| >>Measurement Configuration Application Layer ID | O |  |  |  | INTEGER (0..15, ...) |  |  |  |  |
| >>QoE Configuration Sending Option [FFS] |  | O |  |  |  |  |  |  |  |
| >>QoE Reporting Option [FFS] |  | O |  |  |  |  |  |  |  |
| >>RVQoE Reporting Option [FFS] |  | O |  |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

- 9.2.3.z	QMC Modification

This IE indicates the information exchanged between the M-NG-RAN node and the S-NG-RAN node for the modification of QoE and/or RVQoE configuration and reporting.

Editor’s Note: The messages used to convey this IE are FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QMC Modification List |  |  |  |  |
| >QMC Modification Item |  |  | 1..<maxnoofUEAppLayerMeas> |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) |  |
| >>QoE Reporting Modification [FFS] | O |  |  |  |
| >>RVQoE Reporting Modification [FFS] | O |  |  |  |
| >> RAN Visible QoE Session Indication [FFS] | O |  |  |  |
| >>RAN Visible QoE Configuration Preference | O |  |  |  |
| >>RAN Visible QoE Reporting Deactivation | O |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

End of Changes