3GPP TSG-RAN WG3 #121						              										R3-233821

21th – 25th Aug 2023

Toulouse, France

Agenda Item:	21.2

Source:	ZTE

Title:	Discussion on NR Redcap enhancement

Document for:	Approval

- Introduction

In the last RAN3 #120 emeeting, RAN3 has achieved the following progress.

RAN3 agrees that AMF sends the assistance information to NG-RAN node to decide the paging priority. RRC State indication is not needed in the MT COMMUNICATION HANDLING REQUEST message when indicating of UE connection resume FFS on signalling a UE Reachability indication to AMF Add a CHOICE structure in the MT COMMUNICATION HANDLING REQUEST: - Activate HLCOM - NR RAN Paging eDRX Cycle for RRC INACTIVE (M) - NR RAN PTW length (M) (FFS if should be always provided by the RAN) - NR RAN PTW start H_SFN ( FFS on the IE details) - Deactivate HLCOM - Deactivate HLCOM indication P3) PPI, ARP, PDU session ID and QFI are included in the DL DATA NOTIFICATION message. FFS on 5QI. P4) Send LS to SA2 to ask if feasible to provide DL Data size to NG-RAN during the DL DATA NOTIFICATION for the purpose of MT-SDT paging decision.

This paper addresses on the open issue to enhance eDRX long than 10.24s.

- Discussion

  - NGAP impact

- Activate HLCOM

  - NR RAN Paging eDRX Cycle for RRC INACTIVE (M)

  - NR RAN PTW length (M) (FFS if should be always provided by the RAN)

  - NR RAN PTW start H_SFN ( FFS on the IE details)

For CN based MT communication handling, only in case that the UE is in RRC inactive mode and all the UE’s bearer are suspended, the NR Paging eDRX Cycle will be sent to CN.

More, in legacy CN paging for Idle UE, the CN sends NGAP Paging message based on the Paging eDRX Cycle and PTW length, so that, for long eDRX>10.24s, the CN shall also sends NGAP DL DATA NOTIFICATION message based on the Paging eDRX Cycle and PTW length.

On the contrary, without PTW length, the CN has to send NGAP DL DATA NOTIFICATION message at each of the PTW start point. If the PTW length is more than tens of seconds, it will introduce uncessary RAN paging delay.

Figure 1

Although the CN can derive the PTW start location from the CN configured eDRX cycle, since the RAN eDRX length may be shorter than the CN eDRX cycle length, and the RAN PTW maybe larger than or shorter than the CN PTW length, both RAN eDRX and RAN PTW length are necessary for CN to decide the RAN PTW occasion. Otherwise, the CN cannot decide the RAN PTW end occasion, when a NGAP message is produced inside the RAN PTW, the CN can only postpone to send it to the gNB just before the next RAN PTW, this will produce unnessary NGAP message sending delay. The CN can decide the PTW Start point based on the eDRX cycle, so the NR RAN PTW start H_SFN is not needed.

So that, we further suggest to introduce an extendable IE (e.g., NR Paging Long eDRX Information for RRC INACTIVE) including Paging eDRX Cycle and PTW length.

Proposal 1: In NGAP MT COMMUNICATION HANDLING REQUEST message, it is suggested to introduce a new IE (e.g., NR Paging Long eDRX Information for RRC INACTIVE) including Paging eDRX Cycle and PTW length, but NR RAN PTW start H_SFN is not needed.

P3) PPI, ARP, PDU session ID and QFI are included in the DL DATA NOTIFICATION message. FFS on 5QI.

In our view, 5QI as well as QFI, ARP, 5QI, PPI and PDU Session ID, is used to enable the paging differentiation, so we suggest to introduce 5QI into the DL DATA NOTIFICATION message.

Proposal 2: 5QI is included in the DL DATA NOTIFICATION message.

  - XnAP/F1AP impact

5) ASN.1 in F1AP and XnAP: FFS if the NRPaging-time-Window defined in ASN.1 can be re-used instead of a new d NRPaging-Time-Window-Inactive

In RAN2 #121bis emeeting, the following progress is achieved.

- UEs configured with Rel-18 enhanced INACTIVE eDRX should apply Rel-18 enhanced INACTIVE eDRX if Rel-18 enhanced INACTIVE eDRX is allowed in the serving cell, regardless of whether Rel-17 INACTIVE eDRX is allowed in the serving cell. - UEs configured with Rel-18 enhanced INACTIVE eDRX should apply INACTIVE DRX if both Rel-18 enhanced INACTIVE eDRX and Rel-17 INACTIVE eDRX are not allowed in the serving cell. - Working assumption (pending specification complexity and NW complexity evaluation): UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX.

More, RAN2 #122 meeting makes the following agreement

- UE can support Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t support Rel-17 INACTIVE eDRX. - A cell can allow Rel-18 INACTIVE eDRX (which comprises eDRX cycles and PTWs), even if it doesn’t allow Rel-17 INACTIVE eDRX, but the cell must allow IDLE eDRX. - We confirm the working assumption: UEs configured with Rel-18 enhanced INACTIVE eDRX should fall back to use Rel-17 INACTIVE eDRX (if capable and configured with Rel-17 INACTIVE eDRX) if the Rel-18 enhanced INACTIVE eDRX is not allowed but the Rel-17 INACTIVE eDRX is allowed by the current cell. gNB has the possibility to configure both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX, allowing the UE to fall back to use Rel-17 INACTIVE eDRX. - A UE configured with Rel-18 INACTIVE eDRX will fallback to use INACTIVE RAN DRX if it is either not configured with Rel-17 INACTIVE eDRX or the cell does not allow Rel-18 INACTIVE eDRX and Rel-17 INACTIVE eDRX.

According to the RAN2 WA, the UE configured with R18 enhanced eDRX may enter into the new RAN node in which some cells are configured with R18 enhanced eDRX but some other cells are configured with R17 eDRX.

Proposal 3: In XnAP and F1AP, the targe RAN node and gNB-DU shall be configured both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX together.

For the value of eDRX cycle and PTW length, it has agreement in RAN2 #121meeting.

      - PTW length value range of enhanced INACTIVE eDRX is same as IDLE eDRX, i.e. from 1.28s to 40.96s in the step of 1.28s.

      - Long eDRX cycle (>10.24 s) value range of enhanced INACTIVE eDRX is same as IDLE eDRX from 20.48s to 10485.76s, i.e. hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024

Proposal 4: In XnAP and F1AP Paging message, it is suggested to introduce a new IE (e.g., NR Paging Long eDRX Information for RRC INACTIVE) including NR Paging eDRX Cycle for Inactive IE and NR Paging Time Window for Inactive IE.

NR Paging eDRX Cycle Inactive IE: ENUMERATED (hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024)

NR Paging Time Window for Inactive IE: ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32,…)

- Conclusion

Based on the above analysis, we have the following proposals.

Proposal 1: In NGAP MT COMMUNICATION HANDLING REQUEST message, it is suggested to introduce a new IE (e.g., NR Paging Long eDRX Information for RRC INACTIVE) including Paging eDRX Cycle and PTW length, but NR RAN PTW start H_SFN is not needed..

Proposal 2: 5QI is included in the DL DATA NOTIFICATION message.

Proposal 3: In XnAP and F1AP, the targe RAN node and gNB-DU shall be configured both Rel-17 INACTIVE eDRX and Rel-18 enhanced INACTIVE eDRX.

Proposal 4: In XnAP and F1AP Paging message, it is suggested to introduce a new IE (e.g., NR Paging Long eDRX Information for RRC INACTIVE) including NR Paging eDRX Cycle for Inactive IE and NR Paging Time Window for Inactive IE.

NR Paging eDRX Cycle Inactive IE: ENUMERATED (hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024)

NR Paging Time Window for Inactive IE: ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32,…)

We also provode corresponding TPs.