3GPP TSG-RAN WG3 Meeting #121	R3-233912

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:		12.2.2.1

Source:				Intel Corporation

Title:	Discussion on Partial Reporting and UE Performance Feedback

Document for:		Discussion and Decision

# Introduction

Following agreements were discussed during RAN3 #121 meeting:

Partial reporting: The group discussed and had the common understanding as below: Cause value measurement not supported -> is covered by legacy cause “Measurement not Supported For The Object,” Cause value measurement temporarily not available -> is covered by legacy cause “Measurement Temporarily not Available,” It is FFS whether any more cause values should be added to the legacy list of causes. UE performance feedback: The problem of how to signal to the reporting node time configurations for the UE Performance Feedback measurement reporting is acknowledged. Time configuration for UE Performance Feedback measurement reporting consists at least of: - Time duration of the UE Performance Feedback measurement collection (the time duration starting at handover execution and including the last UE Performance Feedback report) - Whether, within such time duration, measurements are reported periodically or one time - In case of periodic reporting, the period of UE Performance Feedback measurements (whether existing of new IE)

In this contribution, we mainly focus on the remaining open issues on partial reporting and UE performance feedback.

# Discussion

## Partial Reporting

During RAN3 #120 meeting, following two options were down-selected with below agreements:

- Option 1: Introduce the indicator in the request message that informs the requested node if the partial reporting is allowed or not allowed.

- Option 3: No explicit IE in the request message.

There is consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part. It needs to be further discussed whether such enhancements can be introduced in Rel18.

As captured in TS 36.423 [1], partial success indicator is used to indicate whether partial success is allowed or not. It is argued that the partial reporting is mainly used to save signaling and processing in case one AI/ML algorithm may not be able to produce all inference outputs per request.

It was agreed in RAN3 #117e meeting that the AI/ML information request/response procedure design is similar as resource status report procedure.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

In current resource status report procedure, if the requested NG-RAN node is not capable to provide all requested resource status information, it is required in the procedure description (i.e. Section 8.4.10.3) that the requested NG-RAN node should send the Resource Status Failure message instead.

To support partial reporting, instead of introducing a separate indicator in the request message, we can simply release the limitation of sending AI/ML information response message as below:

For AI/ML information response message:

If NG-RAN node2 is capable to provide any requested AI/ML related information, it shall initiate the corresponding measurement as requested by NG-RAN node1 and respond with the AI/ML Information RESPONSE message.

For AI/ML information failure message:

If none requested measurements can be initiated, NG-RAN node2 shall send the RESOURCE STATUS FAILURE message.

Proposal 1: No explicit IE is introduced in the AI/ML information request message to request partial response.  Requested node can provide partial results if it cannot provide full results.

The next question is how to indicate the failure measurement to the requesting NG-RAN node. As discussed above, if partial reporting is allowed, the AI/ML information response message can be extended to also indicate partial failure measurement together with the failure cause value. Upon receiving the failure measurement information, the source NG-RAN node can resend another request message for the expected measurement according to the cause value. For the successful measurements, they can be included in the follow-up class-2 AI/ML information update message.

Proposal 2: Introduce a list of failure measurements and the corresponding cause indicating the reason for failure per measurement in the AI/ML Information response message.

## Cause Values

It was agreed that “measurement not supported” for one prediction information is covered by the existing cause value “measurement not supported for the object” and “measurement temporarily not available” can reuse existing cause value “Measurement Temporarily not Available”. It is still FFS whether any new cause values should be added to the legacy list of causes.

In our understanding, the below proposed cause values are subset of “measurement not supported for the object” and “Measurement Temporarily not Available”. However, even if the requested NG-RAN node replies with a detailed cause value, e.g. measurement not supported with requested reporting periodicity, there’s no way for the source NG-RAN node knows what is the suitable reporting periodicity or combination required by the requested NG-RAN node. Hence, the source NG-RAN node still cannot encode a follow-up request message that successfully receive the requested measurement/predictions other than by trial and error. The source NG-RAN node may keep receiving failure messages due to unsuitable configuration.

- measurement not supported with requested reporting periodicity, - measurement temporarily not available with requested reporting periodicity, - measurement not supported with current combination of requested information, - measurement temporarily not available with current combination of requested information,

Observation 1: The detailed cause value provides more detailed failure reason on top of existing cause values. Even with detailed failure cause value, the requesting NG-RAN node still cannot encode suitable reporting periodicity and/or combination of requested information for successfully reception of measurement results, and may keep receiving failure messages.

To help the requesting NG-RAN node successfully set a suitable configuration and avoid keep receiving failure message, the requested NG-RAN node can include its expected configuration, e.g. desired requested reporting periodicity or desired combination of requested information, to the requesting NG-RAN node together with the failure cause value in the AI/ML information response message. Upon receiving such information, the source NG-RAN node can directly use those values/configuration in the follow-up request message.

Proposal 3: The requested NG-RAN node includes the following failure cause values and the desired values in the AI/ML Information Response message:

- measurement not supported with requested reporting periodicity and requested NG-RAN node’s supported value,

- measurement temporarily not available with requested reporting periodicity and requested NG-RAN node’s supported value,

- measurement not supported with current combination of requested information and requested NG-RAN node’s supported combination,

- measurement temporarily not available with current combination of requested information and requested NG-RAN node’s available combination.

## UE Performance Feedback

As agreed in previous meetings, post-handover measurements are requested by the AI/ML class 1-message, referred in the Handover Request message, and reported through the AI/ML class-2 message. In this section, we mainly focus on the remaining open issues of UE performance feedback.

Report Time Configuration

It was heatedly discussed whether to use AI/ML class-2 message or a separate message to report UE performance feedback. In our understanding, the main benefit of using AI/ML class-2 message is to reduce signaling overhead. Therefore, when configuring reporting time of UE performance feedback, it is better align the time configuration between UE performance feedback with other requested information in AI/ML information request message.

Proposal 4: The configuration of reporting time for UE performance feedback should be consistent with or reuse the configuration for other requested information in AI/ML information request message.

UE performance feedback no longer available/supported

It is possible that the UE performance feedback becomes no longer available after some time. For example, once the UE is handed over to a subsequent NG-RAN node by target NG-RAN node (i.e. a 3rd NG-RAN node after UE’s 2nd handover), the target NG-RAN node needs to provide its UE performance feedback to the original source node according to the configuration, either using a one-time report or a periodic report. However, after the UE is handed-over to 3rd NG-RAN node, the target NG-RAN node (i.e. the NG-RAN node after UE’s 1st handover) can no longer provide the UE performance feedback to the source (i.e. the NG-RAN node before UE’s 1st handover). For such scenario, where the target NG-RAN node cannot continue to provide the requested information the source node needs to be informed about this..

One possible approach is to send a separate class-2 message indicating the unavailability/unsupported status of requested information. This message can include the corresponding measID and cause values.

Alternatively, such information can be carried over AI/ML Information Update message. As discussed above, partial reporting can be supported based on stage-2 procedure description. Therefore, once UE performance feedback becomes no longer available/unsupported, the AI/ML information update message can decide not to include such unavailable/unsupported UE performance feedback in the message.

It is possible that, during periodic reporting, the UE is handed-over to another cell or the requested information is no longer available/supported in the middle of reporting periodicity. For such situation, should the requested NG-RAN node still include the measured UE performance since the latest reporting time and send it to the requesting NG-RAN node?

If a separate class-2 message is used, the requesting NG-RAN node can trigger the reporting immediately after the data becomes unavailable. If multiple requested measurements become unavailable at different time, the requesting NG-RAN node can trigger messages individually. This approach is flexible but may introduce extra signaling overhead.

If AI/ML information update message is used, the measurement results measured since last reporting can still be sent together with other information in the next reporting time instance (based on periodicity). Recalling that RAN3 agreed UE performance feedback are average values among a certain period (e.g. average packet delay, average UE throughput DL, average UE throughput UL, average packet error rate, etc). The average measurement values since the latest reporting can be still useful for the requesting NG-RAN node for its model’s performance optimization.

Proposal 5: If a UE performance feedback measurement becomes no longer available/supported after a certain time (e.g., when UE moves out of this node), the requested NG-RAN node includes the measurement results since the latest reporting and the corresponding failure cause value in AI/ML information update message.

# Conclusion

In this contribution, we discussed the remaining issue on partial reporting, cause value and UE performance feedback.

We propose the following observations and proposals:

Proposal 1: No explicit IE is introduced in the AI/ML information request message to request partial response.  Requested node can provide partial results if it cannot provide full results.

Proposal 2: Introduce a list of failure measurements and the corresponding cause indicating the reason for failure per measurement in the AI/ML Information response message.

Observation 1: The detailed cause value provides more detailed failure reason on top of existing cause values. Even with detailed failure cause value, the requesting NG-RAN node still cannot encode suitable reporting periodicity and/or combination of requested information for successfully reception of measurement results, and may keep receiving failure messages.

Proposal 3: The requested NG-RAN node includes the following failure cause values and the desired values in the AI/ML Information Response message:

- measurement not supported with requested reporting periodicity and requested NG-RAN node’s supported value,

- measurement temporarily not available with requested reporting periodicity and requested NG-RAN node’s supported value,

- measurement not supported with current combination of requested information and requested NG-RAN node’s supported combination,

- measurement temporarily not available with current combination of requested information and requested NG-RAN node’s available combination.

Proposal 4: The configuration of reporting time for UE performance feedback should be consistent with or reuse the configuration for other requested information in AI/ML information request message.

Proposal 5: If a UE performance feedback measurement becomes no longer available/supported after a certain time (e.g., when UE moves out of this node), the requested NG-RAN node includes the measurement results since the latest reporting and the corresponding failure cause value in AI/ML information update message.

Reference

[1] TS36.423, Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)