3GPP TSG-RAN WG3 Meeting #121	R3-233913

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:		12.2.2.2

Source:	Intel Corporation

Title:	Discussion on mobility optimization

Document for:		Discussion and Decision

# Introduction

During the RAN3 #120 meeting, the following agreements and FFS were captured for UE trajectory prediction:

In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node, and the source NGRAN node can collect measured UE trajectory (in the form of UE History Information) related to one or multiple cells within the single next hop target NGRAN node after handing over the concerned UE(s).

Take the procedure for UE performance feedback report as the baseline to also support the measured UE trajectory collection by the source gNB.

The details on how to request the measured UE trajectory collection needs to be further discussed.

In this contribution, we mainly discuss the input information used for UE trajectory prediction and further usage of UE trajectory prediction for AI/ML based mobility optimization use case.

# Discussion

## UE trajectory Collection

It is still FFS how to request the measured UE trajectory collection by using UE performance feedback report as baseline.

During previous RAN3 discussion, there are two parts of UE trajectory collection. The first part is the UE trajectory history before UE’s handover, the second part is the actual UE trajectory after UE’s handover. For the first part, the source NG-RAN node can obtain the corresponding UE trajectory history over UHI and use such information as initial input for AI/ML training and inference. For the second part, similar as UE performance feedback, since the target NG-RAN node can only provide the actual UE’s trajectory after UE’s handover, UE trajectory can be considered as one of the UE performance feedback, e.g. all configurations, including reporting periodicity (periodic or one-time) and time configuration follow configurations for UE performance feedback.

Proposal 1: The agreed procedure for UE performance feedback is extended to support the future UE trajectory collection by the source gNB.

## Handover prediction

It was agreed in previous RAN3 meetings on UE trajectory prediction:

Cell-based UE Trajectory prediction is provided as a list of cells into the future, each of which is indicated together with an expected time of stay into the cell. UE Trajectory Prediction is transferred to the target gNB via the Handover Request. Predicted UE Trajectory conveyed in the Handover Request can span across multiple NG-RAN nodes. In Rel18, the UE trajectory prediction is only limited to the next one hop target NGRAN node

Based on different AI/ML model implementation, it is possible that the model is able to generate multiple predicted UE trajectories within the next one hop target NG-RAN node as output. If multiple predicted UE trajectories in cell level (within one NG-RAN node) can be generated, after the 1st handover, the target cell may have multiple options for its next handover. Hence, it would be beneficial for the source NG-RAN node to include some assistance information generated from UE trajectory prediction to help handover decision making at the target NG-RAN node for the follow-up hops.

Observation 1: Predicting multiple potential handover UE trajectory in cell level as the output of AI/ML based mobility optimization is possible. The source NG-RAN node can provide some assistance information to help handover decision making at the target NG-RAN node after 1st handover.

As part of the output, we think that the estimated arrival probability, priority, handover execution timing and duration of stay per predicted cell in the predicted UE trajectory can be also carried in the Handover Request message.

First of all, the predicted handover execution timing can be useful for the target cell, since the target is able to prepare resources for potential handover UEs in advance. Moreover, if some AI/ML model for other use cases is running in the target that can predict its future status (e.g., load), then the target may take its future status into account in deciding whether to accept handover request or not, especially at the timing of the received predicted handover execution time from the source.

Observation 2: Providing the predicted handover execution timing to the potential target cell has the following advantages: 1) the target can prepare resources for potential handover UEs; 2) the target cell may take its future status into account for admission control, at the received predicted handover execution timing.

In addition to the predicted handover execution timing, other predicted information (e.g. estimated arrival probability, priority, and confidence level) will be also useful for the target to optimize the handover decisions. For example, if the target NG-RAN node cannot host the UE to be handed-over, following legacy procedure, a Handover Preparation Failure message will be sent from the target to the source NG-RAN node. With the arrival probability and confidence level corresponding to the predicted UE trajectory (multi-hop cells), instead of refusing UE’s handover, the target NG-RAN node can directly handover the “rejected” UE to the cell (within the target NG-RAN node) with highest arrival probability/confidence level.

Proposal 2: For normal handover, the source NG-RAN node provides the predicted information (e.g. estimated arrival probability, priority, handover execution timing, confidence level, etc.) in the Handover Request message associated with the predicted target cell(s).

# Conclusion

In this contribution, we discussed the remaining opens for mobility optimization use cases.

We propose the following observations and proposals:

Proposal 1: The agreed procedure for UE performance feedback is extended to support the future UE trajectory collection by the source gNB.

Observation 1: Predicting multiple potential handover UE trajectory in cell level as the output of AI/ML based mobility optimization is possible. The source NG-RAN node can provide some assistance information to help handover decision making at the target NG-RAN node after 1st handover.

Observation 2: Providing the predicted handover execution timing to the potential target cell has the following advantages: 1) the target can prepare resources for potential handover UEs; 2) the target cell may take its future status into account for admission control, at the received predicted handover execution timing.

Proposal 2: For normal handover, the source NG-RAN node provides the predicted information (e.g. estimated arrival probability, priority, handover execution timing, confidence level, etc.) in the Handover Request message associated with the predicted target cell(s).