3GPP TSG-RAN WG3 Meeting #121	R3-234162

Toulouse, France, 21st – 25th August 2023

Agenda item:	  25.2

Source:		Lenovo

Title:		Discussion on PDU Set QoS Parameters and PDU Set Information

Document for:		Discussion and Decision

- Introduction

PDU Set concept has been introduced for XR service. This paper discusses the potential impact on NG, Xn, F1 and E1 interfaces.

- Discussion

2.1   NG Aspects

- PDU Set Concept in NG Interface

As specified in TS 23.501, PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling.

The following PDU Set QoS Parameters are specified:

1.	PDU Set Delay Budget (PSDB). PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). The AN PSDB is derived at NG-RAN by subtracting CN PDB from the PSDB

2.	PDU Set Error Rate (PSER). PSER defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access)

3.	PDU Set Integrated Handling Information (PSIHI). The PSIHI indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter.

The SMF sends them to NG-RAN as part of the QoS parameters.

From RAN3 point of view, the PDU Set QoS Parameters should be per QoS flow and provided in the QoS Flow Level QoS Parameters IE of the PDU Session Resource Setup Request Transfer IE, PDU Session Resource Modify Request Transfer IE, and Path Switch Request Acknowledge Transfer IE.

PDU Set QoS Parameters (PSDB, PSER and PSIHI) per QoS flow are provided by NG-C signalling in the QoS Flow Level QoS Parameters IE in NG-AP.

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

From RAN3 point of view, the PDU Set Information should be added in the ‘PDU Session Container’ GTP-U extension header in TS 38.415.

PDU Set Information (PDU Set Sequence Number, Indication of End PDU of the PDU set, PDU Sequence Number within a PDU Set, PDU Set Size and PDU Set Importance) is provided in the GTP-U extension header, i.e., PDU Session container as specified in TS 38.415.

- Non-homogenous support PDU Set based handling in NG-RAN

- By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. It means that the NG-RAN needs to provide the SMF with an indication of whether the PDU Set QoS parameter is accepted or not in the PDU Session Resource Setup Response Transfer IE and PDU Session Resource Modify Response Transfer IE.

- In case of Xn handover, the source gNB may provide PDU Set QoS parameters to the target gNB in Handover Request message, the target gNB also performs admission control of the PDU set. It is possible that the target gNB may accept or reject PDU Set QoS parameter but accept the QoS flow. In this case, the target gNB also needs to provide the indication of whether the PDU Set QoS parameter is accepted or not to SMF in the Path Switch Request Transfer IE.

- Similar for NG handover, the target gNB needs to provide the indication of whether the PDU Set QoS parameter is accepted or not to SMF in the Handover Request Acknowledge Transfer IE.

NG-RAN node provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not in the PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE, Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

- At NG-RAN Xn handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking. It means that the target NG-RAN node provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling in the Path Switch Request Transfer IE.

Target NG-RAN node provides to SMF with an indication of whether target NG-RAN node supports PDU Set based handling in the Path Switch Request Transfer IE.

- In the case of handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, there would be temporarily both PDU set unmarked packets and PDU set marked packets in the target NG-RAN node. For example, the source gNB may forward downlink PDU set unmarked PDCP SDUs or SDAP SDUs to the target gNB and after path switch request procedure the downlink PDU set marked data may arrive at the target NG-RAN node. For uplink, the UE may also have both PDU set unmarked packets and PDU set marked packets temporarily to be transmitted to the target NG-RAN node.

RAN3 discusses how to handle temporarily having both PDU set unmarked packets and PDU set marked packets of a QoS flow during handover for both DL and UL.

- End of Data Burst

- SA2 has defined a data burst that is a set of multiple PDUs generated and sent by the application in a short period of time. A Data Burst can be composed of one or multiple PDU Sets. An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g., to configure UE power management schemes like connected mode DRX. RAN2 has also confirmed that the indication of End of Data Burst is beneficial e.g. by sending the UE entering DRX sleep state.

End of Data Burst indication is provided to the in the GTP-U extension header of the last PDU of the date burst, i.e., PDU Session container as specified in TS 38.415.

- XR traffic assistance information

- As specified in TS 23.501, The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

- The DL periodicity associated N6 jitter indicates the positive or negative deviation of the arrival time of first packet of a Data Burst compared to the ideal Data Burst start time which is be determined based on the DL periodicity.

- The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI.

UL and DL Periodicity of XR traffic per QoS flow is provided by SMF e.g., by reusing TSC Traffic Characteristics IE.

DL data arrive time and DL jitter range associated with DL periodicity of flow is provided by SMF e.g., by reusing TSC Traffic Characteristics IE.

- RAN2#122 meeting has discussed UL assistant information and reached the following:

- UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity). - Do not use PIN delay budget request for jitter reporting for XR services.

UL data arrive time and UL jitter range are reported by UE as agreed by RAN2.

2.2	Xn Aspects

- Xn handover and data forwarding

- For Xn handover, the source gNB needs to provide PDU Set QoS parameters to the target gNB in the HANDOVER REQUEST message i.e. in the QoS Flow Level QoS Parameters IE.

PDU Set QoS Parameters (PSDB, PSER, PSIHI) per QoS flow are provided from source gNB to target gNB in the Handover Request message.

- There are two types of data forwarding, DRB based data forwarding and PDU session based data forwarding. As in the legacy, we believe that both DRB based data forwarding and PDU session based data forwarding should be supported. For PDU session based data forwarding, the PDU Set Information is added in the in the GTP-U extension header, i.e., PDU Session container as specified in TS 38.415. However, for DRB based data forwarding, how to provide the PDU Set Information needs to be discussed, which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

For DRB based data forwarding, the PDU Set Information should added in the GTP-U extension header. RAN3 to discuss which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

- PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). The AN PSDB is derived at NG-RAN by subtracting CN PDB from the PSDB. Then AN-PSDB represents the difference between the time when the first PDU of the PDU Set is arrived at the gNB and the time when the last PDU or all PDUs of the PDU Set has been delivered to the UE successfully. However, handover may happen during transmission of a PDU set. There is a case that partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB. In this case, the target gNB should be aware of the remaining AN-PSDB for transmitting the remaining PDUs of the PDU set for more accurate QoS management and scheduling.

- Figure 1. Handover happens during data transmission of a PDU Set

RAN3 discusses how the target gNB is aware of the remaining AN-PSDB of a PDU set if partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB.

- To support PDU Set based QoS handling, an indication of End PDU of the PDU set is provided by UPF in the GTP-U header. In order to support lossless handover, data forwarding from source gNB to target gNB is performed. How the target gNB knows the complete transmission of a PDU set needs to be solved at least in the following case:

-	The source gNB receives the End PDU of a PDU set from UPF.

-	Before handover, the End PDU (PDU#3) has been transmitted to UE successfully, but a PDU (PDU#2) before the End PDU have not been acknowledged from UE.

-	Currently, the source gNB shall forward PDU#2 to the target gNB during data forwarding.

-	Since the End PDU is not forwarded to the target gNB, the target gNB does not know whether the transmission of the PDU set is complete and thus is not able to perform PDU set integrated handling for the PDU set

- Figure 2. Issue of End PDU Indication during handover

RAN3 discusses how to handle End PDU Indication during data forwarding if the End PDU has been transmitted to UE in the source side but there are still other PDUs to be transmitted of the PDU Set.

- NR-DC

- It is still not clear whether PDU Set related handling is supported in NR-DC in this release. If NR-DC is supported, the PDU Set QoS parameters needs to be sent to S-NG-RAN node as a part of QoS Flow Level QoS Parameters. PDU Set Information may also need to be sent to the peer node depends on the bearer type. For example, for MN terminated split bearer, the MN needs sends the PDU Set Information in the GTP-U extension header of PDCP PDU.  The standard impact should be the same as handover.

2.3	F1 Aspects

- In case of CU-DU split, the gNB-CU performs QoS flows to DRB mapping. As discussed by RAN2, both 1:1 and N:1 mapping between QoS flow and DRB should be supported. In F1AP, for a DRB to be setup, both DRB level QoS and QoS flow level QoS are provided. Similarly, both DRB level and QoS flow level PDU Set Parameters should be provided from the gNB-CU to the gNB-DU. Both DRB level and QoS flow level QoS parameters are use the same IE QoS Flow Level QoS Parameters.

The PDU Set Parameters are introduced in the QoS Flow Level QoS Parameters IE in F1-AP.

- In NG interface, the PDU Set Information is included in NG-U header per QoS flow. Since the gNB-CU transmits PDCP PDU per DRB to the gNB-DU, the PDU Set Information is removed before processing be PDCP layer. The PDU Set Information needs to be re-added in the GTP-U extension header of PDCP PDU per DRB.

PDU Set Information should added in the GTP-U extension header of PDCP PDU per DRB in F1-U. RAN3 to discuss which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

2.3	E1 Aspects

- For E1AP, similar with F1, the PDU Set parameters needs to be introduced in the QoS Flow Level QoS Parameters IE. No other E1AP impact is foreseen except PSI based PDCP discard that is discussed in a separate paper.

The PDU Set Parameters are introduced in the QoS Flow Level QoS Parameters IE in E1-AP.

3	Conclusion

This paper discusses the potential impact on NG, Xn, F1 and E1 interfaces to support PDU Set concept.

NG Aspects:

- PDU Set QoS Parameters (PSDB, PSER and PSIHI) per QoS flow are provided by NG-C signalling in the QoS Flow Level QoS Parameters IE in NG-AP.

PDU Set Information (PDU Set Sequence Number, Indication of End PDU of the PDU set, PDU Sequence Number within a PDU Set, PDU Set Size and PDU Set Importance) is provided in the GTP-U extension header, i.e., PDU Session container as specified in TS 38.415.

NG-RAN node provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not in the PDU Session Resource Setup Response Transfer IE, PDU Session Resource Modify Response Transfer IE, Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

Target NG-RAN node provides to SMF with an indication of whether target NG-RAN node supports PDU Set based handling in the Path Switch Request Transfer IE.

RAN3 discusses how to handle temporarily having both PDU set unmarked packets and PDU set marked packets of a QoS flow during handover for both DL and UL.

End of Data Burst indication is provided to the in the GTP-U extension header of the last PDU of the date burst, i.e., PDU Session container as specified in TS 38.415.

UL and DL Periodicity of XR traffic per QoS flow is provided by SMF e.g., by reusing TSC Traffic Characteristics IE.

DL data arrive time and DL jitter range associated with DL periodicity of flow is provided by SMF e.g., by reusing TSC Traffic Characteristics IE.

UL data arrive time and UL jitter range are reported by UE as agreed by RAN2.

Xn Aspects

PDU Set QoS Parameters (PSDB, PSER, PSIHI) per QoS flow are provided from source gNB to target gNB in the Handover Request message.

For DRB based data forwarding, the PDU Set Information should added in the GTP-U extension header. RAN3 to discuss which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

RAN3 discusses how the target gNB is aware of the remaining AN-PSDB of a PDU set if partial PDUs of the PDU set has been successfully transmitted in the source gNB, while the other PDUs of the PDU set will be transmitted by the target gNB.

RAN3 discusses how to handle End PDU Indication during data forwarding if the End PDU has been transmitted to UE in the source side but there are still other PDUs to be transmitted of the PDU Set.

F1 Aspects

The PDU Set Parameters are introduced in the QoS Flow Level QoS Parameters IE in F1-AP.

PDU Set Information should added in the GTP-U extension header of PDCP PDU per DRB in F1-U. RAN3 to discuss which GTP-U extension header is used, i.e., RAN container as specified in 38.425 v.s. PDU session container as specified in TS 38.415.

E1 Aspects

The PDU Set Parameters are introduced in the QoS Flow Level QoS Parameters IE in E1-AP.

Reference

[1] TS 23.501.