# 3GPP TSG-RAN WG3 Meeting #121	R3-234275

# 21-25 August 2023, Toulouse, France

Agenda Item:	25.2

Source:	Ericsson

Title:	Discussion on XRM support

Document for:	Discussion, Other

# Introduction

In RAN#99, the Work Item on "XR Enhancements for NR" was agreed in [1], which includes the following RAN3-related objectives:

Specify the enhancements related to capacity: - […] - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - […] - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In this contribution we go through the subtopics of the R18 eXtended Reality (XR) work item and highlight the RAN3 impacts, taking into consideration other WGs agreements and progress.

Meanwhile, SA2 has introduced main content for R18 XR in relevant TSs under their responsibility, RAN3 is receiving information explicitly in an LS in [2].

# Discussion

## XR Awareness

### PDU Set QoS Parameters and PDU Set Information

According to TS 23.501 V18.2.2, new Information Elements (IEs) related to the XR PDU Set concept need to be signalled from 5GC to NG-RAN. The PDU Set related IEs are carried via CP signalling over NGAP, in this case they are part of the QoS profile and are referred to as "PDU Set QoS parameters"; and via UP signalling over GTP-U extension header and are referred to as "PDU Set Information".

In the final Annex of this paper, we provide extracts from TS 23.501 V18.2.2 describing the PDU Set QoS parameters and PDU Set Information and their usage by NG-RAN. We summarize all the relevant information from the spec in the below table:

| PDU Set QoS Parameters (CP) | PDU Set Information (UP) |
| --- | --- |
| - PDU Set Error Rate (PSER) (Optional) - PDU Set Delay Budget (PSDB) (Optional) - PDU Set Integrated Handling Indication (PSIHI) indicates not all PDUs are needed to NG-RAN (Optional) | - PDU Set Sequence Number (the PDU Set Identifier) - Indication of End PDU of the PDU Set (Optional) - PDU Sequence Number within a PDU Set (Optional) - PDU Set Size in bytes - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow (Optional) |

Observation 1: The following PDU Set QoS Parameters, all optional IEs, need to be signalled over NGAP to NG-RAN:

- PDU Set Error Rate (PSER) UL and DL

- PDU Set Delay Budget (PSDB) UL and DL

- PDU Set Integrated Handling Indication (PSIHI)

Observation 2: The following PDU Set Information need to be signalled over GTP-U Ext to NG-RAN:

- PDU Set Sequence Number

- Indication of End PDU of the PDU Set (Optional)

- PDU Sequence Number within a PDU Set (Optional)

- PDU Set Size in bytes

- PDU Set Importance (Optional)

Furthermore, we note the SA2 liaison in [2] where SA2 have provided the following stage 2 description regarding the issue of non-homogeneous deployments of NG-RAN nodes supporting or not the PDU Set concept:

#### 5.37.5.3 Non-homogenous support of PDU set based handling in NG-RAN By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set. Editor's note: In the case of handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, whether there is a reason for the target NG-RAN node to temporarily handle both unmarked PDUs and marked PDUs within the same QoS flow is FFS.

As we can see, this can be categorized as yet another discussion on how to address non-homogeneous support of a certain feature in the network, which RAN3 has discussed several times over the past releases. Although RAN3 has a long-standing principle to not specify exchange of supported capabilities between different nodes in the network and to rely on OAM and the principle of criticalities as described in Chapter 10 of TS 38.413, we can remark that SA2 has decided to explicitly specify yet another solution where the NG-RAN sends a support indication to SMF when it receives one of the optional PDU Set QoS parameters and accepts it or not, i.e., indicating that it supports handling it.

Observation 3 : According to SA2, upon receiving a PDU Set QoS parameter for a given QoS flow, if the receiving NG-RAN supports PDU Set handling, it sends an ACK indication to SMF

Obviously, such solution needs first to be discussed in RAN3 as it may require adding signalling support in NGAP. The SA2 LS action indicates: “SA2 kindly asks RAN3/CT4 to take the above into account in their work and provide feedback as deemed necessary”.

We list below based on our analysis the different options to support this issue:

- Option 1 : Doing nothing, rely on OAM and Criticalities principle. This option challenges SA2 agreement.

- Option 2 : Going ahead with SA2’s agreed CR [3]. Although the signalling can be better enhanced to fit with NGAP design. For instance, the SMF should signal one or more of the PDU Set QoS parameters in the QoS profile - or instead, the SMF signals a new PDU Set Handling indication IE in the PDU SESSION RESOURCE SETUP/MODIFY REQUEST messages to enquire gNB about its capability support - and gNB replies with a new PDU Set handling support Indication IE to indicate its supported capability. We remark that similar solution exists for MBS on PDU Session level (see MBS Support Indicator IE in 9.3.1.210). The new PDU Set handling support indication IE can follow similar design.

- Option 3: Another option, which can be considered more future-proof, is to follow an equivalent approach as was agreed for RACS capability exchange in Rel-17 RAN3#116-e meeting: while the “RACS solution” aims at exchanging support information via the transparent HO containers during CN based HO between source and target NG-RAN node, based on NGAP IE-Ids, option 3 would foresee introducing a mechanism for the SMF to request support information from the NG-RAN node, i.e. exchanging information of the supported (NGAP) IE-Id during a given PDU Session Management procedure, For example, during the PDU Session resource Setup procedure (Figure 1), or during the establishment of PDU Session resources in the course of a CN based HO (Figure 2) or by relaying the SMF’s collected request to the target NG-RAN node during an Xn based HO. (Figure 3).

Figure 1: SMF query of RAN support during PDU Session resource Setup/Modify procedures

Figure 2: SMF query of RAN support during the establishment of PDU Session resources in the course of a CN based HO

Figure 3: Target node relaying the SMF’s collected request during an Xn based HO to the SMF

The basic principle remains the same: option 3 represents a general approach for exchanging support information based on protocol IE-IDs (of IEs that were introduced in order to support a certain (new) feature).
By that, it is possible to very much re-use the NGAP Protocol IE-Id and NGAP Protocol IE Support Information IEs already existing in NGAP, and they will be exchanged on a PDU session ID level over NGAP and XnAP to inform SMF about RAN capabilities for all supported features (of course, it is also possible to introduce a separate set of IEs, if preferred):

#### 9.3.1.X1 NGAP IE Support Information Request List This IE provides information about requesting support of functions associated to a list of NGAP Protocol IE-Ids. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NGAP IE Support Information Response Item; 1..<maxnoofIESupportInfo>. >NGAP Protocol IE-Id; M; 9.3.1.239. Range bound; Explanation. maxnoofIESupportInfo; Maximum no. of IE Support Information. Value is 32.. #### 9.3.1.X2 NGAP IE Support Information Response List This IE provides information about support of functions associated to a list of NGAP Protocol IE-Ids. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NGAP IE Support Information Response Item; 1..<maxnoofIESupportInfo>. >NGAP Protocol IE-Id; M; 9.3.1.239. >NGAP Protocol IE Support Information; M; 9.3.1.240. Range bound; Explanation. maxnoofIESupportInfo; Maximum no. of IE Support Information. Value is 32..

The new NGAP IE Support Information Request List can be introduced in:

- PDU Session Resource Setup Request Transfer IE

- PDU Session Resource Modify Request Transfer IE

The new NGAP IE Support Information Response List IE can be introduced in:

- the NGAP-SMF related IEs:

  - PDU Session Resource Setup Response Transfer

  - PDU Session Resource Modify Response Transfer

  - Path Switch Request Transfer (for the case of inter-node handover)

  - Handover Request Acknowledge Transfer (for the case of inter-node handover)

- And in the XnAP SMF-related IE :

  - PDU Session Resources To Be Setup List (for inter-node handover, CONNECTED and INACTIVE)

We consider that Option 3 is more future-proof, since it allows to introduce the same mechanism for capability support information that was introduced for RACS on AMF level, but now on a PDU Session ID level, to inform SMF about NG-RAN supported capabilities.

Proposal 1 : RAN3 to discuss the options to support Non-homogenous support of PDU set based handling in NG-RAN and to agree on a similar design as the one adopted for RACS, i.e., re-using the NGAP Protocol IE-Id and NGAP Protocol IE Support Information IEs existing in NGAP, to be signalled on a PDU Session ID level in the SMF related IEs

A LS to SA2 with RAN3 preferred option to support the case of non-homogeneous network deployment is provided in Annex 2 of this document.

Regarding mobility case, the source NG-RAN node needs to indicate the PDU Set QoS parameters to be used by the target gNB during handover (Xn-based handover, or NG-based Handover procedure). The target NG-RAN should inform the SMF whether it accepts or not the PDU Set QoS parameters. We understand that this should happen during the PATH SWITCH REQUEST message in case of Xn HO. The SMF may, based on this knowledge, indicate the updated PDU Set QoS Parameters which can be used by the new NG-RAN to replace the parameters received by the source NG-RAN, in case target node cannot fulfil them. In that case, we need to discuss whether explicit IEs on the PDU Set QoS parameters (PDSB DL/PDSB UL(FFS)/ PDER/ PDIHI) need to be indicated to the AMF, so that AMF can signal new values to NG-RAN during the PATH SWITCH REQUEST ACKNOWLEDGE message.

Proposal 2: RAN3 to discuss the signalling of the PDSB DL/PDSB UL(FFS)/ PDER/ PDIHI in the PATH SWITCH REQUEST message following a handover

Considering that PDU Set QoS parameters are part of the QoS profile, the PDU Session Notification procedure may also need to be enhanced to add the parameters in the PDU Session Resource Notify Transfer IE to SMF

Proposal 2bis: RAN3 to discuss the signalling of the PDU Set QoS parameters in the PDU Session Resource Notify Transfer IE

### XR traffic assistance information

Some information have been agreed to be signalled to NG-RAN over CP and UP in the context of UE Power saving using the TSC Assistance Information (TSCAI) framework. The TSCAI are already defined in NGAP 9.3.1.131 including the periodicity, Burst arrival time and Survival time. TS 23.501 V18.2.2 describes further assistance information to be signalled from 5GCN to NG-RAN, which we capture in the annex section of this document and that we summarize in the table below.

| CP | UP |
| --- | --- |
| - UL and/or DL traffic Periodicity of the QoS flow; - N6 Jitter Information associated with the Periodicity (Optional) | - Optional, End of Data Burst indication in the header of the last PDU of the Data Burst |

We can see that in NGAP TSC Assistance Information already includes the periodicity Information, which is defined in a generic way. However we should add indication of associated Jitter information:

#### 9.3.1.131 TSC Assistance Information This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]). IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Periodicity; M; 9.3.1.132; -. Burst Arrival Time; O; 9.3.1.133; -. Survival Time; O; 9.3.1.221; YES; ignore. #### 9.3.1.132 Periodicity This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Periodicity; M; INTEGER (0..640000, …); Periodicity expressed in units of 1 us.. N6 Jitter Information; O; INTEGER(FFS – pending RAN2); indicates the positive or negative deviation of the arrival time of first packet of a Data Burst compared to the ideal Data Burst start time which is be determined based on the DL periodicity; YES; ignore.

Observation 4: The following TSCAI info needs to be signalled over NGAP to NG-RAN:

- N6 Jitter Information associated with the DL Periodicity

Observation 5: The following UE power saving info needs to be signalled over GTP-U Ext to NG-RAN:

- End of Data Burst indication in the header of the last PDU of the Data Burst

Furthermore, to change the XR TSCAI periodicity dynamically, the information needs to be signalled during the setup or the modification of the PDU session procedure from SMF to NG-RAN, which requires to run the PDU Session Management procedure over many network interfaces (NG, F1, E1, Xn, Uu). This is too onerous in terms of signalling resources, especially for frequent changes of the periodicity value. We think that the periodicity information should be also signalled in-band, in the GTP-extension header, in order to let NG-RAN be aware rapidly of its change and use it for scheduling of DL traffic resources.

Observation 5bis: in case of frequent change of periodicity information, different network interfaces need to run the PDU Session management procedures, this can be too onerous in terms of signalling

Proposal 3 : RAN3 to discuss the addition of the periodicity information in in-band signalling from UPF to NG-RAN

SA2 has also answered to RAN2 in [2] the following regarding whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR:

SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

In our previous contribution [3], we have analysed the RAN3 impacts if SA2 was to provide such answer. In NGAP, the TSCAI is provided as part of the TSC Traffic Characteristics IE (TS 38.413 section 9.3.1.130), which is included in the PDU Session Resource Setup Request Transfer (TS 38.413 section 9.3.4.1) and PDU Session Resource Modify Request Transfer (section 9.3.4.3) IEs. There, we can see that the IE is bound only to GBR QoS flows and is ignored otherwise by the NG-RAN.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

NG-RAN node according to the current specification ignores the TSCAI IE when the QoS flow is non-GBR. The TSCAI related function will not be performed but the QoS flow may be setup successfully.

Observation 6: In Release 17, the TSCAI in NGAP is valid only for GBR QoS flow and is ignored otherwise

Now that SA2 clarified that the TSCAI in the context of XR can be used for both GBR and non-GBR QoS flows, the NGAP limitation needs to be removed. One possible change is to delete the "semantics description" in NGAP. There is no other change related to ASN.1 and the change can be considered ASN.1 backward compatible.

Observation 7: The limitation of tying TSCAI to GBR QoS flows can be lifted by deleting the semantics description in the TSC Traffic Characteristics IE

However, from functionality perspective, a non-upgraded gNB may be implemented to ignore the received TSC Traffic Characteristics in case of non GBR QoS flow, and in this case the IE might be ignored and cause performance degradation. Therefore, another option is to define a new XR TSC Traffic Characteristics IE in 9.3.4.1 and 9.3.4.3, without the semantics.

Observation 8: A non-upgraded gNB may be implemented to always ignore the TSC Traffic Characteristics for non-GBR QoS flow even if the semantics is removed

Proposal 4: RAN3 to discuss the options of either defining a new XR TSC Traffic Characteristics IE in 9.3.4.1 and 9.3.4.3 or removing the semantics to support TSCAI for non-GBR

A TP to XR NGAP BL CR with possible changes is proposed in the Annex 1 of this paper for consideration.

Furthermore, similar to signalling of the PDU Set QoS parameters are a handover, we think 5GCN should signal new UE XR Traffic Characteristics such as periodicity, jitter and burst arrival in time to be used for scheduling to NG-RAN in the PATH SWITCH REQUEST ACKNOWLEDGE message.

Proposal 5: RAN3 to discuss the addition of UE XR Traffic Characteristics such as periodicity, jitter and burst arrival in time to be used for scheduling to NG-RAN in the PATH SWITCH REQUEST ACKNOWLEDGE message

Considering that TSCAI are part of the QoS profile, the PDU Session Notification procedure may also need to be enhanced to add the parameters in the PDU Session Resource Notify Transfer IE to SMF

Proposal 5bis: RAN3 to discuss the signalling of the TSCAI parameters in the PDU Session Resource Notify Transfer IE

## Other impacted interfaces

Other interfaces need to be impacted to support XR awareness and signalling of TSCAI, notably:

### XnAP

- Addition of the PDU Set QoS parameters in the  QoS Flow Level QoS Parameters IE (9.2.3.5)

- Update the TSC Traffic Characteristics IE (9.2.3.114) with new TSCAI information for XR

### F1AP

- Addition of the PDU Set QoS parameters in the QoS Flow Level QoS Parameters IE (9.3.1.45)

- Update the TSC Traffic Characteristics IE (9.3.1.141) with new TSCAI information for XR

### E1AP

- Addition of the PDU Set QoS parameters and update of the TSC Traffic Characteristics with the new TSCAI XR information in the QoS Flow QoS Parameters List IE (9.3.1.25)

### F1-U/Xn-U

Considering the DU is in charge of scheduling, and thus of dropping of packets in congestion, the PDU Set Information needs to be signalled over F1-U for scheduling purpose.

Considering the BL CR allocation made by Rapporteur, we provide a BLCR capturing the XnAP changes in [4].

Proposal 6: Discuss the RAN3 changes to other interfaces Xn, E1, F1 and to TS 38.425 to support XR

## Discard Operation

Related to the other WI Objective:

Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

In the current NG-RAN UP protocol, the discard from the PDCP entity to the corresponding node can be performed per PDCP PDU. Now there is a need to allow discarding on PDU Set level.

Proposal 7 RAN3 to discuss to introduce discarding on PDU Set SN over NG-RAN User Plan protocol.

For discarding of packets in congestion, the PDU Set Information needs to be signalled over F1-U for scheduling purpose. This is aprt of the F1-U impacts.

## User Plane design

The PDU Set Information introduced in XRM could be included in the existing GTP-U header extension “PDU Session Container”, which content is specified in TS 38.415, or the “NR RAN Container”, which content is specified in TS 38.425. However, the case of Non-homogenous support of PDU set based handling in NG-RAN must be considered. Especially, this would entail that the XR User Plane has to be designed in a way that a node receiving PDU Set  Information within the user plane packets is able to cope with protocol additions without being forced to discard the whole User Plane packet, so that data forwarding from source to target NG-RAN node is possible  in case the source node is supporting “PDU set based QoS handling” while the target is not. Currently neither the PDU Session User Plane protocol (TS 38.415) nor the NR User Plane protocol (TS 38.425) support Non-homogenous deployments of (user plane) protocol additions, as requested in XR.

Observation 9: The XR User Plane signalling must be designed in a way to consider to Non-homogenous support of PDU set based handling in NG-RAN. However, neither the PDU Session User Plane protocol (TS 38.415) nor the NR User Plane protocol (TS 38.425) support Non-homogenous deployments of (user plane) protocol additions, as requested in XR.

The PDU Set related information in the user plane has therefore to be defined in a backwards compatible way, so that when it is sent to the node that does not understand this set of parameters, the node could ignore the set and proceed as it is handling the normal NR data payload (i.e. without XR feature). One way to achieve this is to introduce the PDU Set related information in a new GTP-U Extension Header, for example defined as “PDU Set Container” that can be specified in TS 38.415 and/or TS 38.425 [Note: it is probably sufficient to specify the Radio Network Layer content of this assumed new GTP-U extension header in a single TS, also creating a new TS could be an option]. This way, in case of non-homogenous deployment, the new GTP-U Extension Header “PDU Set Container” present, the GTP-U Extension Header PDU Session Container shall be used when the operation is related to normal data payload transfer and the new “GTP-U Extension Header PDU Set Container” for XR related data transfer. When the data payload is sent from the supporting Node to the non-supporting node, the non-supporting node could ignore the PDU Set Container and use the PDU Session container as a fallback. This obviously requires extending TS 38.415 and/or TS 38.425.

Proposal 8: Instead of reusing the existing PDU session container, RAN3 to discuss the introduction of a new “PDU Set Container” that can be specified in TS 38.415 and/or TS 38.425 in a backward compatible way to support the case of Non-homogenous deployment.

A LS to CT4 and SA2 with RAN3 reasoning of defining a new PDU Set container in the GTP-U header is provided in Annex 3 of this document.

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: The following PDU Set QoS Parameters, all optional IEs, need to be signalled over NGAP to NG-RAN:

- PDU Set Error Rate (PSER) UL and DL

- PDU Set Delay Budget (PSDB) UL and DL

- PDU Set Integrated Handling Indication (PSIHI)

Observation 2: The following PDU Set Information need to be signalled over GTP-U Ext to NG-RAN:

- PDU Set Sequence Number

- Indication of End PDU of the PDU Set (Optional)

- PDU Sequence Number within a PDU Set (Optional)

- PDU Set Size in bytes

- PDU Set Importance (Optional)

Observation 3 : According to SA2, upon receiving a PDU Set QoS parameter for a given QoS flow, if the receiving NG-RAN supports PDU Set handling, it sends an ACK indication to SMF

Proposal 1 : RAN3 to discuss the options to support Non-homogenous support of PDU set based handling in NG-RAN and to agree on a similar design as the one adopted for RACS, i.e., re-using the NGAP Protocol IE-Id and NGAP Protocol IE Support Information IEs existing in NGAP, to be signalled on a PDU Session ID level in the SMF related IEs

Proposal 2: RAN3 to discuss the signalling of the PDSB DL/PDSB UL(FFS)/ PDER/ PDIHI in the PATH SWITCH REQUEST message following a handover

Proposal 2bis: RAN3 to discuss the signalling of the PDU Set QoS parameters in the PDU Session Resource Notify Transfer IE

Observation 4: The following TSCAI info needs to be signalled over NGAP to NG-RAN:

- N6 Jitter Information associated with the DL Periodicity

Observation 5: The following UE power saving info needs to be signalled over GTP-U Ext to NG-RAN:

- End of Data Burst indication in the header of the last PDU of the Data Burst

Observation 5bis: in case of frequent change of periodicity information, different network interfaces need to run the PDU Session management procedures, this can be too onerous in terms of signalling

Proposal 3 : RAN3 to discuss the addition of the periodicity information in in-band signalling from UPF to NG-RAN

Observation 6: In Release 17, the TSCAI in NGAP is valid only for GBR QoS flow and is ignored otherwise

Observation 7: The limitation of tying TSCAI to GBR QoS flows can be lifted by deleting the semantics description in the TSC Traffic Characteristics IE

Observation 8: A non-upgraded gNB may be implemented to always ignore the TSC Traffic Characteristics for non-GBR QoS flow even if the semantics is removed

Proposal 4: RAN3 to discuss the options of either defining a new XR TSC Traffic Characteristics IE in 9.3.4.1 and 9.3.4.3 or removing the semantics to support TSCAI for non-GBR

Proposal 5: RAN3 to discuss the addition of UE XR Traffic Characteristics such as periodicity, jitter and burst arrival in time to be used for scheduling to NG-RAN in the PATH SWITCH REQUEST ACKNOWLEDGE message

Proposal 5bis: RAN3 to discuss the signalling of the TSCAI parameters in the PDU Session Resource Notify Transfer IE

Proposal 6: Discuss the RAN3 changes to other interfaces Xn, E1, F1 and to TS 38.425 to support XR

Proposal 7 RAN3 to discuss to introduce discarding on PDU Set SN over NG-RAN User Plan protocol.

Observation 9: The XR User Plane signalling must be designed in a way to consider to Non-homogenous support of PDU set based handling in NG-RAN. However, neither the PDU Session User Plane protocol (TS 38.415) nor the NR User Plane protocol (TS 38.425) support Non-homogenous deployments of (user plane) protocol additions, as requested in XR.

Proposal 8: Instead of reusing the existing PDU session container, RAN3 to discuss the introduction of a new “PDU Set Container” that can be specified in TS 38.415 and/or TS 38.425 in a backward compatible way to support the case of Non-homogenous deployment.

# References

RP-230786, Updated WID on XR Enhancements for NR, RAN#99

R3-233735, Non-homogeneous deployment of PDU Set based handling, LS, SA2

R3-233056, Discussion on TSCAI for XR, Ericsson

R3-234276, Addition of XRM enhancements, proposed BL CR to TS 38.423, Ericsson

R3-234277, LSes to CT4 and SA2

# Annexes

## Annex 1 - TP to NGAP BL CR

START OF CHANGE

#### 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Not Possible | O |  | 9.3.1.63 | This IE may be present in case of HANDOVER REQUEST message and is ignored otherwise. | YES | reject |
| PDU Session Type | M |  | 9.3.1.52 |  | YES | reject |
| Security Indication | O |  | 9.3.1.27 |  | YES | reject |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Setup Request List |  | 1 |  |  | YES | reject |
| >QoS Flow Setup Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 |  | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.64 | This IE may be present in case of inter-system handover and intra-system handover. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |
| MBS Session Setup Request List | O |  | 9.3.1.211 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

NEXT CHANGE

#### 9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| UL NG-U UP TNL Modify List |  | 0..1 |  |  | YES | reject |
| >UL NG-U UP TNL Modify Item |  | 1..<maxnoofMultiConnectivity> |  | This IE(s) are included only for modification of an existing tunnel. | - |  |
| >>UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | - |  |
| >>DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node. | - |  |
| >>Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| >>Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the NG-RAN node for the redundant transmission. | YES | ignore |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  | YES | reject |
| >QoS Flow Add or Modify Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 |  | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 |  | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of UL PDUs for split PDU session. | YES | reject |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s) proposed for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission of the Redundant QoS Flow(s). | YES | ignore |
| Security Indication | O |  | 9.3.1.27 |  | YES | ignore |
| MBS Session Setup or Modify Request List | O |  | 9.3.1.212 |  | YES | ignore |
| MBS Session To Release List | O |  | 9.3.1.215 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |

## Annex 2 - Draft LS to SA2

3GPP TSG-RAN WG3 Meeting #121	R3-23xxx1

21-25 August 2023, Toulouse, France

Title:	Reply LS on Non-homogeneous deployment of PDU Set based handling

Response to:	R3-233735/S2-2308252

Release:	Release 18

Work Item:	XRM

Source:	RAN3

To:	SA2, CT4

Cc:	RAN2

Contact Person:

- Name:		Yazid Lyazidi

- E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 of their LS of non-homogeneous deployments of NG-RAN nodes supporting PDU Set based handling. Based on the discussion, RAN3 has identified three solutions to address the issue:

- Option 1 : Doing nothing, rely on OAM and Criticalities principle. This option challenges SA2 agreement.

- Option 2 : the SMF signals a new PDU Set Handling indication IE in the PDU SESSION RESOURCE SETUP/MODIFY REQUEST messages to enquire gNB about its capability support - and gNB replies with a new PDU Set handling support Indication IE to indicate its supported capability..

- Option 3: follow an equivalent approach as was agreed for RACS capability exchange in Rel-17, where the SMF can request support information from the NG-RAN node, by exchanging information of the supported (NGAP) IE-Id during a given PDU Session Management procedure, or during the establishment of PDU Session resources in the course of a CN based HO, or by relaying the SMF’s collected request to the target NG-RAN node during an Xn based HO.

RAN3 agreed to adopt the solution described in option 3 (TBD during the discussion)

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the RAN3 solution into account and to update their specification as needed.

3. Date of Next RAN3 Meetings:

3GPP RAN3#121bis                               09 – 13 October 2023			Xiamen, China

3GPP RAN3#122	                       13 – 17 November 2023	                          Chicago, US

## Annex 3 - Draft LS to CT4

3GPP TSG-RAN WG3 Meeting #121	R3-23xxx1

21-25 August 2023, Toulouse, France

Title:	LS on XRM UP design

Response to:

Release:	Release 18

Work Item:	XRM

Source:	RAN3

To:	CT4, SA2

Cc:	RAN2

Contact Person:

- Name:	Yazid Lyazidi

- E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

During Rel-18 XRM discussion, RAN3 has discussed that the XR User Plane signalling must be designed in a way to consider to Non-homogenous support of PDU set based handling in NG-RAN. However, neither the PDU Session User Plane protocol (TS 38.415) nor the NR User Plane protocol (TS 38.425) support Non-homogenous deployments of (user plane) protocol additions, as requested in XR case and supported by SA2 in TS 23.501.

Instead of reusing the existing PDU session container, RAN3 considers the introduction of a new “PDU Set Container” that can be specified in TS 38.415 or TS 38.425, or in a new TS, in a backward compatible way to support the case of Non-homogenous deployment. This allows a non-supporting target to ignore the XR PDU Set Information that belong to PDU Sets and fallback to the PDUs container in the legacy PDU Session container in case of TS 38.415.

(TBD during the discussion)

2. Actions:

To CT4, SA2:

ACTION: 	RAN3 kindly asks CT4 to take the RAN3 solution into account and to update their specification as needed.

3. Date of Next RAN3 Meetings:

3GPP RAN3#121bis                               09 – 13 October 2023			Xiamen, China

3GPP RAN3#122	                       13 – 17 November 2023	                          Chicago, US

## Annex 4 – extracts from TS 23.501 V18.2.2

### PDU Set QoS Parameters:

Below we find the related description of the PDU Set QoS parameters from TS 23.501 V18.2.2:

#### 5.7.1.1 QoS Flow […] A QoS Flow may be enabled with PDU Set based QoS handling as described in clause 5.37.5. For such QoS Flows, PDU Set QoS Parameters (see clause 5.7.7) are determined by the PCF and provided by SMF to the NG-RAN as part of the QoS profile. #### 5.7.1.2 QoS Profile A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the (R)AN and it contains QoS parameters as described below (details of QoS parameters are described in clause 5.7.2): - For each QoS Flow, the QoS profile shall include the QoS parameters: - 5G QoS Identifier (5QI); and - Allocation and Retention Priority (ARP). - For each QoS Flow, the QoS profile may also include the QoS parameters: - PDU Set QoS Parameters (described in clause 5.7.7). […] ### 5.7.7 PDU Set QoS Parameters #### 5.7.7.1 General PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling. The following PDU Set QoS Parameters are specified: 1. PDU Set Delay Budget (PSDB). 2. PDU Set Error Rate (PSER). 3. PDU Set Integrated Handling Information (PSIHI). The QoS Profile may include the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile. If the NG-RAN receives PDU Set QoS Parameters and supports them, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in this clause. Editor's note: [XRM] The applicability and details of supporting PDU Set handling in uplink direction may be updated based on RAN WG's progress. #### 5.7.7.2 PDU Set Delay Budget The PDU Set Delay Budget (PSDB) defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. the duration between the reception time of the first PDU (at the N6 termination point for DL or the UE for UL) and the time when all PDUs of a PDU Set have been successfully received (at the UE for DL or N6 termination point for UL). PSDB applies to the DL PDU Set received by the PSA UPF over the N6 interface, and to the UL PDU Set sent by the UE. NOTE: To enable support for PSDB, it is required that a maximum inter arrival time between the first received PDU and the last received PDU of a PDU Set complies with SLA. This maximum inter arrival time does not exceed PSDB. NG-RAN behaviour when the SLA is not fulfilled is out of scope of this specification. A QoS Flow is associated with only one PDU Set Delay Budget. The value of the PDU Set Delay Budget is the same in UL and DL. PSDB is an optional parameter that may be provided by the PCF. The provided PSDB can be used by the NG-RAN to support the configuration of scheduling and link layer functions. When the PSDB is available, the PSDB supersedes the PDB for the given QoS Flow. The AN PSDB is derived at NG-RAN by subtracting CN PDB (as described in clause 5.7.3.4) from the PSDB. #### 5.7.7.3 PDU Set Error Rate The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related PDU Set losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). NOTE 1: In this Release, a PDU Set is considered as successfully delivered only when all PDUs of a PDU Set are delivered successfully. NOTE 2: How RAN enforces PSER is up to RAN implementation. A QoS Flow is associated with only one PDU Set Error Rate. PSER is an optional parameter. If the PSER is available, the PSER supersedes the PER. The value of the PDU Set Error Rate is the same in UL and DL. #### 5.7.7.4 PDU Set Integrated Handling Information The PDU Set Integrated Handling Information (PSIHI) indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. PSIHI is an optional parameter.

### PDU Set Information:

Below we find the related description of the PDU Set Information from TS 23.501 V18.2.2:

### 5.37.5 PDU Set based Handling #### 5.37.5.1 General A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters specified in clause 5.7.7 and PDU Set information provided by the PSA UPF as described in clause 5.37.5.2. In addition to the PDU related service information, the AF may provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3]. - PDU Set QoS parameters as described in clause 5.7.7 - Protocol Description: Indicates protocol and payload type used by the service data flow. AF provided PDU Set QoS Parameters and Protocol Description may be used in determining PCC Rules by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the Protocol Description may be used for identifying the PDU Set information by the PSA UPF. When the PCF determines that PDU Set based QoS Handling is to be performed for an application service flow, the PCF generates a PCC rule containing the PDU Set QoS parameters (PSER, PSDB and PSIHI) and the SMF determines a QoS Profile for the QoS Flow. Alternatively, the SMF may be configured to support PDU Set QoS without receiving PCC rules from a PCF. PSA UPF identifies PDUs that belong to PDU Sets. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2. NOTE: If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration. #### 5.37.5.2 PDU Set Information and Identification To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above. The PDU Set Information comprises: - PDU Set Sequence Number. - Indication of End PDU of the PDU Set. - PDU Sequence Number within a PDU Set. - PDU Set Size in bytes. - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion. NOTE 1: In addition to considering the PDU Set Importance within a QoS Flow, NG-RAN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator. NOTE 2: The PDU Set Information can be different for different PDU Sets within a QoS Flow. The SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the Protocol Description indicating the header, extension header (e.g. RTP/SRTP) and payload type (e.g. H.264) used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1. PSA UPF can identify the PDU Set Information using the Protocol Description and the received RTP/SRTP headers or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179]. For each DL PDU received on N6 for which PDU Set based QoS handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides PDU Set Information which is available to the RAN in the GTP-U header.

### TSCAI

### 5.37.8 UE power saving management #### 5.37.8.1 General The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX: - UL and/or DL Periodicity; - N6 Jitter Information associated with the DL Periodicity; - Indication of End of Data Burst. The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI. #### 5.37.8.2 Periodicity and N6 Jitter Information associated with Periodicity In the procedures for setting up or updating an AF session with QoS, the 5G System may be provided with Periodicity information for NG-RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted. If Periodicity information is available at the PCF, the PCF sends the Periodicity information received from the AF/NEF to the SMF and in accordance with the operator's local policies, includes an indication for SMF to request the UPF to perform N6 Traffic Parameter(s) measurement (i.e. DL periodicity associated N6 jitter range and if not provided by the AF, UL/DL periodicity) within the PCC Rules. Upon reception of a PCC rule with Periodicity information, the SMF determines the TSCAI and forwards it to the NG-RAN. If the PCC rule indicates to perform N6 Traffic Parameter measurements, the SMF shall request the UPF to monitor and periodically report the N6 Traffic Parameters (i.e. the N6 jitter range associated with the DL Periodicity and, if not provided by the AF, UL/DL periodicity) using the N4 Session Modification procedure, see clause 5.8.5.11. If the measurement of N6 jitter range associated with the DL Periodicity is required and the DL Periodicity is available at the SMF, the SMF also sends the DL Periodicity to the UPF. The UPF reports the measured N6 Traffic Parameters to SMF via N4 interface. NOTE 1: How the UPF derives the N6 jitter and periodicity (i.e. when periodicity is not provided by the AF) is implementation dependent. At reception of measured N6 Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range together with the associated DL periodicity and the UL periodicity if not provided by the AF in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2. NOTE 2: In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the N6 jitter is larger than a threshold. The DL periodicity associated N6 jitter indicates the positive or negative deviation of the arrival time of first packet of a Data Burst compared to the ideal Data Burst start time which is be determined based on the DL periodicity. #### 5.37.8.3 End of Data Burst Indication An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX. The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies. SMF should request the UPF to detect the last PDU of the data burst and mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies. According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the protocol header as defined in Protocol Description (see TS 26.522 [179]) or UPF implementation and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst. NOTE: There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.