3GPP TSG-RAN WG3 Meeting #121	R3-234331

Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0710 | rev | 2 | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction of NIA0 algorithm in gNB-CU-UP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NIA0 algorithm for integrity protection shall not be used, as per TS 33.501 specifications: - In section 5.3.3: “Integrity protection of the user data between the UE and the gNB is optional to use, and shall not use NIA0.” - In Annex D.1: “The NIA0 shall not be used for data radio bearers (DRBs).” However, it can be signalled as the selected integrity protection algorithm to the gNB-CP-UP, via the Integrity Protection Algorithm IE. Therefore, if received, the gNB-CU-UP shall reject the Bearer Context Setup or the Bearer Setup Modification with an appropriate cause value (e.g. Integrity protection algorithms not supported). It needs to be noted that this behavior was introduced to be backwards compatible with UEs not supporting UP integrity protection. Otherwise, the network would have to signal use of NIA0 to UEs that do not know about that it can be signaled. Those UEs expect UP to never have integrity protection in any form. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the restriction of NIA0 in Abnormal Conditions of Bearer Context Setup and Bearer Context Modification procedures. Add in semantics description of the Integrity Protection Algorithm IE that NIA0 codepoint is not used in the specification. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release), limited to the integrity protection function. This CR has a functionality impact, for implementations that were using NIA0 algorithm for DRBs. This CR has no impact on the protocol. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong usage of security algorithms for integrity protection may occur |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.4, 8.3.2.4, 9.3.1.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 37.483 CR0053 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.2: Resubmission to RAN3#121 Rev.1: Resubmission to RAN3#120 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

#### 8.3.1.4	Abnormal Conditions

If the gNB-CU-UP receives a BEARER CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE in the DRB To Setup List IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-CU-UP shall report the establishment of the corresponding DRB as failed in the DRB Failed List IE of the BEARER CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT SETUP REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource To Setup List IE for a GBR QoS Flow but where the GBR QoS Flow Information IE is not present, the gNB-CU-UP shall report the establishment of the corresponding QoS Flow as failed in the corresponding  Flow Failed List IE of the BEARER CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT SETUP REQUEST message containing the Integrity Protection Algorithm IE in the Security Information IE set to “NIA0”, the gNB-CU-UP shall reject the procedure using the BEARER CONTEXT SETUP FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 8.3.2.4	Abnormal Conditions

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE in the DRB To Setup List or the DRB To Modify List IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding DRB as failed in the DRB Failed List IE or the DRB Failed To Modify List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE for a GBR QoS Flow but where the GBR QoS Flow Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding QoS Flow as failed in the corresponding  Flow Failed List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing the Integrity Protection Algorithm IE in the Security Information IE set to “NIA0”, the gNB-CU-UP shall reject the procedure using the BEARER CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< End of 2nd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED --

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.31	Security Algorithm

This IE defines the type of ciphering algorithm and/or integrity protection used for the DRBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ciphering Algorithm | M |  | ENUMERATED (NEA0, 128-NEA1, 128-NEA2, 128-NEA3) | As defined in TS 33.501 [13]. |
| Integrity Protection Algorithm | O |  | ENUMERATED (NIA0, 128-NIA1, 128-NIA2, 128-NIA3) | As defined in TS 33.501 [13] for NG-RAN. The value NIA0 is not used in the specification. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>