3GPP TSG-RAN WG3 Meeting #121	R3-234347

Toulouse, France, 21 – 25 August, 2023

Agenda item:	12.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Continuous MDT Evaluation

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #119bis-e, we made some agreements and further captured some open aspects with respect to MDT:

The existing MDT framework is used as baseline for data collection from the UE.

FFS on using s-based MDT, m-based MDT or both of them for data collection from the UE.

In RAN3 #120 we continued the discussion on continuous MDT.  We captured the following points as common understanding in RAN3:

For signaling based MDT, TCE could identify the UE via UE identifier

For management based MDT, it is not possible for TCE to identify the UE with UE identifier that can lead to the unique UE identity since m-based MDT is intentionally made anonymous.

In addition, we captured the following open points for further evaluation:

Further evaluate the gap of continuous MDT during idle to active transition for both management based MDT and signaling based MDT, e.g. how many milliseconds? What is the maximum acceptable gap for continuous MDT? e.g. consider the report periodic of UE measurement report.

Further check whether the management based MDT identifier e.g. IMEI-TAC, NR- CGI, TR/TRSR allows the TCE to correlate logged MDT and immediate MDT collected during continuous MDT.

If correlation of log MDT and immediate MDT is possible for management based MDT, FFS whether the current mechanism already support configuration of continuous MDT i.e. when UE connected to a new cell and report the availability of log MDT, the connected cell could configure the UE with immediate MDT immediately.

If correlation of log MDT and immediate MDT is possible for management based MDT, FFS on selection of group of UE when deciding to implement continuous MDT.

In this contribution, we provide our views with respect to continuity of MDT data collection.

# 2	Continuity with respect to measurement gap at transition from RRC_Idle/RRC_Inactive to RRC_Connected

Continuous MDT collection may be defined as MDT data collection that enables to collect data from the same UE across RRC states (RRC_Connected, RRC_Idle, RRC_Inactive). MDT Continuity has been discussed in RAN3 during several meetings with respect to enabling a NG-RAN node to trigger Immediate MDT when UE reconnects from RRC Idle state. The legacy MDT framework is based on configuration in the UE that is separate for idle and inactive modes (Logged MDT) and connected mode (Immediate MDT). While activation of a Logged MDT configuration configured in the UE takes place automatically when the UE transits to idle or inactive modes, the activation of Immediate MDT measurements in the UE is based on RRC measurement configuration sent to the UE by the network at each transition to connected mode. The obtention of a continuous measurement series therefore depends on the delay required for activation of the Immediate MDT measurement in the UE when the UE connects to the network in comparison with the standardized values for logging interval, which are, in milliseconds (TS 38.331):

LoggingInterval-r16 ::= ENUMERATED {

ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440, infinity }

Thus it can be seen that the defined logging intervals span a range from 320 ms to around 1 minute.

When connecting to the network, a UE configured with Logged MDT will inform the network about the existence of such configuration by including the logMeasAvailable-r16 IE in the RRCReconfigurationComplete message. A Rel-17 UE will additionally inform the network whether the logged measurement configuration was configured via management-based activation by including the sigLogMeasConfigAvailable IE set to false (value true is used for signalling-based activation).

The current MDT framework therefore offers the possibility for a NG-RAN node implementation, combined with appropriate configuration, to identify UEs needing to be configured with management-based Immediate MDT in some scenarios during the UE initial access scenario. A possible achievable value for the duration of UE initial access can be estimated to be 50 ms, which would then also correspond to the measurement gap in transition from RRC_Idle to RRC_Connected in this scenario, with a maximum reasonable value reaching at most 100ms depending on deployment and equipment aspects.

In some scenarios, e.g. with a higher number of ongoing management-based Logged MDT sessions configured in the network, the NG-RAN node implementation may retrieve TR/TRSR (Trace Reference / Trace Recording Session Reference) of the Logged MDT configuration from the UE using the RRC UEInformationRequest / UEInformationResponse message exchange towards the UE. The NG-RAN node implementation may then map the TR/TRSR of the Logged MDT session with an associated immediate MDT session. A possible value for the delay related to the UEInformationRequest / UEInformationResponse message exchange can be estimated to be around 20 ms.

The measurement gap induced by transition from RRC_Idle to RRC_Connected would in a reasonable scenario not exceed the order of 120 ms, which would also include some internal delay (e.g. 5 ms) in the UE linked to processing and initiation of the immediate MDT measurement. This means that the gap is significantly lower than the shortest Logged MDT measurement interval.

Observation 1: The measurement gap induced by transition from RRC_Idle to RRC_Connected would, in a reasonable scenario, not exceed the order of 120 ms, which is significantly lower than the shortest Logged MDT measurement interval.

This observation is valid independently of whether the UE reconnects to the same or to a different NG-RAN node, because no UE context is anyway kept in the RAN when the UE is in idle mode.

Observation 2: There is no risk of loss of logged information when the UE moves to Idle and then reconnects to the network (same or different NG-RAN node).

If the UE is moved to RRC_Inactive instead of RRC_Idle, then the RRC Resume procedure to bring the UE back to RRC connected states is much shorter than the RRC Setup procedure and hence, the total delay in re-configuring the log in the UE will be smaller than 120 ms.

Observation 3: There is no risk of loss of log information when the UE moves to RRC_Inactive and then connects to the same or a different NG-RAN node.

Based on the above observations, it is concluded that with the current MDT mechanism, all the possible continuous MDT scenarios shall work without any potential loss of logs and hence, does not require any enhancement in Rel-18.

# 3	Continuity with respect to a UE upon RRC transition

In signalling-based MDT, continuous MDT measurements with respect to a specific UE are supported by default since trace activation is activated per UE and the CN may store the signalling-based MDT configuration and apply it when a UE moves from RRC Idle to RRC Connected state. Therefore, when a UE transits to RRC Idle or Inactive state it activates its configured Logged MDT configuration and logs the corresponding measurements while when it transits to RRC_Connected state the CN can immediately configure the UE with an Immediate MDT Configuration. This is because in Signalling-based MDT a UE can be identified through one of its permanent identifiers, e.g., IMSI or IMEI by the CN.

However, for AI/ML Training, data may need to be collected from specific cells. Cell-level information is not available at the OAM system and hence signalling-based activation is not suitable for scenarios when one or more UEs in a certain area (e.g., in a cell or set of cells) need to be selected. In those scenarios, data collection using management-based methods would be needed. In management-based MDT, a UE is not uniquely identified in the MDT activation. Hence when a UE transits to RRC connected state the network does not have standardized means to select a “specific” UE for continuous MDT.

Thus, a further issue with respect to continuity relates to the fact that for management-based MDT a UE is not uniquely identified due to intentionally introduced anonymization. The anonymization of MDT data depends on the configuration parameter received at the MDT configuration. If UE identity is sent, the IMEI-TAC of the UE is used instead of a permanent identifier. TCE needs to use the IMEI-TAC of the UE to create an association between Logged MDT measurements (corresponding to a Trace Reference) and Immediate MDT measurements (corresponding to a different Trace Reference). IMEI-TAC is the first 8-digits of the IMEI. IMEI-TAC can characterize a device make and model but doesn’t uniquely identify the UE, meaning that multiple UEs of same or similar types may have the same IMEI-TAC. In a general situation, these UEs will be located in different areas of the cell, and the logs of different trace references from different UEs should therefore not be combined.

Observation 4: IMEI-TAC characterizes a device make and model, but doesn’t intentionally uniquely identify a specific UE.

Similarly, the MDT session of a UE cannot be identified through the TR and TRSR since TRSR is allocated by a NG-RAN node to identify a session but if a UE reconnects to a different NG-RAN node it will receive a different TRSR which is uncorrelated from the first one.

Observation 5: A continuous MDT session (comprising Logged and Immediate MDT) is not identified uniquely by the pair TR and TRSR.

One proposal to enable continuity for management-based MDT is to provide in the trace activation from OAM towards a NG-RAN node a common configuration for Immediate and Logged MDT, using a single Trace Reference for both types of logs to indicate that it corresponds to a continuous MDT configuration. Subsequently a NG-RAN node receiving such continuous MDT configuration from OAM can configure a UE in RRC_Connected state both with an Immediate MDT and Logged MDT configuration with a flag indicating that this is continuous MDT. However, this solution is inflexible since it does not allow adapting Immediate MDT configurations to specific NG-RAN nodes as the Trace Reference should uniquely identify the MDT configuration.

Observation 6: Using a single Trace Reference for both Logged and Immediate MDT provides a limited and inflexible solution as it does not allow adapting Immediate MDT configurations to specific NG-RAN nodes.

In this case, UE may report the Immediate MDT measurements during RRC_Connected state and may log measurements according to the received Logged MDT configuration. When UE moves back to RRC_Connected state it indicates to the NG-RAN node to which it reconnects that it is configured with continuous MDT so that the NG-RAN node can configure it immediately with the next Immediate and Logged MDT configuration (to maintain continuity). Such solution further has other drawbacks. It has UE impacts since a UE needs to support such “continuous MDT flag” and may only be supported by a limited number of Rel.18 UEs that implement such flag.

Observation 7: Continuous MDT flag has a significant drawback of impacting the UE and therefore of being supported only by a limited number of Rel.18 UEs.

In addition, such solution is limited in that the UEs to perform continuous MDT cannot be selected by OAM but it is left to NG-RAN node’s implementation in case of management-based activation. Furthermore, such solution is not flexible enough in that it does not enable the possibility to train different ML Models by collecting data from different management-based MDT sessions. This is because the binary “continuous MDT flag” does not allow a NG-RAN node to select among different Immediate MDT configurations when a UE reconnects to RRC_Connected state.

Observation 8: Continuous MDT flag does not allow a NG-RAN node to select among different Immediate MDT configurations when a UE reconnects to RRC_Connected state.

One possible way to enable MDT continuity when a UE reconnects from RRC_Idle to RRC_Connected state is to configure with Immediate MDT all the UEs that indicate data availability of Logged MDT. However, similarly to the “Continuous MDT flag” this solution cannot distinguish among different management-based MDT configurations in case multiple immediate MDT Configurations are available.

One option to maintain continuity when a UE is chosen for management-based MDT is to let a NG-RAN node store the immediate MDT Configuration in the CN (after checking that the MDT Configuration is still valid), including the full reference of the Immediate MDT session (namely the Trace Reference (TR) and the Trace Recording Session Reference (TRSR)). When a UE reconnects back from RRC Idle or RRC Inactive states, the CN may then send back the information to the NG-RAN node and the latter may use this information to configure Immediate MDT measurements. Similarly, the NG-RAN node may store the full reference of the Logged MDT session in the CN.

Proposal 1: In order to maintain management-based MDT continuity a NG-RAN node stores the Immediate MDT configuration for which continuity is required to the CN. When UE transits from RRC_Idle or RRC_Inactive to RRC_Connected states, the CN sends this information back to the NG-RAN node to configure the UE with a management-based Immediate MDT configuration.

To enable TCE correlate different management-based MDT sessions corresponding to different TR and TRSR some additional information is needed at the TCE.

One solution would be to send the TR and TRSR of the Logged MDT configuration to the TCE together with the Immediate MDT data record. The NG-RAN node would need to store the TR and TRSR of the Logged MDT session in the UE context for later use and send it to the TCE together with the stored Immediate MDT measurements (and the TR and TRSR of the Immediate MDT session). This alternative would be a simple alternative but would require that the network, in order to retrieve TR/TRSR of the Logged MDT session, uploads the logged measurement reports stored in the UE always before Immediate MDT. This may have a limitation in case an urgent handover has to be triggered or if there is an RLF.

One simple alternative to enable correlation of Logged and Immediate MDT logs is to create a random number at the AMF generated when the UE registers to the network and UE context is created in the CN and maintained in the CN for the duration of the UE context. In this way, UE identity is not revealed as the number is random but it enables TCE to correlate MDT logs with different TRs and TRSRs pertaining to the same UE. This random number can be sent to the TCE together with the IMEI-TAC, TR and TRSR to enable TCE correlate different MDT logs corresponding to different TR and TRSR without revealing UE identity.

Proposal 2: RAN3 to discuss the two options to enable TCE correlate different logs pertaining to the same UE, namely:

- Option 1: Use a random number allocated at the CN per UE upon registration.

- Option 2: Include the TR and TRSR of the Logged MDT session in MDT records for Immediate MDT data sent to the TCE.

# 4	Conclusion

In this paper we make the following observations and proposals:

Observation 1: The measurement gap induced by transition from RRC_Idle to RRC_Connected would, in a reasonable scenario, not exceed the order of 120 ms, which is significantly lower than the shortest Logged MDT measurement interval.

Observation 2: There is no risk of loss of logged information when the UE moves to Idle and then reconnects to the network (same or different NG-RAN node).

Observation 3: There is no risk of loss of log information when the UE moves to RRC_Inactive and then connects to the same or a different NG-RAN node.

Observation 4: IMEI-TAC characterizes a device make and model, but doesn’t intentionally uniquely identify a specific UE.

Observation 5: A continuous MDT session (comprising Logged and Immediate MDT) is not identified uniquely by the pair TR and TRSR.

Observation 6: Using a single Trace Reference for both Logged and Immediate MDT provides a limited and inflexible solution as it does not allow adapting Immediate MDT configurations to specific NG-RAN nodes.

Observation 7: Continuous MDT flag has a significant drawback of impacting the UE and therefore of being supported only by a limited number of Rel.18 UEs.

Observation 8: Continuous MDT flag does not allow a NG-RAN node to select among different Immediate MDT configurations when a UE reconnects to RRC_Connected state.

Proposal 1: In order to maintain management-based MDT continuity a NG-RAN node stores the Immediate MDT configuration for which continuity is required to the CN. When UE transits from RRC_Idle or RRC_Inactive to RRC_Connected states, the CN sends this information back to the NG-RAN node to configure the UE with a management-based Immediate MDT configuration.

Proposal 2: RAN3 to discuss the two options to enable TCE correlate different logs pertaining to the same UE, namely:

- Option 1: Use a random number allocated at the CN per UE upon registration.

- Option 2: Include the TR and TRSR of the Logged MDT session in MDT records for Immediate MDT data sent to the TCE.