3GPP TSG-RAN WG3 #121                                                                                 R3-234365

Toulouse, France, 21 – 25 August, 2023

Source:	CATT

Title:	(TP to BL CR for TS 38.305) Support of cross-cell SRS configuration

Agenda Item:	23.2

Document for:	Discussion and Decision

- Introduction

In the previous RAN3 meetings, we discussed the RAN3 impact on LPHAP, for the SRS configuration enhancement, it’s not decided yet which node to decide to validity area, how to configure the SRS configuration for the validity area. In the last RAN3 meeting, we expected to further discuss the call flow with details on how to determine SRS validating area and which node allocates SRS resources in validating area .

LPHAP:

Call flow with details on how to determine SRS validating area and which node allocates SRS resources in validating area.

In this contribution, we will further discuss how to support cross-cell SRS configuration. Base on the discussion, we provide the observations and proposals accordingly.

2. Discussion

2.1 Overall procedures for area specific SRS allocation

To support area specific SRS configuration for a UE, how to allocate the specific SRS configuration and how to decide the validity area are the key issues to be addressed.

On how the SRS resources are (pre)configured for a validity area, RAN3 discussed the issue the last meeting. Majorities of companies believed that it should be the last serving gNB to allocate the SRS resources, and some companies believed the involved gNBs should be able to allocate the SRS resources themselves.

We understand that, to support area specific SRS configuration, corresponding SRS resources are reserved in the gNBs and cells within the validity area, and it seems needed to coordinate between the gNBs to select proper SRS resource for the validity area. Xn based coordination or NRPPa based coordination are both feasible. Some potential solutions are provided below:

- Option 1: gNB check the available (pre-configured) SRS resources with its neighbour gNBs over Xn, then, it selects proper SRS configuration and decides the validity area for the UE. The gNB should also notify the corresponding gNBs to reserve the SRS resource for cross-cell SRS configuration.

- Option 2: LMF check the available (pre-configured) SRS resources with gNBs, it selects the proper SRS configuration and corresponding validity area for the UE, and provide these information towards the serving gNB and corresponding gNBs involved in the validity area to reserve the SRS configuration.

- Option 2bis: the difference with the option 2 is LMF provide the selected SRS configuration and corresponding recommended validity area towards the serving gNB, and the serving gNB final decides the validity area and response to the LMF, LMF reserves the SRS configuration in the validity area.

The Option 1 is illustrated with the figure below:

The serving gNB may receive the NRPPa Positioning Information Request in RRC connected or RRC_INACTIVE. If needed, the serving gNB checks with the neighbour gNBs to which Xn connectivity is available, and decides the SRS configuration and corresponding validity area. Then it could provide area specific SRS configuration towards the UE in RRCRelease message. This option requires to introduce new XnAP procedures to check and response the available SRS configuration, to reserve the area specific SRS configuration.

Observation 1: Xn based solution (the serving gNB decides the area specific SRS configuration and validity area) requires some new XnAP procedures to check the available SRS configuration with the neighbour gNBs, and to reserve the SRS configuration in the validity area.

Option 2 is illustrated with the figure below:

Option 2 is totally different with the option 1, the determination of area specific SRS configuration and the validity area is done in LMF. LMF may check with the gNBs to get available SRS configurations before determination of the area specific SRS configuration.

However, maybe there’s some issue on determination of the validity area, as the LMF does not know whether the UE is in RRC Connected, or in RRC Inactive state, it does not know the RNA configuration of the UE, it does not know the Xn reachability between the gNBs. For example, if the validity area decided by the LMF involves two gNBs without available Xn interface, the gNB may configure a smaller RNA (compared to the validity area) for the UE to guarantee the Xn reachability within the RNA. In this case, RNAU procedure maybe triggered before UE moves out of the SRS validity area. This is feasible, but not a good implementation.

Observation 2: NRPPa based solution (LMF decides the area specific SRS configuration and validity area) requires LMF to collect the available SRS configurations from gNBs, and to reserve the SRS configuration in the validity area.

Option 2bis is illustrated with the figure below:

To address the potential issue mentioned in the Option 2, Option 2bis allows the serving gNB to make final decision on the validity area, base on to the recommended area provided by the LMF. The validity area should also be notified to LMF in the NRPPa Positioning Information Response, the LMF use it to reserve the SRS configuration in the validity area.

Observation 3: For the option 3, the validity area decided by the gNB, should be notified to LMF for the purpose of SRS resources reservation in the validity area.

The overall procedures as discussed above are provided in the Annex, the option, if agreeable, should be captured in the stage 2. We further consider the Xn or NRPPa signalling details, e.g. introduction of new signalling(s) or reuse the existing signalling(s) the next meeting.

Proposal 1: RAN3 should first discuss and decide the overall procedures for area specific SRS configuration, the stage 2 TP is provided in the Annex.

2.2 Overall procedures when UE moves out of validity area

In the last RAN3 meeting, an LS from RAN2 is received [3], as below:

During the discussion for LPHAP, RAN2 has reached the following agreement for SRS transmission in RRC_INACTIVE:

Agreements:

RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

RAN2 assumes when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. As there’s no progress in RAN3, we postponed the potential LS response.

Some further agreements have been reached in RAN2#121bis-e meeting, as below:

Agreements:

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Observation 4: RAN2 assumes when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Following the RAN2 agreements, UE will request for the SRS resources from the new gNB (out of validity area) directly. The most straightforward way is to retrieve the full UE context from the last serving gNB, then the new gNB performs SRS resource allocation according to the SRS characteristics, the new allocated SRS configuration is provided to the UE via RRCRelease message.

To support that, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, with that, the last serving gNB could decide whether to relocate the UE context, and provide the Positioning Information (i.e. SRS characteristics, Routing ID, etc.).

Proposal 2: To support UE moves out of the validity area, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, to indicate the last serving gNB to perform the UE context relocation, including the Positioning Information.

The overall procedures when UE moves out of SRS validity area could be illustrate with the figure below:

Figure 1: Overall procedure UE moves out of SRS validity area

From the figure above, we could see there’s clear SRS request from UE to the receiving gNB, when UE moves out of the validity area, and the receiving gNB should retrieves the full UE context including Positioning Information with specific indicator or cause value, then the receiving gNB becomes the new serving gNB and do corresponding SRS allocation.

During this procedure, whether to configure a new area specific SRS configuration for the UE is pending to the network, details on area specific SRS resource allocation is pending to the discussion in section 2.1.

Observation 5: When UE moves out of the validity area, the network may or may not configure UE with a new area specific SRS configuration.

Above all, we should discuss the overall procedures on handling of UE moves out of validity area, as shown in the Annex.

Proposal 3: RAN3 should discuss the overall procedure on handling of UE moves out of validity area.

- Conclusion

According to the discussion on support of area specific SRS in section 2, the following observations and proposals are provided:

Observation 1: Xn based solution (the serving gNB decides the area specific SRS configuration and validity area) requires some new XnAP procedures to check the available SRS configuration with the neighbour gNBs, and to reserve the SRS configuration in the validity area.

Observation 2: NRPPa based solution (LMF decides the area specific SRS configuration and validity area) requires LMF to collect the available SRS configurations from gNBs, and to reserve the SRS configuration in the validity area.

Observation 3: For the option 3, the validity area decided by the gNB, should be notified to LMF for the purpose of SRS resources reservation in the validity area.

Proposal 1: RAN3 should first discuss and decide the overall procedures for area specific SRS configuration, the stage 2 TP is provided in the Annex.

Observation 4: RAN2 assumes when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Proposal 2: To support UE moves out of the validity area, a specific cause value or indicator in Xn Retrieve UE Context Request is needed, to indicate the last serving gNB to perform the UE context relocation, including the Positioning Information.

Observation 5: When UE moves out of the validity area, the network may or may not configure UE with a new area specific SRS configuration.

Proposal 3: RAN3 should discuss the overall procedure on handling of UE moves out of validity area.

- Reference

- RP-223549	  New WID on Expanded and Improved NR Positioning,  Intel Corporation, CATT, Ericsson

- RAN3#120 Chair Notes

- R3-231116 LS on SRS configuration request, source: RAN2, To: RAN3

- Annex

5.1 TP for BL CR for TS 38.305

x.y Procedures for multi-cell SRS configuration

x.y.1 General

Editor’s Note: To be updated by RAN2.

x.y.2 Area specific SRS resource allocation procedure (option 1)

Figure x.y.2-1 shows the area specific SRS resource allocation procedure.

Figure x.y.2-1: Overall procedure for area specific SRS resource allocation

- LMF initiates NRPPa Positioning Information Request towards the serving gNB.

- The serving gNB check with its neighbour gNBs on the available SRS configuration.

- The serving gNB decides the area specific SRS configuration and corresponding validity area.

- The serving gNB informs the corresponding gNBs within validity area to reserve the SRS resources for the UE.

- The serving gNB moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the area specific SRS configuration, and corresponding validity area.

- The serving gNB responses the NRPPa Positioning Information Response to the LMF.

x.y.2 Area specific SRS resource allocation procedure (option 2)

Figure x.y.2-1 shows the area specific SRS resource allocation procedure.

Figure x.y.2-1: Overall procedure for area specific SRS resource allocation

- LMF check with gNBs the available SRS configuration.

- LMF initiates NRPPa Positioning Information Request towards the serving gNB, including the selected area specific SRS resource and corresponding validity area.

- The serving gNB moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the area specific SRS configuration, and corresponding validity area.

- The serving gNB responses the NRPPa Positioning Information Response to the LMF.

- LMF reserves the configured SRS configuration in the validity area.

x.y.2 Area specific SRS resource allocation procedure (option 2bis)

Figure x.y.2-1 shows the area specific SRS resource allocation procedure.

Figure x.y.2-1: Overall procedure for area specific SRS resource allocation

- LMF check with gNBs the available SRS configuration.

- LMF initiates NRPPa Positioning Information Request towards the serving gNB, including the selected area specific SRS resource and recommended validity area.

- The serving gNB decides the validity area, and moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the area specific SRS configuration, and corresponding validity area.

- The serving gNB responses the NRPPa Positioning Information Response to the LMF, which includes the validity area.

- LMF reserves the configured SRS configuration in the validity area.

x.y.3 Area specific SRS resource update procedure

Figure x.y.2-1 shows the SRS validity area update procedure.

Figure x.y.2-1: Overall procedure area specific SRS resource update

- The UE in RRC_INACTIVE is configured an area specific SRS configuration.

- When UE moves out of the SRS validity area, it request for SRS.

Editor’s note: The message to be used to request for SRS could be revisited by RAN2.

- The gNB which receives the request from UE triggers the Retrieve UE Context procedure, and indicate new SRS is request.

- The last serving gNB relocate the full UE context including the Positioning Information when receives the Retrieve UE Context Request with the SRS request.

- The receiving gNB allocate new SRS resource(s) for the UE, new validity area may also be decided.

Editor’s note: Details on determination of new SRS and new validity area are FFS.

- The receiving gNB triggers the Path Switch Request procedure towards the AMF.

- The receiving gNB moves the UE to RRC_INACTIVE by sending RRCRelease message, which includes the new SRS configuration, and probably the new validity area.

- The receiving gNB indicates the last serving gNB to delete the UE context.