# 3GPP TSG-WG3 Meeting #121    	                   R3-234438

# Toulouse, France, Aug, 2023

# Source: 			CMCC

# Title:  Discussion on downlink lossless delivery

# Agenda item:	16.3

# Document for: 	Discussion

Introduction

Service continuity had been discussed in last RAN3 meeting. For the handover procedure and lossless handover, it has reached the following agreements and left some open issues for further discussion. In this contribution, we will further discuss service continuity for SL relay.

RAN3 Agreements:

During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID.

RAN3 do not pursue the two optimizations in R18:

-The list of candidate Relay UEs is an ordered list, e.g. based on the Remote UE’s measurement report on candidate Relay UEs.

-Target gNB include the selected target Relay UE in the XnAP HANDOVER REQUEST ACKNOWLEDGE message.

Down-select between D4 and D5 in next meeting.

The criticality of the candidate relay UE info list IE should be “reject” in the XnAP HANDOVER REQUEST message and the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE over NGAP.

FFS whether move step 7 before step 5 in inter-gNB d2i/i2i path switch procedures.

Discussion

In last RAN2 and RAN3 meeting, the DL solutions for lossless handover have been discussed. RAN3 excluded solution D3 and decided to down select between D4 and D5 for downlink data delivery.

Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

Solution-D5: Proactive Data forwarding from source gNB to target gNB (i.e. allow the source gNB to forward all the buffered data to the target gNB).

For solution-D4, the target gNB requests the source gNB to additionally forward the missing DL packets that were not forwarded earlier after receiving the PDCP status report. An indicator from target gNB to source gNB is needed at Xn interface to indicate the missing DL packets. The source gNB should forward the missing data as indicated by target gNB. As in solution-D4, an indicator for missing data and data forwarding procedure are introduced, which may increase the complexity for network implementation.

Proposal 1: Indicator from target gNB to source gNB is needed to inform source gNB about the missing data as solution-D4.

Proposal 2: Data forwarding procedure after RAN HO Completion shall be introduced as solution-D4.

For solution-D5, source gNB may forwarding all the buffered data at PDCP transmitting entity to target gNB. Comparing with solution-D4, the indicator from target gNB to source gNB is not needed. However, as solution-D5, all the data in PDCP transmitting buffer should be forwarded to target gNB, which brings some overhead to Xn interface.

Proposal 3: Data forwarding procedure after RAN HO Completion shall be introduced as solution-D5.

Comparing solution-D4 and solution-D5, we have the following information:

|  | Solution-D4 | Solution-D5 |
| --- | --- | --- |
| Source gNB | - Storing all the data without acknowledge by lower layer in PDCP transmission buffer - Receiving the indicator - For warding the data as indicated | - Storing all the data without acknowledge by lower layer in PDCP transmission buffer - None - For warding all data in PDCP transmission buffer |
| Target gNB | - Indicate the missing data, as informed in PDCP status report - None | - None - Discard the duplicated data |
| Xn | - Indicator from target gNB to source gNB - Missing data | - None - All data |

Proposal 4: Supporting solution-D5 as downlink lossless delivery solution.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Indicator from target gNB to source gNB is needed to inform source gNB about the missing data as solution-D4.

Proposal 2: Data forwarding procedure after RAN HO Completion shall be introduced as solution-D4.

Proposal 3: Data forwarding procedure after RAN HO Completion shall be introduced as solution-D5.

Proposal 4: Supporting solution-D5 as downlink lossless delivery solution.

References

- [1] R2-2304305, Summary of candidate solutions for lossless delivery, NEC