3GPP TSG-RAN WG3 Meeting #121                 	                                             R3-234452

Toulouse, FR, Aug 21- Aug 25, 2023

Agenda Item:	25.2

Source:	CMCC

Title:	Discussion on non-homogeneous deployment of PDU Set handling

Document for:	Discussion

Introduction

From the new WID on XR enhancements for NR approved in RAN#99 meeting, the objective for XR enhancement with RAN3 impact are listed below:

Specify the enhancements related to capacity: - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Before this meeting, RAN3 received the LS[1] from SA2(R3-233735) on non-homogeneous deployment of PDU Set handling before the start of the meeting. In sum, SA2 has discussed the issue of non-homogeneous deployments of NG-RAN nodes supporting PDU Set based handling and provided the agreed CR(S2-2308246) to RAN3. SA2 kindly asks RAN3 to take the information provided by SA2 into account. In this contribution, we will analyse the impact of SA2 agreed conclusions on RAN3 specification work.

Discussion

From the CR(S2-2308246) in SA2 LS, SA2 describes non-homogenous support of PDU Set based handling in NG-RAN. The agreed CR is shown as follows:

#### 5.37.5.x Non-homogenous support of PDU set based handling in NG-RAN By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [xx]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set. Editor’s Note: In case of handover from a source NG-RAN node not supporting PDU Set handling to a target NG-RAN node supporting PDU Set handling, whether there is a reason for the target NG-RAN node to temporarily handle both unmarked PDUs and marked PDUs within the same QoS flow is FFS.

From the CR, it mentions PDU Set QoS Parameters are sent to the NG-RAN node. Also, from the definition of PDU Set in SA2, all the PDUs of a PDU set are transmitted within the same QoS flow. Hence, the PDU Set QoS Parameters is QoS flow level and should be included in QoS Flow Level QoS Parameters IE signalled by NGAP signalling. Additionally, QoS Flow Level QoS Parameters IE are transmitted in XnAP, F1AP and E1AP, so the PDU Set QoS Parameters are also transmitted in XnAP, F1AP and E1AP.

Observed from the definition of PDU Set QoS Parameters in TS 23.501 shown below, inside the PDU Set QoS Parameters, it contains the PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI). Obviously, PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) could be signalled in NGAP, XnAP, F1AP and E1AP.

Proposal 1: It is proposed that PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) could be signalled in NGAP, XnAP, F1AP and E1AP.

### 5.7.7 PDU Set QoS Parameters #### 5.7.7.1 General PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling. The following PDU Set QoS Parameters are specified: 1. PDU Set Delay Budget (PSDB). 2. PDU Set Error Rate (PSER). 3. PDU Set Integrated Handling Information (PSIHI). The QoS Profile may include the PDU Set QoS Parameters described in this clause (see clause 5.7.1.2). The PCF determines the PDU Set QoS Parameters based on information provided by AF and/or local configuration. The PDU Set QoS parameters are sent to the SMF as part of PCC rule. The SMF sends them to NG-RAN as part of the QoS Profile. If the NG-RAN receives PDU Set QoS Parameters and supports them, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters as described in this clause.

In addition, SMF can request the NG-RAN node to activate PDU Set QoS handling for a given QoS flow, NG-RAN node should provide the SMF with an indication of whether the PDU Set handling is accepted or not. If NG-RAN node supports the capability, SMF configures the PSA UPF to activate/deactivate the PDU. Considered that SA2 already agreed that NG-RAN node should provide the SMF with an indication, we want to follow the conclusion from SA2. From our view, it is acceptable to signal an explicit indicator indicating the capability of supporting PDU Set handling. The supporting indicator can be carried in PDU Session Resource Setup Response Transfer message, PDU Session Resource Modify Response Transfer message.

Mentioned by SA2 conclusion, at Xn handover and NG handover, the target NG-RAN needs to provide to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling. The supporting indicator can also be carried in Path Switch Request Transfer message and Handover Request Acknowledge Transfer message. Since this indication is related to whether the target NG-RAN node supports PDU Set handling, we suggest to name this new indicator as PDU Set Support Indicator IE.

Proposal 2: It is proposed to introduce PDU Set Support Indicator IE over NGAP, the new IE can be carried in PDU Session Resource Setup Response Transfer message, PDU Session Resource Modify Response Transfer message, Path Switch Request Transfer message and Handover Request Acknowledge Transfer message.

Conclusions

In this contribution, related to XR enhancement, we provide our understanding on the impact of SA2 agreed conclusions on RAN3 specification. The following observations or proposals are listed below:

Proposal 1: It is proposed that PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) could be signalled in NGAP, XnAP, F1AP and E1AP.

Proposal 2: It is proposed to introduce PDU Set Support Indicator IE over NGAP, the new IE can be carried in PDU Session Resource Setup Response Transfer message, PDU Session Resource Modify Response Transfer message, Path Switch Request Transfer message and Handover Request Acknowledge Transfer message.

References

- R3-233735, Non-homogeneous deployment of PDU Set based handling;

- S2-2308246, PDU Set QoS handling: non-homogenous support;

- RP-230786, Updated WID on XR Enhancements for NR