3GPP TSG-RAN WG3 Meeting #121	R3-234469

Toulouse, France, 21 – 25 Aug, 2023

Title: 	Discussion on DTLS for SCTP next steps - message size

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

The RAN3#121 meeting received a reply LS on the SCTP-AUTH and DTLS in [1]. This contribution discusses the questions wherein related to RAN3, and provides our views.

# 2. Background

It is beneficial to refresh the DTLS over SCTP first.

At previous RAN3-111-e meeting, a LS was agreed to IETF on the user message size limitation for DTLS over SCTP with the following contents in [2]. It can be observed that the RFC 6083 restricts its message size is 16K Bytes while the message size over the network interface could easily exceed it.

From the first version of the 5G specification, 3GPP has specified to use DTLS over SCTP. RAN3 has found an issue related to RFC 6083 DTLS user message size limitation over SCTP that impacts several of 3GPP RAN application protocols. The RFC specifies a user message limit of approximate16k Bytes. This should be compared to the unlimited user message size that exists when SCTP is used without DTLS. There are several RAN application messages that can exceed the limit of approximate 16k Bytes. The same issue may exist for the other 3GPP groups using the DTLS over SCTP. A general solution to this issue is desirable rather than changing multiple different protocols. We understand the limitation in RFC 6083 is due to a lack of a secure fragmentation mechanism of user messages into multiple DTLS records. The DTLS over SCTP specification appear to be the right layer to resolve this issue and achieve feature parity between DTLS over SCTP and unsecured SCTP. RAN3 would like to ask the IETF TSVWG to investigate and would greatly appreciate a solution to the issue related to the size limitation for DTLS over SCTP.

At RAN3-119 meeting, the IETF sent a LS back to RAN3 and SA3, with the following contents related to RAN3 [7].

We want to inform RAN3 that, due to the discovery of the security issues and the need to address them, there will be additional delay before an updated DTLS over SCTP specification will be done. It’s currently uncertain how long this will take, but it will likely take until at least to the later part of 2023 before a complete solution would be available.

At this RAN3-121 meeting, the IETF sent a rely LS based on the SA3 response in [1]. Basically, it can be observed that the security issue for the DLTS over SCTP is not yet solved, and further 3GPP WG input is needed.

# 3. Discussion

As specified in [1], two security solutions are proposed to address the security issue. One is to continue with previous solutions with DTLS on top of SCTP and relying on an updated version of SCTP-AUTH, the other one is to create an encryption chunk that encapsulates all the payload of SCTP. We understand RAN3 has no security expertise to provide comparison analysis for these two options. But RAN3 can provide RAN input to address the proposed questions.

Generally, there are two issues raised in the rely LS in [1].

- Issue 1: Requirements of network message size

It [1], IETF requests to clarify which SCTP message sizes that a solution is required to support, i.e., the theoretical maximum message, or would it sufficient that a smaller message size is supported. IETF also thinks that the message size of larger then 500 kb, which appear to the approximate theoretical maximum size of Xn (3GPP TS 38.423).

The 500kb, approximate the 62.5K bytes, can somehow meet the RAN requirements of the network message size. But we want to provide two highlights as follows.

- No hard limitation of the message size is expected from RAN perspective. This was also somehow mentioned in [2]. With the development of different release, more and more new features are introduced. It can be expected that the network message size would be extended continually. In this sense, no theoretical maximum message is expected either.

- It is expected to have a mechanism adapting to the ever-increasing message size. For example, a secure fragmentation mechanism should be in place to support the such continuously extended network message size. Note that over the Uu interface, due to the PDCP PDU size limitation (approximately 9K bytes), the RRC segmentation (especially for the RRC UE radio capability message) is introduced. The similar fragmentation is needed for the network interface.

- For the message size over the network interface, RAN3 expects neither hard limitation nor theoretical maximum message, considering that the message size would be extended continuously at later releases. RAN3 expects a mechanism adapting to the ever-increasing message size.

- Issue 2: userland and kernel implementations of SCTP.

IETF [1] also asks to confirm if implementation possibilities in both userland and kernel implementations of SCTP are required for the solution, and if any additional concerns with implementation of either of the solutions are perceived.

It is a hard for RAN3 to give thorough replies for the security related question, since RAN3 has no security expertise. This issue can be left to other groups, e.g., SA3.

- For the userland and kernel implementations of SCTP, RAN3 has no security expertise, and leaves it to SA3 feedback.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

- For the message size over the network interface, RAN3 expects neither hard limitation nor theoretical maximum message, considering that the message size would be extended continuously at later releases. RAN3 expects a mechanism adapting to the ever-increasing message size.

- For the userland and kernel implementations of SCTP, RAN3 has no security expertise, and leaves it to SA3 feedback.

# 5. Reference

- R3-233836	Reply LS on SCTP-AUTH and DTLS (S3-233355)

- R3-211274	Remove the user message size limitation for DTLS over SCTP, RAN3

- R3-210892	Remove the user message size limitation over DTLS/SCTP (Ericsson)

- R3-210893	[DRAFT] Remove the user message size limitation over DTLS/SCTP (Ericsson)

- R3-210456	Discussion on the size limitation of DTLS over SCTP (Huawei, China Telecom)

- R3-210457	Discussion on the size limitation of DTLS over SCTP (Huawei, China Telecom)

- R3-230873	Updated LS to 3GPP Regarding SCTP-AUTH and DTLS, IETF TSVWG.