- 3GPP TSG-RAN3 Meeting #121	R3-234630

- Toulouse, France, 21th Aug - 25th Aug 2023

Title:	[draft] LS to RAN2 on RAN3 progress on QoE in NR-DC

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:	Zhang Man

Zhang.man4@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- 1	Overall description

RAN3 has discussed the support for QoE in NR-DC during the RAN3#121 meeting. The following agreements/WAs are achieved:

- For s-based QoE configuration received by MN

- MN sends the QoE configuration via SRB1

- QoE reports can be sent via SRB4 or SRB5

- WA: The transparent reporting for RVQoE over RRC is not supported.

- Define two different reporting leg indications for QoE and RVQoE.

- For a UE in NR-DC, each legacy QoE configuration can have only one corresponding RVQoE configuration when needed.

- 2	Actions

To RAN2:

ACTION:

RAN3 respectfully asks RAN2 to consider the above RAN3 agreements in their work and provide the necessary specification support.

- 3	Dates of next RAN3 meetings

RAN3#121-bis			2023-10-09 - 2023-10-13		Xiamen, CN

RAN3#122			2023-11-13 - 2023-11-17		Chicago, US