3GPP TSG-RAN WG3 Meeting # 122	R3-237468

Chicago, USA,  13th  - 17th November, 2023

Agenda Item:	14.2

Source:	Google

Title:	(TP for L1L2Mob BL CR for TS 38.473) UE Context identification after successful cell switch

Document for:	Discussions & Approval

# 1	Introduction

The RAN3 impact on support of subsequent LTM is listed as one of the remaining open issues.

The Cell RNTI (C-RNTI) currenlty in use between the UE and the base station (CU and DU) can identify a UE context and a RRC connection. It is also used to help validate the UE, for example, in case of RRC connection resumption, RNA update, or RRC connection re-establishment as described in TS 38.331 [1] and TS 33.501 [2] as cited below.

“The ResumeMAC-I/shortResumeMAC-I is a 16-bit message authentication token, the UE shall calculate it using the integrity algorithm (NIA or EIA) in the stored AS security context, which was negotiated between the UE and the source gNB/ng-eNB and the current KRRCint with the following inputs: - KEY : it shall be set to current KRRCint; - BEARER : all its bits shall be set to 1. - DIRECTION : its bit shall be set to 1; - COUNT : all its bits shall be set to 1; - MESSAGE : it shall be set to VarResumeMAC-Input/VarShortInactiveMAC-Input as defined in TS 38.331 [22] for gNB and in TS 36.331 [69] for ng-eNB with following inputs: source PCI, target Cell-ID, source C-RNTI.” "When the target gNB/ng-eNB receives the RRCResumeRequest message from the UE, the target gNB/ng-eNB extracts the I-RNTI from the RRCResumeRequest message. The target gNB/ng-eNB contacts the source gNB/ng-eNB based on the information in the I-RNTI by sending an Xn-AP Retrieve UE Context Request message with the following included: I-RNTI, the ResumeMAC-I/shortResumeMAC-I and target Cell-ID, in order to allow the source gNB/ng-eNB to validate the UE request and to retrieve the UE context including the UE 5G AS security context. The source gNB/ng-eNB retrieves the stored UE context including the UE 5G AS security context from its database using the I-RNTI. The source gNB/ng-eNB verifies the ResumeMAC-I/shortResumeMAC-I using the current KRRCint key stored in the retrieved UE 5G AS security context (calculating the ResumeMAC-I/shortResumeMAC-I in the same way as described above).”

“The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.”

“In order to calculate the token, the source gNB shall use the negotiated NIA-algorithm from the 5G AS Security context from the source gNB with the following inputs: source C-RNTI, source PCI and target Cell-ID, where source PCI and source C-RNTI are associated with the cell the UE last had an active RRC connection with and target Cell-ID is the identity of the target cell where the RRCReestablishmentRequest is sent to. - KEY shall be set to KRRCint of the source cell; - all BEARER bits shall be set to 1; - DIRECTION bit shall be set to 1; - all COUNT bits shall be set to 1. The token shall be the 16 least significant bits of the output of the used integrity algorithm.” “If the target gNB receiving the RRCReestablishmentRequest has a prepared KNG-RAN* key and token for the specific cell, the target gNB receiving the RRCReestablishmentRequest shall validate the token received in the RRCReestablishmentRequest. However, if the target gNB has not prepared token for the cell, the target gNB extracts the C-RNTI and PCI from the RRCReestablishmentRequest message. The target gNB contacts the source gNB based on PCI by sending an Xn-AP Retrieve UE Context Request message with the following included: C-RNTI, PCI, the token and target Cell-ID, in order to allow the source gNB to validate the UE request and to retrieve the UE context including the UE 5G AS security context. ”

Observation 1: Cell RNTI can identify a UE context and a RRC connection. It is also used to help validate the UE.

For conditional mobility like LTM, a base station could prepare multiple LTM configurations and assign corresponding C-RNTIs to be used. Once the UE is commanded to apply one of the LTM configuration, a corresponding C-RNTI shall be applied for that cell group. Different from the legacy conditional mobility in Rel-16 and Rel-17, the prepared resources of other LTM configurations might be kept as subsequent LTM is supported.

Observation 2: As subsequent LTM is supported, the prepared resources of LTM configuration might be kept after cell switching.

It is noticed that the C-RNTI IE is optional in the UE Context Setup Response message and the UE Context Modification Response message. As only receiving side behavior is described for C-RNTI is captured in the current TS 38.473 [3] for the UE context response messages (i.e., during the preparation phase), it is not clear, upon LTM execution, whether and how the CU determines the C-RNTI to use (while keeping the other prepared C-RNTIs for other cell groups for subsequent cell switching) for the UE.

“If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.” “If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.”

After several rounds of subsequent LTM cell switching, without a clear guidance how the C-RNTI is determined after each LTM completion, the CU might lose track of the exact C-RNTI and fail in validating a UE during an RRC connection re-establishment or a RRC state transitions.

Since the C-RNTI is assigned by a DU and the DU commands and detects the UE access the target cell during LTM execution, the DU can provide the C-RNTI to the CU once the cell switching is successful. It is therefore proposed to add in the Access Success message a C-RNTI IE to help the CU to confirm the exact C-RNTI and the UE context to be used in communication with the UE.

Proposal: gNB-DU provides the applied C-RNTI in the Access Success message.

# 2 	Conclusion

In this paper, we discuss the necessity of updating C-RNTI upon LTM cell switching and have the following proposal.

Observation 1: Cell RNTI can identify a UE context and a RRC connection. It is also used to help validate the UE.

Observation 2: As subsequent LTM is supported, the prepared resources of LTM configuration might be kept after cell switching.

Proposal: gNB-DU provides the applied C-RNTI in the Access Success message.

# 3	References

[1] 3GPP TS 38.331v17.5.0

[2] 3GPP TS 33.501v17.7.0

[3] 3GPP TS 38.473v17.6.0

[4] R2-2311604, RRC running CR for LTM, Ericsson

# 4	Text Proposal

<<<<<< START OF CHANGES>>>>>>

### 8.3.8	Access Success

#### 8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or LTM. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO preparations or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled. Upon reception of the ACCESS SUCCESS message, the gNB-CU shall, if supported, consider that the UE context corresponding to the included C-RNTI IE is selected by the UE. The other LTM preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU are kept for subsequent LTM.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

#### 8.3.8.3	Abnormal Conditions

If the ACCESS SUCCESS message refers to a context that does not exist, the gNB-CU shall ignore the message.

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change or LTM.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI applied by the UE | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<< SKIP UNCHANGED >>>>>>

-- **************************************************************

--

#### -- Access Success

--

-- **************************************************************

AccessSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ AccessSuccessIEs}},

...

}

AccessSuccessIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	}|

{ ID id-C-RNTI								CRITICALITY ignore	TYPE C-RNTI									PRESENCE optional		},

...

}

<<<<<< END OF CHANGE >>>>