3GPP TSG-RAN WG3 #122	R3-237829

Chicago, USA, 13th – 17th November 2023

Agenda Item:	16.4

Source:	LG Electronics (moderator)

Title:	SoD of CB: # SLRelay

Document for:	Approval

# Introduction

This paper summarizes the following CB:

CB: # SLRelay - discuss on the above issues and merge all the agreements above. - check with stage-2 and stage-3 TPs. (moderator – LGE) Offline discussion summary in R3-237829

# For the Chairman’s Notes

Propose the following:

///To be added

# Discussion

## Inclusion of T420-like time into Indirect Path Addition IE

In this CB, RAN3 needs to further check whether the T420-like timer is included into the Indirect Path Addition IE or not.

During the offline discussion, ZTE mentioned the followings:

- In Rel-17, the Path Switch Configuration IE including the T420 IE is sent to the gNB-DU because the gNB-DU generates the CellGroupConfig IE including the SL-PathSwitchConfig-r17 IE. However, according to the RAN2 running CR [1], the SL-IndirectPathAddChange IE is included in the RRCReconfiguration message. This means that the gNB-CU itself can generate the SL-IndirectPathAddChange IE. Therefore, there is no need to include the T420-like timer into the Indirect Path Addition IE.

RRC running CR [1]: … RRCReconfiguration-v18xx-IEs ::= SEQUENCE { sl-IndirectPathAddChange-r18 SetupRelease { SL-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathAddChange-r18 SetupRelease { N3C-IndirectPathAddChange-r18 } OPTIONAL, -- Need M n3c-IndirectPathConfigRelay-r18 SetupRelease { N3C-IndirectPathConfigRelay-r18 } OPTIONAL, -- Need M otherConfig-v18xx OtherConfig-v18xx OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } … - – SL-IndirectPathAddChange The IE SL-IndirectPathAddChange specifies the configuration information of SL indirect path for SL indirect path addition/change. SL-IndirectPathAddChange information element -- ASN1START -- TAG-SL-INDIRECTPATHADDCHANGE-START SL-IndirectPathAddChange-r18 ::= SEQUENCE { sl-IndirectPathRelayUE-Identity-r18 SL-SourceIdentity-r17, sl-IndirectPathCellIdentity-r18 CellIdentity, t4xx-r18 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, ... } -- TAG-SL-INDIRECTPATHADDCHANGE-STOP -- ASN1STOP …

From the moderator’s view, ZTE’s comment seems to be reasonable. So, the moderator suggests the following proposal based on ZTE’s comment.

Proposal 1: The T420-like timer is not included into the Indirect Path Addition IE.

If the above proposal is agreed, this can be captured into the F1AP TP (in R3-237821, Huawei).

Question 1: Companies are invited to provide views on whether above proposal is agreeable.

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE | Yes |  |

# Conclusion, Recommendations [if needed]

If needed

# References

- R2-2311970, “RRC running CR for Rel-18 multi-path,” Huawei, HiSilicon, November, 2023.