3GPP TSG-RAN WG3 #122							R3-237789

13 – 17 November 2023

Chicago, USA

Agenda item:	10.2.5

Title: 	Summary of offline discussion on CB: # SONMDT4_NR-U

Source: 	Ericsson

Document Type:	Discussion and decision

# Introduction

CB: # SONMDT4_NR-U

- Discuss the open issue above

- Provide TPs if agreeable

(moderator - Ericsson)
Ref: R3-237789

# For the Chair’s Notes

The following proposals are made:

Proposal 1: In case of mobility, reporting of DL LBT failures at the target during handover execution is enabled in case of Handover failures when the UE reestablish to the source and the UE can be identified by the target. The reporting applies to normal HO and conditional HO.

Proposal 2: The number of DL LBT failures at the target during handover execution is provided if the source node requests for it.

Proposal 3: Agree the TP for stage 2 in R3-237207.

Proposal 4: Agree the TP for stage 2 in R3-237802.

Proposal 5: Agree the TP for stage 3 in R3-237803.

# Discussion

During the online discussion it was proposed to turn into an agreement the following WA reached at previous meeting:

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified).

It was agreed that some further details needed to be sorted out in relation to the scenarios, and corresponding stage 2 and stage 3 details to be discussed.

## Scenarios when number of DL LBT failures is reported

Based on the online discussion, it is moderator’s understanding that scenarios for which reporting of the number of DL LBT failures is enabled are:

- Too early Handover “like”, i.e., a certain handover execution fails, during which the target gNB suffered DL LBT failures. The target gNB can identify the UE, and the UE re-establishes towards the source gNB

- Handover Wrong Cell “like”, i.e., a certain handover execution fails, during which the target gNB suffered DL LBT failures. The target gNB can identify the UE, and the UE re-establishes towards a gNB that is not the source gNB nor the target gNB.

As further consideration, it is moderator’s understanding that RAN3 needs to address both “normal” HO and CHO.

Related to the handover failure scenarios above, R3-237187 (Nokia) proposes that when HO fails and UE re-establishes to the source gNB, the source uses the Failure Information to ask the target node to retrieve the number of failed transmission due to LBT. The target sends the number of failed transmission using the Handover Report message.

Moderator’s note: it is assumed that “Failure Indication” message is intended in the proposal above (there is no “Failure Information” message in Xn :-) ). In any case, such message belongs to a class 2 procedure, so it does not seem appropriate to “ask” something, since there is no corresponding “return” message.

In R3-237404 (Lenovo), it is proposed that a new Xn message is used instead, to indicate the DL LBT failures.

Q1) What is your view with respect to the scenarios to be supported for reporting of DL LBT failures between target gNB and source gNB?

| Company | Comment |
| --- | --- |
| Ericsson | We don’t see big benefit to enable reporting of DL LBT failures and in any case both “normal” HO and CHO need to be considered. As a compromise we could agree that target sends to source some information on DL LBT failures at the target, but only if the source requests for it already at HO preparation (in the CHO case, the source has no clue of when the UE will perform the handover), and if the target can identify the UE for with HO failed. The sending of DL LBT failures would occur: - in case of HO with contention-based RA, after Msg3 arrives at target. - in case of HO with contention free RA |
| Lenovo | We are fine to consider the scenarios to be supported for reporting of DL LBT failures between target gNB and source gNB: - in case of HO with CBRA after Msg3 arrives at target; - in case of HO with CFRA; - in case of 2-step CBRA. |

Q2) How to enable the reporting of DL LBT Failures from target gNB to source gNB for the scenarios identified above?

| Company | Comment |
| --- | --- |
| Ericsson | For stage 2 TS 38.300 describes the following actions as part of the detection mechanism: In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report. Some additions are needed to distinguish the “legacy” TEH / HWC scenarios where the UE reconnects to the source and the target suffered DL LBT failures during handover execution. For stage 3 We prefer to reuse legacy procedures, it looks simpler. As already commented above, we have raised concerns in the past on how tackle the CHO case, since the time of HO execution is not known by the source. At least the source should request to receive DL LBT failures the target suffered during handover execution (and this can apply to both normal HO and CHO), but reusing the Failure Indication message does not seem right. So, for stage 3 we propose that HANDOVER REQUEST is reused, adding a new optional IE to ask for this information. If the target suffers DL LBT failures during HO/CHO execution and can pinpoint the UE, it sends the number of DL LBT failures using the legacy HANDOVER REPORT message. |
| Lenovo | As SS commented in the TP for TS38.423, HANDOVER REPORT message is unsuitable, because the included IE “Handover Report Type” and “Handover Cause” are mandatory but they are not for the NR-U case. For SS proposed ACCESS AND MOBILITY INDICATION message, it may be not suitable either, because currently this message is used for near-failure successful case rather than failure case. It seems better to introduce a new Xn message to indicate the DL LBT failures. |

The following proposals are made for further check:

Proposal 1: In case of mobility, reporting of DL LBT failures at the target during handover execution is enabled in case of Handover failures when the UE reestablish to the source and the UE can be identified by the target. The reporting applies to normal HO and conditional HO.

Proposal 2: The number of DL LBT failures at the target during handover execution is provided if the source node requests for it.

## TPs for stage 2

A stage 2 TP for TS 38.300 was proposed in R3-237207 and discussed online. It is moderator’s understanding that with the clarification that the proposed TP only applies to Rel-18, the TP can be agreed.

Proposal 3: Agree the TP for stage 2 in R3-237207.

Regarding how to capture stage 2 impacts related to the sending of DL LBT failures at target, it is proposed to use as basis the TP submitted in R3-237508 (and revised in R3-237802). The original Tdoc already contained clarifications to TS 38.300 to capture the agreements reached at previous RAN3 meetings on LBT failures, and the new impact can be captured in the same document.

Companies are invited to check the content of the draft TP provided in R3-237802 and provide feedback directly in the file.

Proposal 4: Agree the TP for stage 2 in R3-237802.

## TP for stage 3

Regarding how to capture stage 3 impacts related to the sending of DL LBT failures at target, a draft TP is provided in the server (ref: R3-237803).

Companies are invited to check the content of the draft TP provided in R3-237803 and provide feedback directly in the file.

Proposal 5: Agree the TP for stage 3 in R3-237803.