3GPP TSG RAN3 123-bis	R3-242035

# Changsha, China, 15 - 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1271 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to QMC support in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RAN2 has defined two separate parameters on whether the segmentation of RRC message for QoE reporting are enabled on SRB4 and SRB5, i.e. rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5. However, if the QoE configuring node requests the non QoE configuring node to receive the QoE report, the QoE configuring node cannot know whether the non QoE configuring node enables RRC segmentation for QoE report according to the current spec. Therefore, the new coordination is needed. - RAN2 has defined a session status information in the QoE report to indicate whether the application layer session has started or stopped. However, if the QoE reporting leg is configured to be over the non QoE configuring node, the QoE configuring node cannot obtain the session status information according to the current spec, resulting in the situation that the QoE configuring node cannot release the QoE/RVQoE configurations based on the session status information. Therefore, the new coordination is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add RRC Segmentation Allowed Request IE in QMC Information Request, and RRC Segmentation Allowed Response IE in QMC Information Response - Add Application Layer Session Status IE in RRC TRANSFER message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some sub-features for QMC in NR-DC are not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.20, 9.2.3.197, 9.2.3.198, 9.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

9.1.2.20	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message or from the S-NG-RAN-NODE to the M-NG-RAN-NODE to report the DL RRC message delivery status.

This message is also sent by the new NG-RAN-NODE to the old NG-RAN-NODE or from the old NG-RAN-NODE to the new NG-RAN-NODE to transfer an RRC message containing the SDT SRB in case of RACH based SDT without UE context relocation.

This message is also sent by the M-NG-RAN-NODE to the S-NG-RAN node or from the S-NG-RAN node to the M-NG-RAN node to forward QoE measurement results and/or the RAN visible QoE measurement results received from the UE.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node (Dual Connectivity).

Direction: new NG-RAN node  old NG-RAN node or old NG-RAN node  new NG-RAN node (SDT).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| >>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<< |  |  |  |  |  |  |
| SDT SRB between New NG-RAN node and Old NG-RAN node |  | 0..1 |  |  | YES | ignore |
| >RRC Container | M |  | OCTET STRING | Contains a PDCP-C PDU encapsulating an RRC message as defined in subclause 6.2.1 of TS 38.331 [10]. | – |  |
| >SRB ID | M |  | 9.2.3.165 | In this version of the specification, values "0", "1", "3", “4” and "5" are not set by the sender and ignored by the receiver. | – |  |
| QoE Measurement Results |  | 0..1 |  |  | YES | ignore |
| >QoE Reference | M |  | OCTET STRING(SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. | – |  |
| >RRC Container for RAN Visible QoE Report | O |  | OCTET STRING | Contains the RAN-VisibleMeasurements IE of the MeasurementReportAppLayer message as defined in TS 38.331 [10]. | – |  |
| >RRC Container for QoE Report | O |  | OCTET STRING | Contains the measReportAppLayerContainer IE of the MeasurementReportAppLayer message as defined in TS 38.331 [10]. | – |  |
| >Application Layer Session Status | O |  | ENUMERATED (start, stop, ...) | The session status of the application layer. |  |  |

-------------------------------------------Next change-------------------------------------------

9.2.3	General IE definitions

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-------------------------------------------Next change-------------------------------------------

9.2.3.197	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. This IE is allocated by the M-NG-RAN node. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, …) | This IE indicates that the configuration has been released by the M-NG-RAN node. |
| >>RRC Segmentation Allowed Request | O |  | ENUMERATED (true, …) | This IE is used to request on whether the S-NG-RAN node enables RRC segmentation over SRB5. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the S-NG-RAN node when the S-NG-RAN node is the RAN Visible QoE configuring node and the M-NG-RAN node provides the bearers for the application session, to request from the M-NG-RAN node to indicate whether the M-NG-RAN node is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration. |
| >>Available RAN Visible QoE Metrics |  |  | 9.2.3.158 |  |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the S-NG-RAN node. |
| >>RRC Segmentation Allowed Request | O |  | ENUMERATED (true, …) | This IE is used to request on whether the M-NG-RAN node enables RRC segmentation over SRB4. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

9.2.3.198	QMC Coordination Response

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the M-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (mn, sn, …) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred path for further RAN Visible QoE reporting. |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| >>RRC Segmentation Allowed Response | O |  | ENUMERATED (enabled, not enabled, …) | This IE is used to indicate whether the M-NG-RAN node enables RRC segmentation over SRB4. |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE indicates whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred path for further RAN Visible QoE reporting. |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>RRC Segmentation Allowed Response | O |  | ENUMERATED (enabled, not enabled, …) | This IE is used to indicate whether the S-NG-RAN node enables RRC segmentation over SRB5. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

-------------------------------------------Next change-------------------------------------------

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

QoE-Measurement-Results ::= SEQUENCE {

qOEReference								OCTET STRING (SIZE(6)),

rrcContainerForRVQoEReport					OCTET STRING									OPTIONAL,

rrcContainerForQoEReport					OCTET STRING									OPTIONAL,

appLayerSessionStatus						ENUMERATED {start, stop, ...}					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {QoE-Measurement-Results-ExtIEs} }	OPTIONAL,

...

}

QoE-Measurement-Results-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

#### -- Q

QMCConfigInfo ::= SEQUENCE {

uEAppLayerMeasInfoList			UEAppLayerMeasInfoList,

iE-Extensions					ProtocolExtensionContainer { {QMCConfigInfo-ExtIEs} }	OPTIONAL,

...

}

QMCConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEAppLayerMeasInfoList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfo-Item

UEAppLayerMeasInfo-Item ::= SEQUENCE {

uEAppLayerMeasConfigInfo	UEAppLayerMeasConfigInfo,

iE-Extensions				ProtocolExtensionContainer { { UEAppLayerMeasInfo-Item-ExtIEs} } 	OPTIONAL,

...

}

UEAppLayerMeasInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QMCCoordinationRequest ::= SEQUENCE {

mN-to-SN-QMCCoordRequestList	MN-to-SN-QMCCoordRequestList,

sN-to-MN-QMCCoordRequestList	SN-to-MN-QMCCoordRequestList,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationRequest-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MN-to-SN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordRequestList-Item

MN-to-SN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID						OPTIONAL,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress				OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}					OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}					OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}						OPTIONAL,

furtherRVQoEReportingPathInquiry		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig									OPTIONAL,

availableRVQoEMetrics			AvailableRVQoEMetrics						OPTIONAL,

configReleaseIndication			ENUMERATED{rvqoe,qoe-and-rvqoe, ...}		OPTIONAL,

rRCSegmentationAllowedRequest	ENUMERATED{true, ...}					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordRequestList-Item

SN-to-MN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress				OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}					OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}					OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}						OPTIONAL,

furtherRVQoEReportingPathInquiry		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig									OPTIONAL,

availableRVQoEMetrics		AvailableRVQoEMetrics							OPTIONAL,

configReleaseIndication			ENUMERATED{rvqoe,qoe-and-rvqoe, ...}		OPTIONAL,

rRCSegmentationAllowedRequest	ENUMERATED{true, ...}					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QMCCoordinationResponse ::= SEQUENCE {

mN-to-SN-QMCCoordResponseList	MN-to-SN-QMCCoordResponseList,

sN-to-MN-QMCCoordResponseList	SN-to-MN-QMCCoordResponseList,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationResponse-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MN-to-SN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordResponseList-Item

MN-to-SN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID						OPTIONAL,

qoEConfigSendingPath			ENUMERATED{mn, sn, ...}						OPTIONAL,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}			OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 			OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{interested, not-interested, ...}	OPTIONAL,

furtherRVQoEReportingPathResponse		ENUMERATED{srb4, srb5, ...}			OPTIONAL,

preferredRVQoEConfig			RVQoEConfig									OPTIONAL,

rRCSegmentationAllowedResponse	ENUMERATED{enabled, not enabled, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordResponseList-Item

SN-to-MN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}										OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 										OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{interested, not-interested, ...}								OPTIONAL,

furtherRVQoEReportingPathResponse		ENUMERATED{srb4, srb5, ...}										OPTIONAL,

preferredRVQoEConfig			RVQoEConfig																OPTIONAL,

rRCSegmentationAllowedResponse	ENUMERATED{enabled, not enabled, ...}									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------End of changes-------------------------------------------