3GPP TSG-RAN WG3 Meeting #123bis	R3-242131

# Changsha, China,  15th - 19th April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on non-homogeneous deployment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, ZTE, Huawei, CATT, Ericsson, Qualcomm Inc, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The non-homogeneous scenario is about gNBs have different capabilities on the PDU set based handling. Current Stage-2 only describe the handover scenario, but not cover the RRC_INACTIVE UE and RRC Re-establishment. For example, - during the network triggered transition from RRC_INACTIVE to RRC_CONNECTED, the UE may connect with a new serving gNB that is different to last serving gNB. The new serving gNB and last serving gNB may have different capability regarding the support of PDU Set handling. - during the RRC re-establishment procedure, the new serving gNB and last serving gNB may have different capability regarding the support of PDU Set handling. It is necessary to add the missing scenario for non-homogeneous deployment. The section uses both “NG-RAN node” and “gNB”. It is better to align the term to use “gNB”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the transition from RRC_INACTIVE to RRC_CONNECTED, and re-establishment are also part of the non-homogeneous deployment. Change the term “NG-RAN node” to “gNB”. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only add the missing scenario for Non-Homogeneous support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how the new serving gNB handles the PDU Set Handling during the transition from RRC_INACTIVE to RRC_CONNECTED, or during the RRC re-establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.15.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR .. CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR .. CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

### 16.15.5	Non-Homogeneous support of PDU set based handling in NG-RAN

During a handover from a gNB supporting PDU Set based handling to another gNB, the source gNB signals the PDU Set Information over Xn-U if the target node has signalled the support of PDU Set based handling in the Xn Handover Request Acknowledge message.

During a handover, transition from RRC_INACTIVE to RRC_CONNECTED or RRC re-establishment from a gNB not supporting PDU Set based handling to a gNB supporting PDU Set based handling, the target/new serving gNB may indicate the support of PDU Set based handling to the SMF during the Path Switch Request procedure  or Handover Resource Allocation procedure (in case of NG handover), the SMF will act as specified in TS 23.501[3]. If the indication is absent, the SMF infers that PDU Set based handling is not supported by the target/new serving gNB, then the SMF will act as specified in TS 23.501[3].

During a handover, transition from RRC_INACTIVE to RRC_CONNECTED or RRC re-establishment from a gNB not supporting PDU Set based handling to a gNB supporting PDU Set based handling, the target/new serving gNB may receive unmarked PDU(s) (i.e. PDU(s) without PDU Set Information Container) forwarded from the source/last serving gNB , and marked PDU(s) (i.e. PDU(s) with PDU Set Information Container) from UPF, how the target/the new serving gNB handles the marked and unmarked PDUs for the same QoS flow is up to implementation.

-----------------End of the Change-------------------