3GPP TSG-RAN WG3 Meeting #123-bis	R3-24xxxx

Changsha, China, 15 - 19 April 2024

Agenda Item:	21.2

Source:	ZTE, Nokia, Nokia Shanghai Bell

Title:	(TP to TS 37.340) Support of XR in DC

Document for:	Discussions & Approval

# 1	Introduction

This TP captures the agreements achieved in RAN3 #123-bis meeting.

# 2	Text Proposal for BL CR TS 37.340

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling. If so, the SN reports the PDU Set based Handling Indicator in the SN Addition Preparation procedure and MN initiated SN Modification Preparation procedure for the MN-terminated SCG bearer, SN-terminated MCG bearer and SN-terminated SCG bearer.

### 13.x.3	Discard

The gNB hosting the PDCP notifies the corresponding gNB whether the PSI based SDU discarding is (re)configured/released for uplink discarding.

### 13.x.4	Non-Homogeneous support of PDU set based handling

If the MN supports PDU Set based handling, the MN signals the PDU Set Information over Xn-U if the SN has signaled the support of PDU Set based handling in the SN Addition Request Acknowledge message or the SN Modification Request Acknowledge message.

### 13.x.4	Discard

The PDCP hosting node notifies the corresponding node whether the PSI based SDU discarding is (re)configured/released for uplink discarding.

### 13.x.5	ECN marking for L4S and congestion information exposure

For ECN marking for L4S at gNB, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request to SN and SN report the status indication back to the MN.

For ECN marking for L4S at UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking UPF request to SN and SN report the status indication back to the MN.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN report the status indication back to the MN.