3GPP TSG-RAN WG3 Meeting #123-bis	R3-24xxxx

Changsha, China, 15 - 19 April 2024

Agenda Item:	21.2

Source:	ZTE, Nokia, Nokia Shanghai Bell

Title:	(TP to TS 37.340) Support of XR in DC

Document for:	Discussions & Approval

1	Introduction

This TP captures the agreements achieved in RAN3 #123-bis meeting.

2	Text Proposal for BL CR TS 37.340

## 13.x	eXtended Reality Services

13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling. .

### 13.x.3	Discard

For MN terminated SCG bearer, the MN notifies SN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling

For SN terminated MCG bearer, the SN notifies MN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling

Editor Note: FFS for split bearer and DL PSI discard

### 13.x.4	Discard

The PDCP hosting node notifies the corresponding node whether the PSI based SDU discarding is (re)configured/released for uplink discarding.

### 13.y	ECN marking for L4S and congestion information exposure

For ECN marking for L4S at gNB, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request to SN and SN report the status indication back to the MN.

For ECN marking for L4S at UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking UPF request to SN and SN report the status indication back to the MN.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN report the status indication back to the MN.