3GPP TSG-RAN WG3 #123bis	R3-24xxxxx

Changsha, 15 – 19 Apr. 2024

Agenda Item:	12.2

Source:	NTTDOCOMO

Title:	TP for WAB general requirements and architecture

Document for:	Agreement

Introduction

This document provides a TP for WAB general requirements and architecture.

Discussion

Proposal1: RAN3 to agree the TP for WAB general requirements and architecture in ANNEX.

ANNEX (WAB TP for TR 38.799)

Start of Change

- X	Wireless Access Backhaul (WAB)

- X.1	General

The study is based on the following requirements:

- the WAB-node includes a gNB component (WAB-gNB) and a MT component (WAB-MT));

- the WAB-gNB is based on the gNB functionality specified in TS 38.300 [4] and TS 38.401 [5]. CU-DU split is not considered in this study;

- the WAB-MT needs to support the subset of UE functionality;

- NR Uu is used for the radio link between an WAB-gNB and served UEs. The NR Uu radio link between the WAB-gNB and served UE does not use NTN;

- In-band scenario is not precluded to be studied;

- the study precludes the scenario where the access and backhaul is in-band and backhaul use NTN;

- the study focus on the using PDU session backhaul. Other non-NR uu (including non-3GPP radio technology) for the backhaul is not precluded (but not part of the study);

- the WAB only supports single hop, which means a WAB-node does not allow any descendant WAB-node to access it;

- the study includes a scenario where the WAB-gNB and the WAB-MT connected to the same or different PLMN

- the WAB-MT may connect to a PLMN or an SNPN;

- the WAB-gNB may connect to a PLMN or an SNPN;

-	legacy UEs can connect to the WAB-node, and there are no WAB-specific enhancements to Rel-19 UEs.

- X.2	WAB Architecture

Figure X.Y-1 shows an example of wireless access and backhaul (WAB) architecture for 5GS:

Figure X.Y-1: WAB architecture for 5GS

Figure X.Y-2 shows an example of wireless access and backhaul (WAB) architecture for 5GS (non-3GPP backhaul):

Figure X.Y-: WAB architecture for 5GS (non-3GPP backhaul)

Figure X.Y-3 shows protocol stack examples of control plane and user plane transport for a UE connected to the network via a WAB-node.

Figure X.Y-3: Protocol stack examples of control plane and user plane transport for UE connected via WAB-node

End of Change