3GPP TSG-RAN3 Meeting #123                                                          R3-240886

- Athens, Greece, Feb 26 – Mar 2, 2024

Agenda Item:	9.1.3.1

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # AIML2_ASN.1

Document for:	Approval

# Introduction

This SoD is to discuss the following CB:

CB: # AIML2_ASN.1

- ASN.1 change on DataCollectionID

- Use individual IE names between legacy Resource Status Update procedure and Data Collection Request procedure?

- BITSTRING(SIZE(32))

- Other corrections?

- Provide single CR for TS38.423

(moderator - ZTE)

# For the Chairman’s Notes

Modify the ASN.1 of DATA COLLECTION ID IE in the HANDOVER REQUEST Message.

Modify the size of bitmap for Node Measurement Failed Request Characteristics IE and Cell Measurement Failed Request Characteristics IE as 32.

Correct the ASN.1 for the Requested Prediction Time IE.

Modify the ASN.1 of cell ID in the DATA COLLECTION RESPONSE/UPDATE message, which should be refer to GlobalNG-RANCell-ID.

Clean Miscellaneous correction in ASN.1, and update ASN.1 to align with the agreement in CB: # AIML3_Stage3.

R3-241086, Revision of R3-240456, Agreed.

# Discussion

## ASN.1 change on Data Collection ID

Regarding the ASN.1 of DATA COLLECTION ID IE in the HANDOVER REQUEST message, it should be NG-RAN node1 Measurement ID and NG-RAN node2 Measurement ID rather than referring to NG-RANnodeUEXnAPID.

Corretion:

- -- A

DataCollectionID ::= SEQUENCE {

ng-ran-node1MeasurementID					Measurement-ID		OPTIONAL,

ng-ran-node2MeasurementID					Measurement-ID		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DataCollectionID-ExtIEs} } OPTIONAL,

...

}

DataCollectionID-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Modify the ASN.1 of DATA COLLECTION ID IE in the HANDOVER REQUEST Message.

## BITSTRING issues

Upon checking DATA COLLECTION REQUEST message and DATA COLLECTION RESPONSE message, the size of bitmap for request characteristic is misaligned. It is 32 in the request message, while it is 128 in the response message. Therefore, we propose to use 32 as the size of bitmap for failed request characteristics.

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

Modify the size of bitmap for Node Measurement Failed Request Characteristics IE and Cell Measurement Failed Request Characteristics IE as 32.

## Requested Prediction Time

There is misalignment between tabular and ASN.1 for requested prediction time.

RequestedPredictionTime ::= INTEGER (1..60, ...)

Correct the ASN.1 for the Requested Prediction Time IE.

Requested Prediction Time IE in Data Collection Request procedure is indicated as an optional IE. However, this IE is only applicable to predictions. Presence of Requested Prediction Time is updated to be conditional on the first, second and third bits of the Report Characteristics IE being set.

| Requested Prediction Time | C-ifPredictionIsRequested |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent reported message, the point in time is shifted by the reporting period. (unit: second) | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifPredictionIsRequested | This IE shall be present if at least one of the first, second or third bits of the Report Characteristics IE is set. |

Proposal 4: Presence of Requested Prediction Time is updated to be conditional on the first, second and third bits of the Report Characteristics IE being set.

In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity.

## Cell ID

Proposal 1: Semantics description of Global NG-RAN Cell Identity is updated such as NG-RAN Cell Identity is an NR Cell Identity in case the request pertains at least to one prediction.

Following semantic description is proposed to captured into DATA COLLECTION REQUEST message and DATA COLLECTION RESPONSE message.

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. This IE is only present if at least one of the first, second, or third bits of the Report Characteristics IE is set to ‘1’. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR cell if at least one of the first, second, or third bits of the Report Characteristics IE is set to ‘1’. | – |  |

Proposal 2: In clauses 9.1.3.26 (Request Message), 9.1.3.27 (Response Message), 9.1.3.29 (Update Message), 9.2.3.181 (Predicted Trajectory Information) and 9.2.3.183 (Measured Trajectory Cell Information) it is clarified that cell IDs (encoded as Global NG-RAN Cell Identities) actually refer to NR cell identities.

Indicates an NR Cell Identity.

ASN.1	correction:

Based on the discussion above, discuss whether to modify the ASN.1 of cell ID, which should be refer to GlobalNG-RANCell-ID.

## Individual IE name for DATA COLLECTION Procedure

Registration Request IE - >  Registration Request for Data Collection IE

Report Characteristics IE -> Report Characteristics for Data Collection IE

Reporting Periodicity IE -> Reporting Periodicity for Data Collection IE

## Other corrections

1.

#### 9.2.3.183	Measured Trajectory Cell Information

The Measured Trajectory Cell Information IE contains the cell IDs of the NG-RAN cells where a UE connected after being handed over to the target NG-RAN node, as well as the time the UE stayed in each visited cell.

2.

#### 9.2.3.185	UE Trajectory Collection Configuration

This IE contains configurations for UE trajectory collection after successful handover.

3.

| >>Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, in seconds. If the duration is more than 4095s, this IE is set to 4095. |
| --- | --- | --- | --- | --- |

4.

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

# Conclusion, Recommendations [if needed]

If needed

# References