3GPP TSG-RAN WG3 Meeting #124 	R3-243186

# Fukuoka, Japan, May 22nd – 26th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Handover Cancel in CHO with SN for MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Samsung, China Telecom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CHO with candidate SCG in MR-DC, multiple candidate target MNs with candidate target SNs are prepared. When the UE selects one target MN & target SN, the source MN sends Handover Cancel to other candidate MNs and may send it to target MN if the target MN is configured with other candidate PCell. Then candidate MNs and target MN trigger SN release to the corresponding SNs. In case both other candidate MN and target MN configured PScell in the target SN, the behaviour of candidate MN and target MN are different upon receiving Handover Cancel message from the source MN: - For target MN, the target MN doesn’t send SN Release to the target SN since the target MN knows this SN is selected as target SN by the UE. - For other MN, the other MN needs to send SN Release to the candidate SNs including target SN, since the other MN doesn’t know which SN is selected as the target SN by the UE. However, in the current figure 10.19.2-1, the other MN configured SCG in the target SN, but miss to send SN release to release the SCG in the target SN . Impact Analysis: Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Figure 10.19.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imply the other MN knows which candidate SN is selected by the UE as the target SN which is not true. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

10.19.2	MR-DC with 5GC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.2-1: Conditional Handover with Secondary Node procedure

<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>