3GPP TSG-RAN3 Meeting #124	R3-243309

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0370 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Huawei, ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-5-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In the present TS 38.401 Sections 8.9.17.2.2/8.9.17.3 for IAB authorization in SA, it says that the IAB-donor performs the SA equivalent of the steps described for NSA in 8.9.17.1 upon reception of the IAB-node authorization status, however, it’s more accurate that Section 8.9.17.2.2 and 8.9.17.3 refer to 8.9.17.2.1 instead of 8.9.17.1, since 8.9.17.2.1 can be reused more. - “Path Switch procedure” used in 8.9.17.2.1 is not correct wording. - 8.9.17.2.1 misuses the term “mobile IAB”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 8.9.17.2.2 and 8.9.17.3, change the sentence “performs the SA equivalent of the steps described for NSA in 8.9.17.1” to “performs the same steps described in 8.9.17.2.1” in several places. Change “Path Switch procedure” in 8.9.17.2.1 to “Path Switch Request procedure”. In 8.9.17.2.1, the misuse of the term “mobile IAB” is corrected. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, will only impact the RAN handling for the IAB-node authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The wording is not correct or misleading in 8.9.17.2.2 and 8.9.17.3 for IAB authorization in SA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.17.1, 8.9.17.2.1, 8.9.17.2.2, 8.9.17.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Compared with Rev#0, Rev#1 removes the changes with respect to the handling for IAB-node de-registration. - Use “IAB inter-CU topology adaptation” instead of “inter-CU topology adaptation” in 8.9.17.2.1. |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

8.9.17 IAB-node authorization

8.9.17.1	IAB-node in NSA

During the IAB-node integration procedure, the eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the updated authorization status is “not authorized”, the IAB-donor-CU performs the following actions in this order: it attempts to hand over the UEs and descendant nodes served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node.

In case the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the IAB-node integration procedure as defined in clause 8.12 are carried out.

8.9.17.2	IAB-node with single IAB-donor in SA

8.9.17.2.1 IAB-node is single-connected

During the IAB-node network integration or RLF recovery, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. Also, during the IAB inter-CU topology adaptation procedure, the target IAB-donor-CU receives the authorization status of the IAB-node from the source IAB-donor-CU as well as from the 5GC when performing the Path Switch Request procedure. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to the IAB-donor-CU. When the authorization status received by the IAB-donor-CU changes, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in clause 8.9.17.1.

8.9.17.2.2 IAB-node is NR dual-connected

In case the IAB-node is dual-connected to the IAB-donor-CU, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. Upon reception of the authorization status, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1.

In case the IAB-node is dual-connected to a non-IAB-capable gNB and to an IAB-donor-CU, the MN receives the authorization status of the IAB-node from the 5GC. If the MN is the non-IAB-capable gNB and the SN is the IAB-donor-CU, the MN forwards the authorization status to the IAB-donor-CU in the S-NODE ADDITION REQUEST message or S-NODE MODIFICATION REQUEST message. Upon reception of the authorization status, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1. If the MN is the IAB-donor-CU and the SN is not the non-IAB-capable gNB, the IAB-donor-CU performs the same steps described in clause 8.9.17.2.1.

8.9.17.3	IAB-node is served by two IAB-donors in SA

In case the IAB-MT only connects to the non-F1-terminating IAB-donor-CU or in case the IAB-MT is NR dual-connected with the non-F1-terminating IAB-donor-CU as the MN, the non-F1-terminating IAB-donor-CU sends the authorization status received from the 5GC to the F1-terminating IAB-donor-CU in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message. Upon reception of the authorization status, the F1-terminating IAB-donor-CU performs the same steps described in clause 8.9.17.2.1. If the authorization status is “not authorized”, the F1-terminating IAB-donor-CU sends to the non-F1-terminating IAB-donor-CU an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message requesting the release of all offloaded traffic, after which the non-F1-terminating IAB-donor-CU releases the offloaded traffic and all backhaul resources, BAP address, TNL address and default BAP configuration for the IAB-node.

In case the IAB-MT is NR dual-connected, where the MN is the F1-terminating IAB-donor-CU and the SN is a non-F1-terminating IAB-donor-CU, upon reception of the authorization status, the IAB-node’s authorization procedure follows the same steps as described in clause 8.9.17.2.1.

-------------------------------------------End of changes-------------------------------------------