3GPP TSG-RAN WG3 Meeting #124	R3-243389

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	Discussion on integration procedure for WAB node

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) fucus on RAN3 and RAN2.

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

In this contribution, the procedures for WAB-node integration are discussed.

- Discussion

A WAB-node composes a MT part and a gNB part, then in case of WAB-node integration, the procedure at least includes MT part setup and gNB part setup. A high-level flow chart for WAB-node integration is shown in the figure below.

Figure 1: WAB-node integration procedure

At the beginning stage of study item, maybe the scenario where WAB-MT and WAB-gNB integrate to same AMF can be firstly studied, and the scenario where WAB-MT and WAB-gNB integrate to different AMFs can be evaluated at the later stage. Following are the brief procedures for WAB-node integration to the same AMF.

Phase 1: WAB-MT setup

For the WAB-MT part setup, it is equivalent to the integration procedure of IAB-node and mobile IAB-node. However, some indications need to be introduced for WAB-node. Firstly, the WAB-MT connects to the network in the same way as a UE, by performing RRC connection setup procedure with gNB-CU serving WAB-MT, authentication with the core network, WAB-node-related context management, WAB-node’s access traffic-related radio bearer configuration at the RAN side (SRBs and optionally DRBs), and, optionally, OAM connectivity establishment by using the WAB-MT’s PDU session.

In this phase, the WAB-MT includes a WAB-node indication in RRCSetupComplete message, to assist the gNB serving WAB-MT to select an AMF supporting WAB, and the gNB serving WAB-MT further includes another WAB-node indication to AMF in the NGAP INITIAL UE MESSAGE for WAB-node registration. Alternatively, as discussed in SA2 [2], there may be no need to specify AS level indication in RRC connection establishment for WAB-node, and dedicated S-NSSAI(s) for WAB operation may be used for the WAB-MT.

In addition, the cell(s) of the gNB serving WAB-MT may need to provide the WAB-barred information in SIB1 for the access control of WAB-MT.

Proposal 1: GNB serving WAB-MT provides the WAB-barred information in SIB1 for the access control of WAB-MT.

Phase 2: BH connectivity

After WAB-MT setup, one or more PDU session between WAB-MT and 5GC can be established to transfer the NG related traffic between WAB-gNB and 5GC or the OAM traffic.

In addition, the WAB-node will obtain itself IP address in this phase for NG setup with AMF. Based on IP address allocation for IAB-node, the IP address of the WAB-node may be also allocated by OAM or the gNB serving WAB-MT. In addition, the transport connection between the IAB-node and its OAM can use the IP address provided by the IAB-MT’s PDU session via 5GC. Similarity, the NG setup between WAB-gNB and AMF is transported via the PDU session of WAB-MT, then, the WAB-gNB may also reuse the IP address provided by the WAB-MT’s PDU session to setup the NG connection.

Proposal 2: An WAB-node may obtain IP address from the gNB serving WAB-MT, from the OAM system, or reuse the IP address provided by the WAB-MT’s PDU session via 5GC.

Phase 3: WAB-gNB setup

In this phase, the WAB-gNB is configured via OAM. Then the WAB-gNB initiates the TNL establishment, NG setup with the AMF for WAB-node using the allocated IP address(es) in phase 2, and the UL/DL traffic for NG setup is transported via the PDU session established in phase 2. After the NG is set up, the WAB-node can start serving the UEs.

Based on the discussion above, a TP to TR 38.799 for WAB-node integration procedure is provided in the Appendix part.

Proposal 3: Agree the TP for WAB-node integration procedure in the Appendix.

# Conclusion

This contribution aims to discuss WAB-node integration procedure. And following observations and proposals are concluded.

Proposal 1: GNB serving WAB-MT provides the WAB-barred information in SIB1 for the access control of WAB-MT.

Proposal 2: An WAB-node may obtain IP address from the gNB serving WAB-MT, from the OAM system, or reuse the IP address provided by the WAB-MT’s PDU session via 5GC.

Proposal 3: Agree the TP for WAB-node integration procedure in the Appendix.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- 3GPP TR 23.700-06. Study on architecture enhancements for vehicle-mounted relays - Phase 2 (Release 19). V0.3.0.

# Appendix: TP to TR 38.799 for WAB-node integration

## 4.3	Operational aspects

### 4.3.x	WAB-node integration

The WAB-node integration procedure is shown in Figure 4.3.x-1.

Figure 4.3.x-1: WAB-node integration procedure

Phase 1: Equivalent procedure to Phase 1 of the IAB-node integration in SA mode in clause 8.12.1, where the WAB-node and the gNB serving WAB-MT correspond to IAB-node 2 and the IAB-donor, respectively, and there is no intermediate node between the WAB-node and the gNB serving WAB-MT. The WAB-node selects the gNB serving WAB-MT based on a WAB-specific over-the-air indication (transmitted in SIB1).

Phase 2: One or more PDU sessions between WAB-MT and MT’s UPF can be established to transfer the NG related traffic between WAB-gNB and 5GC or the OAM traffic. In this phase, the WAB-node will obtain itself IP address in this phase for NG setup with AMF.

Phase 3: In this phase, the WAB-gNB is configured via OAM. Then the WAB-gNB initiates the TNL establishment, NG setup with the AMF for WAB-node using the allocated IP address(es) in phase 2, and the UL/DL traffic for NG setup is transported via the PDU session established in phase 2. After the NG is set up, the WAB-node can start serving the UEs.