3GPP TSG-RAN WG3 Meeting #124	             R3-243727

Fukuoka, Japan, May 20- May 24, 2024

Agenda Item:	17.2

Source:	CMCC

Title:	Discussion on On-Demand SSB SCell Operation Support

Document for:	Discussion

Introduction

In the RAN#102 meeting, a new Work Item for network energy saving enhancement has been approved in [1] including the following objective on on-demand SSB SCell operation:

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss on the support of on-demand SSB SCell operation relevant to above objective.

Discussions

In the RAN1 #116 bis meeting, the following agreements were made for on-demand SSB SCell operation [2]:

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

In the RAN1 #116 meeting, the following agreements were made for on-demand SSB SCell operation [3]:

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission.

In this document we prioritize Scenario #2 and Scenario #2A (Scenario #3 and Scenario #3B is FFS).

For the combination of Case #1 and Scenario #2, there is no always-on SSB transmitted on the SCell, and SCell is configured to a UE but before the UE receives SCell activation command. By triggering on-demand SSB for L3 measurement, it can help the network to understand whether to activate the SCell or not.

For the combination of Case #1 and Scenario #2A, there is no always-on SSB transmitted on the SCell, and SCell is configured to a UE and UE receives SCell activation command. On-demand SSB transmission can be utilized for the cell detection of SSB-less SCell, which it may benefit for reducing the latency of SCell activation procedure.

For the combination of Case #2 and Scenario #2, long always-on SSB is periodically transmitted on the cell, and SCell is configured to a UE but before the UE receives SCell activation command. By triggering a shorter-periodicity on-demand SSB, it can result in faster L1/L3 measurement, synchronization and UE connection.

For the combination of Case #2 and Scenario #2A, long always-on SSB is periodically transmitted on the cell, and SCell is configured to a UE and UE receives SCell activation command. By triggering a shorter-periodicity on-demand SSB, it can result in faster L1/L3 measurement, synchronization and UE connection.

- Observation 1: RAN1 has agreed to prioritize Scenario #2 and Scenario #2A, Case #1 and Case #2 for on-demand SSB SCell operation.

For case #1 (No always-on SSB on the cell), the cell can be activated through cell on/off indication via backhaul.

If the activation is between two NG-RAN nodes, the cell activation procedure that has already been defined in the current specification [4] can support a NG-RAN node to request its neighbouring NG-RAN node through the Xn interface to switch on the whole cell or only some of the SSB beams within one or more cells.

If the activation is between a CU and a DU, the F1 setup procedure and the gNB-CU Configuration Update procedure that have already been defined in the current specification [5] can support a CU to request a DU through the F1 interface to switch on some cells or only some of the SSB beams within one or more cells.

Proposal 1: For Case #1 (No always-on SSB on the cell), RAN3 to agree that:

- The existing cell activation message can be reused to activate the cell over Xn for the SCell on-demand SSB.

- The existing F1 setup procedure and the gNB-CU Configuration Update procedure can be reused to activate the cell over F1 for the SCell on-demand SSB.

For Case #2 (Always-on SSB is periodically transmitted on the cell), this cell may already be in a normal operational state. It's evident that there's no requirement for backhaul signalling to activate this cell. Therefore, RAN3 can proceed with discussions unless new information is required.

Proposal 2: For Case #2 (Always-on SSB is periodically transmitted on the cell), RAN3 to agree that:

- Since the cell might already be in a normal operational state, cell activation by reusing the existing messages is not needed.

Conclusions

In this submission, we discuss the support of on-demand SSB SCell operation. We focus on the combinations of Scenario #2, Scenario #2A, and Case #1, Case #2. The following observations and proposals are made:

Observation 1: RAN1 has agreed to prioritize Scenario #2 and Scenario #2A, Case #1 and Case #2 for on-demand SSB SCell operation.

Proposal 1: For Case #1 (No always-on SSB on the cell), RAN3 to agree that:

- The existing cell activation message can be reused to activate the cell over Xn for the SCell on-demand SSB.

- The existing F1 setup procedure and the gNB-CU Configuration Update procedure can be reused to activate the cell over F1 for the SCell on-demand SSB.

Proposal 2: For Case #2 (Always-on SSB is periodically transmitted on the cell), RAN3 to agree that:

- Since the cell might already be in a normal operational state, cell activation by reusing the existing messages is not needed.

Reference

- RP-234065 New WID: Enhancements of network energy savings for NR

- Draft Report of 3GPP TSG RAN WG1 #116b v0.3.0 (Changsha, China, April 15th – 19th, 2024)

- Final Report of 3GPP TSG RAN WG1 #116 v1.0.0 	(Athens, Greece, February 26th - March 1st, 2024)

- TS 38.423 V18.0.0

- TS 38.473 V18.0.0