3GPP TSG-RAN WG3 Meeting #124	R3-243802

Fukuoka, Japan, 20 – 24 May, 2024

Agenda Item:	11.3

Source:	Qualcomm (moderator)

Title:	Summary of offline for AI/ML enabled CCO

Document for:	Approval

# Introduction

CB: # AIML2_CCO

- Make the decision on whether to support DU inference in R19?

- Discuss the open issues

- Capture the agreements and provide TPs

(moderator - QC)

Summary of offline disc R3-243802

Agreement in Online session

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

Open issues from Online session

For Split architecture, how to derive future CCO state needs to be further checked.

How to support the future CCO status and/or predicted CCO issue transmission over Xn?

Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.

Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time information to NG-RAN node 2.

Model inference in DU for CCO use case has no consensus in R19?

Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status?

# For the Chairman’s Notes

For split architecture, the use of AI/ML to infer future CCO state in gNB-DU is not considered in RAN3 in Rel-19.

Agree TP in R3-243939

# Discussion

## I/O, Feedback and Standard Impact Predicted CCO issue and Future CCO state for Non-Split Architecture

From the TPs in [R3-243255] [R3-243269][R3-243290][R3-243385][R3-243450][R3-243467][R3-243500][R3-243559] the following input, output and feedback for AI/ML inference and standards impacts is derived in this discussion paper.

### Input

From local node:

-	Current CCO state

-	Current Radio Resource Status

-	Legacy Predicted Radio Resource Status

-	Legacy Predicted UE Trajectory

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

Conclusion: The

To be discussed

-	Measured/Predicted UE Traffic (e.g., data volume) (ZTE, Lenovo)

Can this be agreed?

=> Not Discussed

From the UE:

-	UE measurement report

-	SON Reports (RLF, CEF, RA)

-	UE Mobility History Information

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

=> Not Discussed

To be discussed:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) (Lenovo)

Is the above needed?

From neighbouring NG-RAN nodes:

-	Current CCO state

-	Future CCO state

-	Current Radio Resource Status

-	Legacy Predicted Radio Resource Status

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

=> Not Discussed

To be discussed:

-    UE measurements of UEs served by the gNB2 and reporting coverage of cells served by the gNB1 (E///)

Why is the above needed?

### Output

-	Future CCO State

-	Time when the future CCO state can be applied

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

=> Not Discussed

To be discussed:

-	Predicted CCO issue (E///)

-	Time when the predicted CCO issue can occur (E///)

-	future replacement information for cells (e.g. cell shaping, cell splitting, cell merging) (HW)

-	predicted UE offload plan (HW)

-	problem type (coverage gap or cell-edge interference) (CATT)

### Feedback

To optimize the performance of AI/ML based CCO model, following feedback can be considered to be collected from NG-RAN nodes:

-	UE performance for UEs handed over from source NG-RAN node.

-	SON Reports (RLF, CEF, RA) from source node

-	UE Measurement Report from source node

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

=> Not Discussed

To be discussed:

-	Cell level KPIs (e.g., throughput, delay, packet loss of current and neighbouring NG-RAN node) (QC)

-	Measured UE Traffic (ZTE)

### Standard Impact

Xn interface impact:

-	Future CCO State

-	Time when the future CCO state can be applied

The above parameters are the majority view as input to AI/ML inference. Can the above parameters be agreed?

=> Not Discussed

To be discussed

-	Predicted CCO issue (E///)

-	Time when the predicted CCO issue can occur (E///)

-	Signalling of UE measurements for UEs served by gNBs neighboring gNB1 and reporting coverage of cells served by the gNB1 (and vice versa) (E///)

-	future replacement information for cells (e.g. cell shaping, cell splitting, cell merging) (HW)

-	predicted UE offload plan (HW)

-	Predicted/Measured UE Traffic (ZTE)

## Enhanced CCO Signaling

From ZTE [3] and QC [1] paper, the below proposal is mentioned.

QC/ZTE Proposal:  	A gNB can propose a predicted coverage configuration for cells served by its neighboring gNBs. The neighboring gNB can either accept/reject this proposal and provide its own predicted coverage configuration upon request.

Can the above proposal be agreed?

=> Not Discussed

## Future CCO State

Can we agree the below?

For split architecture, the use of AI/ML to infer future CCO state in gNB-DU is not considered in RAN3 in Rel-19.

If the above is not agreeable, how can gNB-DU derive future CCO state using legacy mechanism for a predicted CCO issue?

# Conclusion, Recommendations [if needed]

If needed

# References

R3-243559	Discussion on AI/ML enabled CCO (Qualcomm Incorporated)

R3-243458	Further discussion on AIML support for CCO (Ericsson, InterDigital, Deutsche Telekom, Telecom Italia)

R3-243499	Discussion on AI/ML assisted CCO (ZTE)

R3-243467	(TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization (Huawei)

R3-243124	(TP for TR 38.743) Further discussion on solution for AI/ML-based CCO (Nokia)

R3-243255	(TP to TR38.743) AI/ML based CCO (NEC)

R3-243269	(TP to TR 38.743) AI/ML enabled CCO (Samsung)

R3-243290	(TP for TR 38.743 )Support of AI/ML enabled CCO (CATT)

R3-243385	Discussion on AI/ML based CCO (Lenovo)

R3-243386	TP to 38.743 for AIML based CCO (Lenovo)

R3-243450	TP on AIML support for CCO (Ericsson, InterDigital, Deutsche Telekom, Telecom Italia)

R3-243500	(TP to TR38.743) Discussion on AI/ML assisted CCO (ZTE)

R3-243603	Discussion on AI/ML based Coverage and Capacity Optimization (China Telecommunication)

R3-243604	(TP to 38.743) Support of AI/ML based Coverage and Capacity Optimization (China Telecommunication)

R3-243692	Issues on AI/ML-based CCO (LG Electronics Inc.)

R3-243693	(TP to TR 38.743) Issues on AI/ML-based CCO (LG Electronics Inc.)

R3-243719	Discussion on AI/ML-based CCO (CMCC)

R3-243767	Identification and Rectification of CCO Issues Using AIML (Rakuten Mobile, Inc)